3GPP TSG-RAN WG4 Meeting #92-Bis	R4-1910862

# Chongqing, China, 14th – 18th Oct, 2019

Source: 	Huawei, HiSilicon

Title:	UE RF requirements for addition of channel bandwidths for n66

Agenda Item:	9.25.1

Document for:	Approval

1	Introduction

In last RAN plenary meeting #85, a new WI was approved on addition of asymmetric channel bandwidth for n66 [1].

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |

In this contribution we provide some discussion and proposals on UE RF requirements.

  - Discuss UE REFSENS requirement

  - Discuss additional emission requirement for NS_03

2	Discussion

2.1 UE reference sensitivity

UE reference sensitivity is a band specific requirement. For 25, 30 and 50 MHz channel bandwidth, UE reference sensitivity need to be defined. The UL to DL frequency gap for n66 is 330MHz and it is large enough for avoiding UL tx spectrum regrowth. Hence the reference sensitivity requirement can be derived by scaling on the transmission bandwidth configuration. Table 2-1 shows the reference sensitivity limits proposed for 25, 30 and 50 MHz. And the uplink configuration is proposed in Table 2-2. It is noted that for 50 MHz there is no symmetric UL channel bandwidth, Note 3 is used to address the specific case.

Table 2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n1 | 15 | -99.5 | -96.3 | -94.5 | -93.3 | -92.2 | -91.4 | -90.1 | -89.1 |  |  |  |  | FDD |
|  | 30 |  | -96.6 | -94.6 | -93.5 | -92.3 | -91.5 | -90.2 | -89.2 |  |  |  |  |  |
|  | 60 |  | -97 | -94.9 | -93.7 | -92.5 | -91.6 | -90.4 | -89.3 |  |  |  |  |  |

Table 2-2: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n66 | 15 | 25 | 501 | 751 | 1001 | 1281 | 160 | 216 | NOTE 3 |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 501 | 641 | 751 | 1001 | NOTE 3 |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 24 | 301 | 361 | 501 | NOTE 3 |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). NOTE 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest duplex distance shall be used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 1: UE reference sensitivity for 25, 30 and 50 MHz channel bandwidth is defined in Table 2-1 and 2-2

2.2 Additional emission requirement

Additional emission requirements are required for NS_03, NS_03U and NS_100 for n66. Requirements for NS_03U is the complication of the requirements of NS_03 and NS_100 hence we only need to check the requirements for NS_100 and NS_03 respectively. NS_100 is for UTRA ACLR and is common for all channel bandwidths. Additional requirement for NS_03 is shown in Table 2-4. The limits for 25 MHz and 30 MHz is added.

Table 2-3: Mapping of network signaling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |

Table 2-4: Additional requirements for "NS_03"

|  |  |  | Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) |  | 5 MHz |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | Measurement bandwidth |  |
| 0-1 |  | -13 |  | -13 | -13 | -13 | -13 | -13 | -13 | 1 % of channel BW |  |
| 1-6 |  | -13 |  | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |  |
| 6-10 |  | -25 |  | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |  |
| 10-15 |  |  |  | -25 | -13 | -13 | -13 | -13 | -13 | 1 MHz |  |
| 15-20 |  |  |  |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |  |
| 20-25 |  |  |  |  |  | -25 | -13 | -13 | -13 | 1 MHz |  |
| 25-30 |  |  |  |  |  |  | -25 | -13 | -13 | 1 MHz |  |
| 30-35 |  |  |  |  |  |  |  | -25 | -13 | 1 MHz |  |
| 35-40 |  |  |  |  |  |  |  |  | -13 | 1 MHz |  |
| 40-45 |  |  |  |  |  |  |  |  | -25 | 1 MHz |  |

Proposal 2: Additional emission requirements are required for NS_03 is defined in Table 2-4 for 25 and 30 MHz channel bandwidth.

The A-MPR tables for NS_03 and NS_100 are cited below. It can be found at both cases the A-MPR is specified in channel bandwidth agnostic manner. Since 25 and 30 MHz channel bandwidths are in-between of already supported channel bandwidths, there will be no impact to A-MPR table.

Proposal 3: there is no impact to the A-MPR tables for NS_03 and NS_100

Table 6.2.3.1-2 (38.101-1): A-MPR for NS_100 (UTRA protection)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |
| NOTE 1: Void NOTE 2: Void |  |  |

Table 6.2.3.7-1 (38.101-1) A-MPR for NS_03

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | ≤ 1.5 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 3 |
|  | 64 QAM | ≤ 3.5 |
|  | 256 QAM | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4.5 |
|  | 256 QAM | ≤ 7.5 |
| NOTE 1: Void NOTE 2: Void |  |  |

3	Conclusion

Proposal 1: UE reference sensitivity for 25, 30 and 50 MHz channel bandwidth is defined in Table 2-1 and 2-2

Proposal 2: Additional emission requirements are required for NS_03 is defined in Table 2-4 for 25 and 30 MHz channel bandwidth.

Proposal 3: there is no impact to the A-MPR tables for NS_03 and NS_100

4	References

[1] RP-192276, New WI proposal: Addition of asymmetric channel bandwidth for n66