3GPP TSG-RAN WG4 Meeting #92-Bis	R4-191xxxx

# Chongqing, China, 14th – 18th Oct, 2019

Source: 	Huawei, HiSilicon

Title:	Discussion on the support of asymmetric channel bandwidth

Agenda Item:	9.25.1

Document for:	Approval

1	Introduction

In last RAN plenary meeting #85, a new WI was approved on addition of asymmetric channel bandwidth for n66 [1].

In this contribution we provide some discussion and work plan for the WI.

2	Discussion

From the objective of the WI [1],

- Introduce additional FDD asymmetric UL and DL channel bandwidth combinations as shown in Table below (highlighted by yellow).

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) |
| --- | --- | --- |
| n66 | 5, 10, | 15, 20, 25, 30, 40 |
|  | 20 | 25, 30, 40, 50 |
|  | 25 | 30, 40, 50 |
|  | 30 | 25, 40, 50 |

UE channel bandwidth per operating band for n66 should be updated. For 25 and 30 MHz both uplink and downlink are supported, and 50 MHz is only applicable to downlink. The needed changes can be found in Table 2-1.

Table 2-1 Channel bandwidths for n66

|  | NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 101,2 MHz | 152 MHz | 202 MHz | 252 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 904 MHz | 100 MHz |
| n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes3 |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes3 |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes3 |  |  |  |  |
| NOTE 1: 90% spectrum utilization may not be achieved for 30kHz SCS. NOTE 2: 90% spectrum utilization may not be achieved for 60kHz SCS. NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 1: The needed changes for UE channel bandwidth are proposed in Table 2-1 for n66.

In existing TS 38.101-1, the operating bands and supported asymmetric channel bandwidth combinations are defined in table 5.3.6-1, shown as below. And the TX-RX carrier center frequency separation is defined as a deviation to the default Tx-Rx frequency separation.  In RAN2 signalling, the uplink and downlink channel bandwidths are configured separately, and there is no such a UE capability signalling for asymmetric channel bandwidth operation. The channel bandwidth combinations are mandatory for the band. Therefore in our understanding the additional asymmetric channel bandwidth combinations should be mandatory support Rel-16 onward.

Table 2-2: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) |
| --- | --- | --- |
| n66 | 5, 10 | 15, 20, 25, 30, 40, 50 |
|  | 20 | 25, 30, 40, 50 |
|  | 25 | 30, 40, 50 |
|  | 30 | 25, 40, 50 |
| n70 | 5 | 10, 15 |
|  | 5, 10, 15 | 20, 25 |
| n71 | 5 | 10 |
|  | 10 | 15 |
|  | 15 | 20 |

Observation: The additional asymmetric channel bandwidth combinations should be mandatory support Rel-16 onward.

Proposal 2: The needed changes for FDD asymmetric UL and DL channel bandwidth combinations are proposed in Table 2-2 for n66.

3	Conclusion

Proposal 1: The needed changes for UE channel bandwidth are proposed in Table 2-1 for n66.

Observation: The additional asymmetric channel bandwidth combinations should be mandatory support Rel-16 onward.

Proposal 2: The needed changes for FDD asymmetric UL and DL channel bandwidth combinations are proposed in Table 2-2 for n66.

4	References

[1] RP-192276, New WI proposal: Addition of asymmetric channel bandwidth for n66