3GPP TSG-RAN WG4 Meeting #92-Bis	R4-1910865

# Chongqing, China, 14th – 18th Oct, 2019

Source: 	Huawei, HiSilicon, China Unicom

Title:	UE REFSENS for 25 MHz channel bandwidth for n1

Agenda Item:	9.24.1

Document for:	Approval

1	Introduction

In last RAN plenary meeting #85, a new WI was approved on addition of new channel bandwidth in NR band n1 [1].

In this contribution we provide some discussion and proposal on UE REFSENS for 25 MHz channel bandwidth.

2	Discussion

UE reference sensitivity is a band specific requirement. UE reference sensitivity for 25 MHz channel bandwidth need to be defined according to the WID [1]. The UL to DL frequency gap for n1 is 190 MHz and which might cause a small impact to RX due to UL tx spectrum regrowth for wider channel bandwidth. In LTE CA_1C, RB restriction on uplink configuration is used for 15 MHz + 15 MHz and 20 MHz + 20 MHz CA cases, and no RB restriction for 20 MHz+5 MHz case. ~129 RB is adopted for LTE CA cases, which is larger than the maximum transmission bandwidth configuration NRB for DFT-s-OFDM waveform (128 RB for 15 KHz SCS). Hence no RB restriction is proposed for 25 MHz channel bandwidth. The reference sensitivity requirement can be derived by scaling on the transmission bandwidth configuration. Table 2-1 shows the reference sensitivity limits proposed for 25 MHz. and the uplink configuration is proposed in Table 2-2.

Table 7.3.1A-1 in 36.101: Intra-band contiguous CA uplink configuration for reference sensitivity for Bandwidth Class C

| CA configuration / CC combination / NRB_agg / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink CA configuration | 100RB+25RB |  | 100RB+50RB |  | 75RB+75RB |  | 75RB+50RB |  | 100RB+75RB |  | 100RB+100RB |  | Duplex Mode |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |
| CA_1C | N/A | N/A | N/A | N/A | 75 | 54 | N/A | N/A | N/A | N/A | 100 | 30 | FDD |

Table 2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93 |  |  |  |  |  |  |  |  |

Table 2-2: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n1 | 15 | 25 | 501 | 751 | 1001 | 128 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 501 | 64 |  |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 24 | 30 |  |  |  |  |  |  |  |  |

Proposal 1: UE reference sensitivity for n1 25 MHz channel bandwidth is defined in Table 2-1 and 2-2

3	Conclusion

Proposal 1: UE reference sensitivity for n1 25 MHz channel bandwidth is defined in Table 2-1 and 2-2

4	References

[1] RP- 192275, New WI on addition of new channel bandwidth in NR band n1, China Unicom