3GPP TSG-RAN WG4 Meeting #92-Bis	R4-1910866

# Chongqing, China, 14th – 18th Oct, 2019

Source: 	Huawei, HiSilicon

Title:	Discussion on basket WI for adding new channel bandwidth(s) support to existing NR bands

Agenda Item:	12.1

Document for:	Approval

1	Introduction

In last RAN plenary meeting #85, basket WI for adding new channel bandwidth(s) support to existing NR bands was discussed in [1] and it was concluded that RAN4 will develop framework for a basket WI on addition of channel bandwidths to NR bands and the basket will be introduced in REL-17, for REL-16 separate WIs are considered. In the contribution we provided some views on the framework.

2	Discussion

In Release 15 for LTE re-farmed bands, LTE channel bandwidths (5/10/15/20 MHz) were introduced. In Release 16 some additional channel bandwidths were proposed in separate WIs, based on operators request on the available spectrum.

| Name | Release | WID |
| --- | --- | --- |
| Adding wider channel bandwidths in NR band n7 | Rel-16 | RP-190137 |
| Addition of wider channel bandwidth in NR band n38 | Rel-16 | RP-191472 |
| Addition of 30MHz channel bandwidth for NR band n41 | Rel-16 | RP-191569 |
| Adding wider channel bandwidths in NR band n50 | Rel-16 | RP-182829 |
| Addition of wider channel bandwidth in NR band n1 | Rel-16 | RP-191595 |
| Addition of channel bandwidths in Band n77 and n78 | Rel-16 | RP-191606 |
| Addition of wider channel bandwidth in NR band n28 | Rel-16 | RP-192261 |
| Adding new channel bandwidth in NR band n1 | Rel-16 | RP-192275 |
| Addition of asymmetric channel bandwidth for NR band n66 | Rel-16 | RP-192276 |
| Adding wider channel bandwidth to NR band n38 | Rel-16 | RP-192215 |
| Adding wider channel bandwidth to NR band n75 | Rel-16 | RP-192216 |
| Addition of 25, 30, 35 and 40 MHz channel bandwidth for NR band n25 | Rel-16 | RP-192219 |

In Release 16 with the introduction of 70 MHz UE channel bandwidth, the supported channel bandwidth set is the same for BS and UE, i.e. {5 MHz, 10 MHz,	15 MHz,	20 MHz,	25 MHz,	30 MHz,	40 MHz,	50 MHz,	60 MHz,	70 MHz,	80 MHz,	90 MHz,	100 MHz}. For these channel bandwidth(s), all BS RF requirements are available and no addition work is needed. And for UE the impact will be limited to reference sensitivity for RX and possible A-MPR analysis for TX depend on whether additional emission requirements are defined for the band. Other channel bandwidth(s) should not be within the scope of the basket WI, since it will need more discussion on the use case and implementation complexity, etc.

Proposal 1: addition of channel bandwidth(s) for existing NR bands should be within the Rel-16 channel bandwidth set, i.e. {5 MHz, 10 MHz,	15 MHz,	20 MHz,	25 MHz,	30 MHz,	40 MHz,	50 MHz,	60 MHz,	70 MHz,	80 MHz,	90 MHz,	100 MHz}

To introduce additional channel bandwidth for existing NR bands, it implies that additional design and test effort will be needed to guarantee the performance and co-existence, and overall cost may also be impacted. The spectrum information for the band will be helpful for the vendors to consider the new channel bandwidth. Hence it is required that the detailed spectrum information should be provided in the justification for the endorsement in RAN4.

Proposal 2:  the detailed spectrum information should be provided in the justification for the endorsement in RAN4

3	Conclusion

Proposal 1: addition of channel bandwidth(s) for existing NR bands should be within the Rel-16 channel bandwidth set, i.e. {5 MHz, 10 MHz,	15 MHz,	20 MHz,	25 MHz,	30 MHz,	40 MHz,	50 MHz,	60 MHz,	70 MHz,	80 MHz,	90 MHz,	100 MHz}

Proposal 2:  the detailed spectrum information should be provided in the justification for the endorsement in RAN4

4	References

[1] RP-192011, New basket WID on adding channel bandwidth support to existing NR bands, Ericsson