3GPP TSG-RAN WG4 Meeting #92bis                       	 R4- 1910896

# Chongqing, China, 14th - 18th Oct, 2019

Agenda Item:	8.4.5.2

Source:	MediaTek Inc., LG Electronics

Title:	Link-level simulation assumptions for NR V2X synchronization

Document for:		Approval

- Introduction

In legacy RAN4 meeting, some simulation assumption are agreed.

RAN4 #92 meeting Agreements: - Use band n47 (5.9GHz) as the frequency carrier to define the RRM requirements for NR V2X - The simulation for other frequency bands will depend on the conclusion of RF session - For the evaluation of detection time, the maximum relative velocity is assumed to be 260km/h - For evaluation, 15kHz/30kHz/60kHz will be considered. - No need to define the requirements for all the SCS-es.

RAN1 also had some agreements for the synchronization signals design. RAN4 should start to discuss the evaluation the S-PSS/S-SSS detection time based on RAN1’s newest design.

RAN1 #96bis meeting Agreements: - In NR V2X, S-SSB bandwidth is 11RBs. - PSBCH spans 11RBs. - The S-SSB is designed following combination 1. - Length-127 M-sequences for S-PSS and length-127 Gold sequences for S-SSS - Two symbols are used for each of S-PSS and S-SSS, respectively. RAN1 #97 meeting Working assumption: - For the NR SLSS, - Same sequence is used for both symbols of S-PSS - Same sequence is used for both symbols of S-SSS - S-PSS detection search window: 80ms and 160ms. Agreements - The following parameters are assumed for evaluation: o Power Difference for S-PSS and S-SSS symbols:  Opt.1) MPR values: S-PSS = 0 dB, S-SSS = 3 dB;  Opt.2) MPR values: S-PSS = 3 dB, S-SSS = 3 dB  Opt.3) companies to report the assumed MPR values o Transient period is  10us for FR1; 5us for FR2  Waveform puncturing during the transient period o S-PSS detection search window: 80ms and 160ms  At least for evaluation, one S-SSB transmission with at least the following periodicity: o 160ms period at least for 15kHz SCS. o FFS other value(s) RAN1 #98 meeting Agreements: - The number of NR V2X SSID is 672 with the combination of {2 S-PSS candidates * 336 S-SSS candidates}. Agreements: - NR S-SSB structure for NCP is as follows: - For the case of ECP, the structure is the same as the above except that the number of PSBCH symbols after S-SSS is only 6 - 160ms is supported as the S-SSB periodicity for all SCS. - The number of S-SSB transmissions within one S-SSB period is (pre)configurable For FR1: - For 15kHz SCS, {1, [2]} - For 30kHz SCS, {1, 2, [4]} - For 60kHz SCS, {1, 2, 4, [8]} For FR2: - For 60kHz SCS, {1, 2, 4, 8, 16, 32} - For 120kHz SCS, {1, 2, 4, 8, 16, 32, 64}

In this paper, we propose the simulation assumption for S-PSS/S-SSS detection for SyncRef UE selection/reselection requirement.

- SideLink Search Simulation Assumption

The target NR V2X design is to re-use NR design as much as possible, such as numerology, CP-OFDM, PSS, SSS sequence, …, etc. Thus it’s better to update NR PSS/SSS simulation configuration to evaluate the sidelink S-PSS/S-SSS detection performance.

Proposal 1: NR V2X S-PSS/S-SSS detection simulation assumption should re-use NR PSS/SSS simulation assumption as much as possible.

Table 1: Suggested SyncRef UE Parameters for S-PSS/S-SSS Detection

| Parameter | Unit | SyncRef UE 1 | SyncRef UE 2Note 1 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| Carrier frequency |  | FR1: 5.9GHz; | The same frequency as for SyncRef UE 1 is used |
| Subcarrier spacing | KHz | 15 kHz, 30 kHz, 60kHz for FR1; | The same values as for SyncRef UE 1 are used |
| RB Utilization | % | 100 | 100 |
| Shifted 1st S-SSS symbol location from 2nd S-PSS Note 4 | symbol | 0, 1 | The same values as for SyncRef UE 1 are used |
| Transient Period Note 4 | μs | 10 for FR1; waveform puncturing during the transient period, it applies for only 3dB power boosting of S-PSS | The same values as for SyncRef UE 1 are used |
| Power boosting for S-PSS Note 4 | dB | 0, 3; ‘0’ applies for 0 symbol shift of 1st S-SSS symbol from 2nd S-PSS | The same values as for SyncRef UE 1 are used |
| Power boosting for S-SSS | dB | 0 | 0 |
| Power boosting for Data and Control | dB | 0 | 0 |
| Data Modulation | - | QPSK | QPSK |
| CP Length | - | Normal | Normal |
| Number of transmitted S-SSB within one S-SSB period (K) | - | 1 (baseline) | 1(baseline) |
| S-SSB periodicity | ms | 160 | 160 |
| Relative Delay of 1st Path (synchronous) | μs | 0 | CP/2 |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| Es/Noc for K=1 | dB | 6.4 | 1.25; 0.25; -0.75 (SINR: -6dB; -8dB, -7dB) |
| Number of Tx antennas | - | 1 | 1 |
| PSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3) | To be stated by companies together with the results (examples shows in Table 3) |
| SSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3) | To be stated by companies together with the results (examples shows in Table 3) |
| SLSS resource (Note 2) | - | Resource ‘A’ | Resource ‘A’ |
| Propagation Condition | - | AWGN, TDL- C with 30ns, 1400Hz, TDL- C with 100ns, 300Hz |  |
| Ioc Model Note3 | - | AWGN noise |  |
| Note 1: SyncRef UE 2 has the higher priority than SyncRef UE 1. Note 2: SLSS resources corresponding to sync-OffsetIndicator1 , sync-OffsetIndicator2 and sync-OffsetIndicator3, respectively. The two resources are labelled here as ‘A’, ‘B’ and ‘C’. Note 3: Combination of {Shifted 1st S-SSS symbol location from 2nd S-PSS(symbol), Transient Period(μs), Power boosting for S-PSS(dB)} are {0,N/A,0}, {0,10,3} and {1,10,3}. Note 4: The necessity and exact value of transient period will be decided in RAN4 RF session. |  |  |  |

Table 2: UE-specific Parameters for SLSS Detection

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of SyncRef UE 1 by the UE | Yes |
| False detect threshold | At most 1% |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” syncRef UE detection of both S-PSS and S-SSS sequence IDs. |
| Receive antennas | 2 (uncorrelated) |

Table 3: Example SLSS ID Combinations that can be simulated

| Case # | SyncRef UE 1 | SyncRef UE 2 | Scenario |
| --- | --- | --- | --- |
| 1 | 336+112 | 112 | Synchronous |
| 2 | 336+224 | 112 | Synchronous |

- Time Plan

It is proposed that initial simulation results based on the assumptions above are provided in RAN4#93.

Proposal 2: RAN4 to agree on Tables 1, 2, 3 as the simulation assumption for synchronization alignment in RAN4#93 meeting.

- Summary

In this paper, we give the simulation assumption for RAN4 spec. synchronization based on RAN1’s NR V2X design.

Proposal 1: NR V2X S-PSS/S-SSS detection simulation assumption should re-use NR PSS/SSS simulation assumption as much as possible.

Proposal 2: RAN4 to agree on Tables 1, 2, 3 as the simulation assumption for synchronization alignment in RAN4#93 meeting.