3GPP TSG-RAN WG4 Meeting #92Bis	R4-1910913

# Chongqing, China, 14th – 18th Oct., 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS38.133 for SA SS-SINR tests with PCell in FR2 (Section A.7.7.3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Modify the test 1 for intra-frequency measurement so that the Io value can be close to -50dBm |  |  |  |  |  |  |  |  |
| Summary of change: |  | Increase the Noc and corresponding values for the test 1 of intra-frequency measurement so that the Io value can be close to -50dBm |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Io values for the tests for intra-frequency measurement are too close |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------- Beginning of Change ------------------------------------------------------------

### A.7.7.3	SS-SINR

#### A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell

##### A.7.7.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.13.1.1.

##### A.7.7.3.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configurations are shown in Table A.7.7.3.1.2-1. . The absolute accuracy of SS-SINR intra-frequency measurement is test by using the parameters in Table A.7.7.3.1.2-2 and Table A.7.7.3.1.2-3. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. The TCI status for Cell 1 is defined in Table  A.3.16.2-1 and TRS configuration for Cell 1 is defined in Table A.3.17.2.1-1.

Table A.7.7.3.1.2-1: SS-SINR Intra frequency SS-SINR supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.7.3.1.2-2: SS-SINR Intra frequency test parameters

| Parameter | Unit | Test 1 |  |  |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 |  |  | Cell 1 | Cell 2 |
| SSB ARFCN |  | Freq2 |  |  |  | Freq2 |  |
| Duplex mode |  | TDD |  |  |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  |  |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  |  |  | 100: NRB,c = 66 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 |  |  |  |  |  |
| DRX cycle configuration | ms | Not applicable |  |  |  |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  |  |  |  |
| TCI state |  | TCI.State.0 |  |  |  |  |  |
| AoA setup |  | Setup 3 defined in A.3.15 |  |  |  |  |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |  |  |  | SR.3.1 TDD |  |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - |  |  | CR.3.1 TDD |  |
| Dedicated RMSI CORESET Reference Channel |  | CCR.3.1 TDD | - |  |  | CCR.3.1 TDD | - |
| OCNG Patterns |  | OP.1 | OP.1 |  |  | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 |  |  |  |  |  |
| SSB configuration |  | SSB.1 FR2 | SSB.1 FR2 |  |  | SSB.1 FR2 | SSB.1 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 |  |  | 120 | 120 |
| SS-RSSI-Measurement |  | Not Applicable |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 0 | 0 |  |  | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
|  | dB | 4.54 | 2.66 |  |  | N/A | N/A |
| Propagation conditions |  | AWGN |  |  |  |  |  |
| Antenna configuration |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

Table A.7.7.3.1.2-3: SS-SINR Intra frequency OTA related test parameters

| Parameter |  | Unit | Test 1 |  |  |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |  |  | Cell 1 | Cell 2 |
| Angle of arrival configuration |  |  | According to clause A.3.8.X |  |  |  | According to clause A.3.8.X |  |
| Note1 | NR_TDD_FR2_A | dBm/15kHz Note4 | -96 |  |  |  | N/A |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | N/A |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | N/A |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | N/A |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | N/A |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | N/A |  |
| Note1 | NR_TDD_FR2_A | dBm/SCS Note3 | -87 |  |  |  | N/A |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | N/A |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | N/A |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | N/A |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | N/A |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | N/A |  |
| SS-RSRPNote2 | NR_TDD_FR2_A | dBm/SCS Note4 | -82.46 | -84.34 |  |  | Note7 | Note7 |
|  | NR_TDD_FR2_B |  |  |  |  |  | Note7 | Note7 |
|  | NR_TDD_FR2_F |  |  |  |  |  | Note7 | Note7 |
|  | NR_TDD_FR2_G |  |  |  |  |  | Note7 | Note7 |
|  | NR_TDD_FR2_T |  |  |  |  |  | Note7 | Note7 |
|  | NR_TDD_FR2_Y |  |  |  |  |  | Note7 | Note7 |
| SS-SINR Note2 | NR_TDD_FR2_A | dB | 0 | -3.2 |  |  | 0 | 0 |
|  | NR_TDD_FR2_B |  |  |  |  |  | 0 | 0 |
|  | NR_TDD_FR2_F |  |  |  |  |  | 0 | 0 |
|  | NR_TDD_FR2_G |  |  |  |  |  | 0 | 0 |
|  | NR_TDD_FR2_T |  |  |  |  |  | 0 | 0 |
|  | NR_TDD_FR2_Y |  |  |  |  |  | 0 | 0 |
|  |  | dB | 0 | -3.2 |  |  | 0 | 0 |
| IoNote2 | NR_TDD_FR2_A | dBm/95.04 MHz Note4 | -50.2 |  |  |  | Note8 |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | Note8 |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | Note8 |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | Note8 |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | Note8 |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | Note8 |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: NR operating band groups are as defined in clause 3.5.2. Note 7: SS_RSRP is applied at level 2dB above the minimum level specified in Table B.2.2-2 for sphereical coverage. Note 8: Io is applied at level 10log10(792)+2 dB above the minimum level specified in Table B.2.2-2 for sphereical coverage. |  |  |  |  |  |  |  |  |

##### A.7.7.3.1.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in clause 10.1.13.1.1.

#### A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell

##### A.7.7.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.15.1.1 and 10.1.15.1.2 for inter-frequency measurement.

##### A.7.7.3.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.7.7.3.2.2-1. Both absolute accuracy and relative accuracy requirements of SS-SINR inter-frequency measurement are tested by using test parameters in Table A.7.7.3.2.2-2 and Table A.7.7.3.2.2-3. In all test cases, Cell 1 is the PCell and Cell 2 is target cell. The TCI status for Cell 1 is defined in Table  A.3.16.2-1 and TRS configuration for Cell 1 is defined in Table A.3.17.2.1-1.

Table A.7.7.3.2.2-1: SS-SINR Inter frequency SS-SINR supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.7.3.2.2-2: SS-SINR Inter frequency general test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| Duplex mode |  | TDD |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 |  |  |  |  |  |
| DRX cycle configuration | ms | Not applicable |  |  |  |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  |  |  |  |
| TCI state |  | TCI.State.0 |  |  |  |  |  |
| AoA setup |  | Setup 3 defined in A.3.15 |  |  |  |  |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - | CR.3.1 TDD | - |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
|  | dB | -0.5 | -0.5 | 11.0 | 11.0 | -3.0 | -3.0 |
| Propagation conditions |  | AWGN |  |  |  |  |  |
| Antenna configuration |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

Table A.7.7.3.2.2-3: SS-SINR Inter frequency OTA related test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration |  | degrees | Setup 1 according to A.3.15.1 |  | Setup 1 according to A.3.15.1 |  | Setup 1 according to A.3.15.1 |  |
| Note1 | NR_TDD_FR2_A | dBm/15kHz Note4 | -105 |  | -105 |  | Note7 |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | Note7 |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | Note7 |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | Note7 |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | Note7 |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | Note7 |  |
| Note1 | NR_TDD_FR2_A | dBm/SCS Note3 | -96 |  | -96 |  | Note7 |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | Note7 |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | Note7 |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | Note7 |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | Note7 |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | Note7 |  |
| SS-RSRPNote2 | NR_TDD_FR2_A | dBm/SCS Note4 | -96.5 | -96.5 | -85 | -85 | Note8 | Note8 |
|  | NR_TDD_FR2_B |  |  |  |  |  | Note8 | Note8 |
|  | NR_TDD_FR2_F |  |  |  |  |  | Note8 | Note8 |
|  | NR_TDD_FR2_G |  |  |  |  |  | Note8 | Note8 |
|  | NR_TDD_FR2_T |  |  |  |  |  | Note8 | Note8 |
|  | NR_TDD_FR2_Y |  |  |  |  |  | Note8 | Note8 |
| SS-SINRNote2 | NR_TDD_FR2_A | dB | -0.5 | -0.5 | 11 | 11 | -3.0 | -3.0 |
|  | NR_TDD_FR2_B |  |  |  |  |  | -3.0 | -3.0 |
|  | NR_TDD_FR2_F |  |  |  |  |  | -3.0 | -3.0 |
|  | NR_TDD_FR2_G |  |  |  |  |  | -3.0 | -3.0 |
|  | NR_TDD_FR2_T |  |  |  |  |  | -3.0 | -3.0 |
|  | NR_TDD_FR2_Y |  |  |  |  |  | -3.0 | -3.0 |
|  |  | dB | -0.5 | -0.5 | 11 | 11 | -3.0 | -3.0 |
| IoNote2 | NR_TDD_FR2_A | dBm/95.04 MHz Note4 | -69.3 |  | -55.4 |  | Note9 |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | Note9 |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | Note9 |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | Note9 |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | Note9 |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | Note9 |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: NR operating band groups are as defined in clause 3.5.2. Note 7: Noc for SCS 15kHz is applied at level -10log10(8)+4dB above the minimum level specified in Table B.2.3-2 for sphereical coverage. Noc for SCS 120kHz is applied at 4 dB above the minimum level specified in Table B.2.3-2 for sphereical coverage. Note 8: SS_RSRP is applied at level 3dB above the minimum level specified in Table B.2.3-2 for sphereical coverage. Note 9: Io is applied at level 10log10(792)+6.54dB above the minimum level specified in Table B.2.3-2 for sphereical coverage. |  |  |  |  |  |  |  |  |

##### A.7.7.3.2.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in clause 10.1.15.1.1 and 10.1.15.1.2.

----------------------------------------------------------- End of Change ----------------------------------------------------------------