3GPP TSG-RAN WG4 #92bis Meeting							R4-1910947

Chongqing, China, 14 - 18 Oct, 2019

Agenda Item:	8.7.3

Source: 	CATT

Title:	Link-level simulation assumptions for SSB-based RRM measurement for power saving capable UE

Document for:	Approval

Introduction

In this contribution, simulation assumptions for SSB-based RRM measurement for power saving capable UE are proposed.

Simulation Assumptions

Table 1: General parameters

| Simulation parameters | Comments/values |  |
| --- | --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 4 GHz | 30 GHz |
| System bandwidth | 5 MHz; 10 MHz | 50 MHz; 100 MHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | No / Yes | No / Yes |
| DRX | No | No |
| BS transmit antennas for SS blocks | 1 tx or single layer transmissions | 1 tx or single layer transmissions |
| UE receive antennas | Case A : 2 rx and 4 rx (uncorrelated with equal gain, no rx beamforming); Case B: 2 rx and 4 rx with rx beamforming Note1 | Case A: 2 rx and 4 rx (uncorrelated with equal gain, no rx beamforming); Case B: 2 rx and 4 rx with rx beamforming Note1 |
| Data and control channel subcarrier spacing | The same as SS block subcarrier spacing | The same as SS block subcarrier spacing |
| Measurement period (in number of measurement samples) | 5; 3; 1 | 5; 3; 1 |
| SSB configuration in Cell 1 and Cell 2: |  |  |
| - Subcarrier spacing | 15 kHz; 30 kHz | 120 kHz; 240 kHz |
| - Number of SS blocks per SS burst set, K | 1and 4 | 1 and 4 |
| - SS burst periodicity | 5 ms | 5 ms |
| - Number of transmit antenna ports | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) |
| - SS block BW | 127 subcarriers for SS, 288 subcarriers for PBCH | 127 subcarriers for SS, 288 subcarriers for PBCH |
| - Actual SS block transmissions | always transmitted | always transmitted |
| SMTC configuration in Cell 1 and Cell 2: |  |  |
| - SMTC periodicity | 20ms,40ms,80ms,160ms | 20ms,40ms,80ms,160ms |
| - SMTC offset | 0ms | 0ms |
| - SMTC duration | 1ms | 1ms |
| NOTE1: For Case B, companies are encouraged to state their assumptions on beamforming and rx antenna configuration together with the provided simulation results |  |  |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS and NR-SSS sequences | - | To be indicated by companies | To be indicated by companies |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | Ms | 0 | 3 ms |
| SNR for Case A | dB | K=1: 6 dB K=4: (2, 6,4, -2) dB | K=1: 1 dB K=4: (2, 1, 1, 1) dB and (-1, 5, -3, 2) dB |
| Es/IoT for Case A (calculated from SNR) | dB | N/A | K=1: -5.97 dB K=4: (-2.12, -5.97, -4.45, -1.12) dB and (-5.12, 1.46, -8.45, -0.12) |
| SNR for Case B, including beamforming gain | dB | K=1: 6 dB K=4: (2, 6,4, -2) dB | K=1: 1 dB K=4: (2, 1, 1, 1) dB and (-1, 5, -3, 2) dB |
| Es/IoT for Case B (calculated from SNR), including beamforming gain | dB | N/A | K=1: -5.97 dB K=4: (-2.12, -5.97, -4.45, -1.12) dB and (-5.12, 1.46, -8.45, -0.12) |
| Propagation conditions | - | AWGN, EPA5, ETU30, ETU70, CDL-A/B/C Note, TDL-A/B/C Note |  |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned |  |  |  |

Performance Metrics

For different estimation time periods, the CDF curves and 90th %-iles are to be provided for (without RF impairment margin):

- Delta measurement   = (estimated measurement – ideal measurement) 	[dB];

- Delta cell quality based on N SS block measurements which are L1 filtered but not L3 filtered = (estimated - ideal) [dB], where N=1 (for K=1) and N=K/2 (for K>1).

References

[1] R4-1706904, Link-level simulation assumptions for SS block measurements, Ericsson