3GPP TSG-RAN4 Meeting #92bis		R4-1910950

Chongqing, China, 14th Oct – 18th Oct, 2019

Title:	Simulation results for CSI-RSRP measurement accuracy

Source:	CATT

Agenda Item:	8.16.1

Document for:	Discussion

- Introduction

In RAN4#92 meeting [1], it was agreed to conduct simulation studies to evaluate measurement bandwidth of CSI-RS for RRM measurement requirement. And the simulation assumption for CSI-RS based measurement is agreed in [2]. In this contribution, we provide the simulation results for CSI-RSRP measurement.

- Discussion

In this section, we provide the simulation results of delta RSRP under AWGN, EPA5 for 15 KHz and 30 KHz. Table 1-8 provide the delta RSRP based on different samples, CSI-RS resource density and configured PRB numbers for 15KHz and 30KHz, respectively.

15 KHz case:

Table 1: Delta RSRP, SINR=-5.97 dB, sync, 15 KHz, D=1, 24PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -2.72 | 1.18 | 4.13 | 3.9 | 2.95 | 3.42 |
|  | 5 | -1.84 | 1.25 | 3.33 | 3.09 | 2.08 | 2.58 |
|  | 10 | -0.76 | 1.11 | 2.80 | 1.87 | 1.69 | 1.78 |
| EPA5 | 3 | -3.58 | 1.19 | 4.30 | 4.77 | 3.11 | 3.94 |
|  | 5 | -2.23 | 1.12 | 3.41 | 3.35 | 2.29 | 2.82 |
|  | 10 | -1.15 | 0.92 | 2.71 | 2.07 | 1.79 | 1.93 |

Table 2: Delta RSRP, SINR=-5.97 dB, sync, 15 KHz, D=3, 24PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -2.01 | 0.51 | 2.23 | 2.52 | 1.72 | 2.12 |
|  | 5 | -1.35 | 0.34 | 2.03 | 1.69 | 1.69 | 1.69 |
|  | 10 | -0.99 | 0.39 | 1.71 | 1.38 | 1.32 | 1.35 |
| EPA5 | 3 | -2.74 | 0.27 | 2.53 | 3.01 | 2.26 | 2.63 |
|  | 5 | -2.12 | 0.16 | 2.11 | 2.28 | 1.95 | 2.11 |
|  | 10 | -1.38 | 0.15 | 1.57 | 1.53 | 1.42 | 1.47 |

Table 3: Delta RSRP, SINR=-5.97 dB, sync, 15 KHz, D=1, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -1.82 | 0.51 | 2.44 | 2.33 | 1.93 | 2.13 |
|  | 5 | -1.37 | 0.50 | 2.25 | 1.87 | 1.75 | 1.81 |
|  | 10 | -0.64 | 0.50 | 2.05 | 1.14 | 1.55 | 1.34 |
| EPA5 | 3 | -2.79 | 0.83 | 3.68 | 3.62 | 2.85 | 3.23 |
|  | 5 | -2.11 | 0.76 | 3.02 | 2.87 | 2.26 | 2.56 |
|  | 10 | -1.32 | 0.66 | 2.54 | 1.98 | 1.88 | 1.93 |

Table 4: Delta RSRP, SINR=-5.97 dB, sync, 15 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -1.43 | 0.31 | 1.71 | 1.74 | 1.4 | 1.57 |
|  | 5 | -0.71 | 0.17 | 1.55 | 0.88 | 1.38 | 1.13 |
|  | 10 | -0.64 | 0.31 | 1.28 | 0.95 | 0.97 | 0.96 |
| EPA5 | 3 | -2.11 | 0.40 | 2.22 | 2.51 | 1.82 | 2.16 |
|  | 5 | -1.65 | 0.23 | 1.94 | 1.88 | 1.71 | 1.79 |
|  | 10 | -0.91 | 0.24 | 1.41 | 1.15 | 1.17 | 1.16 |

Table 5: Delta RSRP, SINR=-5.97 dB, syn,15kHz, D=1, 96PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -1.74 | 0.31 | 1.96 | 2.05 | 1.65 | 1.85 |
|  | 5 | -1.31 | 0.28 | 1.52 | 1.59 | 1.24 | 1.41 |
|  | 10 | -0.95 | 0.28 | 1.25 | 1.23 | 0.97 | 1.10 |
| EPA5 | 3 | -2.04 | 0.67 | 3.22 | 2.71 | 2.55 | 2.63 |
|  | 5 | -1.47 | 0.65 | 2.55 | 2.12 | 1.9 | 2.01 |
|  | 10 | -0.96 | 0.67 | 2.15 | 1.63 | 1.48 | 1.55 |

Table 6: Delta RSRP, SINR=-3.97 dB, syn,15kHz, D=1, 96PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -0.78 | 0.31 | 1.26 | -3.78 | 1.09 | -1.34 |
|  | 5 | -0.52 | 0.29 | 1.01 | -5.52 | 0.81 | -2.35 |
|  | 10 | -0.34 | 0.34 | 0.81 | -10.34 | 0.68 | -4.83 |

30 KHz case:

Table 7: Delta RSRP, SINR=-5.97 dB, sync, 30 KHz, D=1, 24PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -3.35 | 1.15 | 4.25 | 4.5 | 3.1 | 3.80 |
|  | 5 | -2.08 | 1.15 | 3.36 | 3.23 | 2.21 | 2.7 |
|  | 10 | -1.14 | 0.90 | 2.61 | 2.04 | 1.71 | 1.87 |
| EPA5 | 3 | -2.65 | 1.46 | 5.24 | 4.11 | 3.78 | 3.94 |
|  | 5 | -1.88 | 1.62 | 4.4 | 3.5 | 2.78 | 3.14 |
|  | 10 | -0.52 | 1.46 | 3.80 | 1.98 | 2.34 | 2.16 |

Table 8: Delta RSRP, SINR=-5.97 dB, sync, 30 KHz, D=3, 24PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -1.80 | 0.47 | 2.43 | 2.27 | 1.96 | 2.11 |
|  | 5 | -1.37 | 0.51 | 2.14 | 1.88 | 1.63 | 1.75 |
|  | 10 | -0.64 | 0.51 | 2.05 | 1.15 | 1.54 | 1.34 |
| EPA5 | 3 | -2.42 | 0.59 | 3.01 | 3.01 | 2.42 | 2.71 |
|  | 5 | -1.78 | 0.52 | 2.60 | 2.3 | 2.08 | 2.19 |
|  | 10 | -0.93 | 0.36 | 2.19 | 1.29 | 1.83 | 1.56 |

Table 9: Delta RSRP, SINR=-5.97 dB, sync, 30 KHz, D=1, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -2.63 | 0.48 | 2.80 | 3.11 | 2.32 | 2.71 |
|  | 5 | -1.96 | 0.47 | 2.23 | 2.43 | 1.76 | 2.09 |
|  | 10 | -1.22 | 0.46 | 1.73 | 1.68 | 1.27 | 1.47 |
| EPA5 | 3 | -2.18 | 1.19 | 4.45 | 3.37 | 3.26 | 3.31 |
|  | 5 | -1.91 | 1.20 | 3.99 | 3.11 | 2.79 | 2.95 |
|  | 10 | -0.78 | 1.19 | 3.26 | 1.97 | 2.07 | 2.02 |

Table 10: Delta RSRP, SINR=-5.97 dB, sync, 30 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -1.29 | 0.31 | 1.87 | 1.6 | 1.56 | 1.58 |
|  | 5 | -1.08 | 0.37 | 1.50 | 1.45 | 1.13 | 1.29 |
|  | 10 | -0.55 | 0.31 | 1.33 | 0.86 | 1.02 | 0.94 |
| EPA5 | 3 | -1.96 | 0.50 | 2.72 | 2.46 | 2.22 | 2.34 |
|  | 5 | -1.38 | 0.50 | 2.45 | 1.88 | 1.95 | 1.91 |
|  | 10 | -0.92 | 0.56 | 1.76 | 1.48 | 1.2 | 1.34 |

Table 11: Delta RSRP, SINR=-5.97 dB, syn,30kHz, D=1, 96PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -1.79 | 0.34 | 2.01 | 2.13 | 1.67 | 1.90 |
|  | 5 | -1.49 | 0.31 | 1.64 | 1.8 | 1.33 | 1.56 |
|  | 10 | -0.92 | 0.26 | 1.11 | 1.18 | 0.85 | 1.01 |
| EPA5 | 3 | -2.27 | 0.85 | 3.37 | 3.12 | 2.52 | 2.82 |
|  | 5 | -1.61 | 0.76 | 3.08 | 2.37 | 2.32 | 2.34 |
|  | 10 | -0.85 | 0.71 | 2.37 | 1.56 | 1.66 | 1.61 |

Table 12: Delta RSRP, SINR=-3.97 dB, syn,30kHz, D=1, 96PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -0.78 | 0.32 | 1.24 | 1.1 | 0.92 | 1.01 |
|  | 5 | -0.67 | 0.30 | 1.09 | 0.97 | 0.79 | 0.88 |
|  | 10 | -0.35 | 0.23 | 0.73 | 0.58 | 0.5 | 0.54 |
| EPA5 | 3 | -1.75 | 0.29 | 2.03 | 2.04 | 1.74 | 1.89 |
|  | 5 | -1.27 | 0.20 | 1.63 | 1.47 | 1.43 | 1.45 |
|  | 10 | -0.83 | 0.20 | 1.18 | 1.03 | 0.98 | 1.00 |

Compared in Table 1, 3, 5, 7 and Table 2, 4, 6, 8, we can observe that the gain of CSI-RSRP accuracy can be obtained by improving the density of CSI-RS resource. Similarity, compared in Table 1, 2, 5, 6 and Table 3, 4, 7, 8, we can observe that the gain of CSI-RSRP accuracy can be obtained by improving the number of PRB for configured CSI-RS resource. According to the above simulation results, when the sample number is larger than 5, for most of cases, the delta RSRP of CSI-RS can be in ±2dB.

Observation 1: The gain of CSI-RSRP accuracy can be obtained by improving the density of CSI-RS resource.

Observation 2: the gain of CSI-RSRP accuracy can be obtained by improving the number of PRB for configured CSI-RS resource.

Observation 3: when the sample number is larger than 5, for most of cases, the delta RSRP of CSI-RS can be in ±2dB.

- Conclusion

In this contribution, we provide the simulation results for CSI-RSRP measurement, and provide the observations as follows:

Observation 1: The gain of CSI-RSRP accuracy can be obtained by improving the density of CSI-RS resource.

Observation 2: the gain of CSI-RSRP accuracy can be obtained by improving the number of PRB for configured CSI-RS resource.

Observation 3: when the sample number is larger than 5, for most of cases, the delta RSRP of CSI-RS can be in ±2dB.

- Reference

[1]	R4-19xxxxx,	“RAN4 #92 Meeting report”,		RAN4

[2]	R4-1910949,	“Link-level simulation assumptions for CSI-RS based measurements”,		CATT

- Annex

15 KHz case:

Case1：D=1, SCS = 15 KHz, 24PRBs

F. AWGN Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms  F. EPA5 Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms

Case2：D=3, SCS = 15 KHz, 24PRBs

F. AWGN Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms  F. EPA5 Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms

Case3：D=1, SCS = 15 KHz, 48PRBs

F. AWGN Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms  F. EPA5 Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms

Case4：D=3, SCS = 15 KHz, 48PRBs

F. AWGN Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms  F. EPA5 Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms

Case5：D=1, SCS = 15 KHz, 96PRBs

F. AWGN Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms  F. EPA5 Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms

F. AWGN Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms  F. EPA5 Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms

15 KHz case:

Case6：D=1, SCS = 30 KHz, 24PRBs

F. AWGN Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms  F. EPA5 Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms

Case7：D=3, SCS = 30 KHz, 24PRBs

F. AWGN Cell ID=674(target) 30 KHz CSI-RS Periodicity=40ms  F. EPA5 Cell ID=674(target) 30KHz CSI-RS Periodicity=40ms

Case8：D=1, SCS = 30 KHz, 48PRBs

F. AWGN Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms  F. EPA5 Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms

Case9：D=3, SCS = 30 KHz, 48PRBs

F. AWGN Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms  F. EPA5 Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms

Case10：D=1, SCS = 30 KHz, 96PRBs

F. AWGN Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms   F. EPA5 Cell ID=674(target) 15KHz CSI-RS Periodicity=40ms