3GPP TSG-RAN4 Meeting #92bis 		R4-1910951

Chongqing, China, 14th Oct – 18th Oct, 2019

Title:	Evaluation on measurement bandwidth of CSI-RS for RRM measurement requirement

Source:	CATT

Agenda Item:	8.16.1.1

Document for:	Discussion

- Introduction

In RAN #84 meeting, a new WID on RRM requirements for CSI-RS based L3 measurement was approved [1]. CSI-RS based mobility plays an important role in mobility and network deployment/optimization. And the objectives of this WI are described as follows:

- Assumptions on Synchronization - Single FFT is assumed for multiple cell measurements per frequency layer for both intra- and inter-frequency measurements. - Measurement bandwidth of CSI-RS for the minimum requirements - CSI-RS based intra-frequency and inter-frequency definition - Intra-frequency and inter-frequency measurement requirements - Evaluate and specify CSI-RS based L3 measurement accuracy - Introduce new UE measurement capability, including number of frequency layer and number of cells etc., for CSI-RS based measurement

In this contribution, we provide some evaluation on CSI-RS based measurement in order to define CSI-RS based measurement requirement, and give our proposal.

- Discussion

CSI-RS resources can be configured by network for mobility purpose, and UE shall be able to perform CSI-RSRP/CSI-RSRQ/CSI-SINR measurements on configured CSI-RS resource(s). According to TS38.214, UE can be configured up to 96 CSI-RS resources for RRM measurement. And according to TS38.331, for each CSI-RS resource, the number of PRB can be configured as 24, 48, 96, 192 or 264 in frequency domain, the density of CSI-RS can be configured as 0.5, 1 or 3 RE(s) in one PRB. Thus, the different combination of measurement bandwidth of CSI-RS and CSI-RS resource density configuration shall be evaluated to achieve a common acceptable CSI-RSRP measurement accuracy.

Based on the agreed simulation assumptions in [2], the simulation results for CSI-RSRP measurement performance are provided in [3]. According to the simulation results, it can be seen that the CSI-RSRP measurement accuracy is improved by increasing the measurement bandwidth of CSI-RS, the CSI-RS resource density and the number of measurement samples. It can be observed that more measurement samples shall be used for filtering when a sparser CSI-RS resource is configured or when a smaller measurement bandwidth is used. In order to achieve the same measurement accuracy performance, a set of CSI-RS based measurement delay requirement shall be defined based on different combination of measurement bandwidth of CSI-RS and CSI-RS resource density.

Proposal 1: CSI-RS based measurement delay can be defined based on different combination of measurement bandwidth of CSI-RS and CSI-RS resource density, which is expressed in Table 1.

Table 1: TCSI-RS_measurement_Period with different configurations

| Configuration | Measurement bandwidth [RB] | CSI-RS resource Density | TCSI-RS_measurement_Period |
| --- | --- | --- | --- |
| 0 | 24 | 1 | 10 samples |
| 1 | 48 | 1 | 5 samples |
| 2 | ≥24 | 3 | 3 samples |

- Conclusion

In this contribution, we discussed the CSI-RS based measurement bandwidth in order to define CSI-RS based measurement requirement, and give our proposal as follows.

Proposal 1: CSI-RS based measurement delay can be defined based on different combination of measurement bandwidth of CSI-RS and CSI-RS resource density, which is expressed in Table 1.

Table 1: TCSI-RS_measurement_Period with different configurations

| Configuration | Measurement bandwidth [RB] | CSI-RS resource Density | TCSI-RS_measurement_Period |
| --- | --- | --- | --- |
| 0 | 24 | 1 | 10 samples |
| 1 | 48 | 1 | 5 samples |
| 2 | ≥24 | 3 | 3 samples |

- Reference

[1]	RP-191580	New WID on RRM requirements for CSI-RS based L3 measurement, CATT

[2]	R4-1910949,	Link-level simulation assumptions for CSI-RS based measurements,	CATT

[3]	R4-1910950,	Simulation results for CSI-RSRP measurement accuracy,		CATT