3GPP RAN WG4 Meeting #92bis	R4-19010957

Chongqing, China, 14-18 October, 2019

Title:	LS on definitions of CSI-RS based intra-frequency and inter-frequency Measurement

Release:	Release 16

Work Item:	NR_CSIRS_L3meas

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

Name:				Xuhua Tao

E-mail Address:	taoxuhua@catt.cn

- Overall Description:

RAN4 has discussed the definitions of CSI-RS based intra-frequency and inter-frequency measurements in Rel-16 WI of “RRM requirements for CSI-RS based L3 measurement” and has reached the following conclusion on their definitions:

CSI-RS based intra-frequency measurement: a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

- the center frequency of CSI-RS resources on the target cell configured for measurement is the same as center frequency of CSI-RS resources on the serving cell configured for measurement, and,

- the measurement bandwidth of CSI-RS resources of target cell configured for measurement is within the measurement bandwidth of CSI-RS resources of serving cell configured for measurement, and,

- the subcarrier spacing of the two CSI-RS resources is the same.

CSI-RS based inter-frequency measurement: a measurement is defined as a CSI-RS based inter-frequency measurement provided that:

- the center frequency of CSI-RS resources on the target cell configured for measurement is not the same as center frequency of CSI-RS resources on the serving cell configured for measurement, and,

- the measurement bandwidth of CSI-RS resources of target cell configured for measurement is not within the measurement bandwidth of CSI-RS resources of serving cell configured for measurement, and,

- the subcarrier spacing of the two CSI-RS resources is not the same.

Actions:

To RAN WG2: RAN4 kindly asks RAN2 to take the above conclusions into consideration in future work.

- Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #93					18-22 December, 2019, Reno, USA

TSG RAN WG4 Meeting #94					24-28 February, 2020, Athens, Greece