3GPP TSG-RAN WG4 Meeting #92bis                                                            R4-1910984

# Chongqing, China, 14th–18th Oct., 2019

Source: 	CATT

Title: 	Discussion on HST PRACH demodulation requirement

Agenda item:	8.17.2.3

Document for:	Agreement

- Introduction

In the recent RAN4#92 meeting, a WF [1] on NR HST BS demodulation requirements was approved. There are some open issues for PRACH demodulation needing to be solved in RAN4 #92bis shown as follows:

- Frequency offset under AWGN for PRACH format 0 - Frequency offset for restricted set Type A - Option 1: 1340Hz with AWGN - Option 2: 0Hz and 1340Hz with AWGN - Frequency offset for restricted set Type B - Option 1: 1944Hz - Option 2: 1875Hz - Option 3: 2334Hz - Frequency offset under fading for PRACH format 0 - Frequency offset for restricted set Type A and B - TDLC300-100 with FO 400 Hz - Frequency offset for PRACH formats with short sequence length targeting 500km/h velocity - To align with PUSCH maximum Doppler shift - 15kHz SCS - Option 1: 1944 - Option 2: 1500Hz - Option 3: 1400Hz - Option 4: 1250Hz - Other options are not precluded - 30KhZ SCS - Option 1: 3334Hz - Option 2: 2600Hz - Option 3: 3000Hz - Option 4: 2300Hz - Other options are not precluded

This paper will give our view on frequency offset for NR HST PRACH based on evaluation results for HST PRACH.

- Discussion

According to our another contribution[2] and agreed WF[1], the detail simuation asumptions for evaluating the frequency offset and PRACH format for NR HST PRACH in Table A-1 in Annex.

The evaluation results of Format 0 for 350km/h and 500km/h are shown in Table 2-1, and the evaluation results of Format A2, A3, B4 and C2 for 500km/h are shown in Table 2-2.

Table 2-1 Evaluation results of Format 0 for 350km/h and 500km/h

| Format | Tx/Rx | frequency(GHz) | SCS | restricted set type | Ncs | v | Logical sequence index | Propagation condition | Frequency offset (Hz) | Proposed time estimation error for AWGN/TDL | SNR@ missed detection: 99% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Format 0 | 1T2R | 2.1 | 1.25kHz | type A | 15 | 0 | 384 | AWGN | 0 | 1.04us | -16.52 |
| Format 0 | 1T2R | 2.1 | 1.25kHz | type A | 15 | 0 | 384 | AWGN | 1340 | 1.05us | -16.41 |
| Format 0 | 1T2R | 2.1 | 1.25kHz | type A | 15 | 0 | 384 | TDLC300-100 low | 400 | 2.55us | -7.77 |
| Format 0 | 1T2R | 2.1 | 1.25kHz | type B | 15 | 30 | 30 | AWGN | 1875 | 1.04us | -13.07 |
| Format 0 | 1T2R | 2.1 | 1.25kHz | type B | 15 | 30 | 30 | AWGN | 1944 | 1.05us | -13.25 |
| Format 0 | 1T2R | 2.1 | 1.25kHz | type B | 15 | 30 | 30 | AWGN | 1750 | 1.06us | -13.61 |
| Format 0 | 1T2R | 2.1 | 1.25kHz | type B | 15 | 30 | 30 | TDLC300-100 low | 400 | 2.55us | -6.79 |
| Format 0 | 1T2R | 3.6 | 1.25kHz | type A | 15 | 0 | 384 | AWGN | 0 | 1.04us | -16.52 |
| Format 0 | 1T2R | 3.6 | 1.25kHz | type A | 15 | 0 | 384 | AWGN | 2334 | 1.05us | -16.27 |
| Format 0 | 1T2R | 3.6 | 1.25kHz | type A | 15 | 0 | 384 | TDLC300-100 low | 400 | 2.55us | -7.84 |
| Format 0 | 1T2R | 3.6 | 1.25kHz | type B | 15 | 30 | 30 | AWGN | 3334 | 1.04us | -7.65 |
| Format 0 | 1T2R | 3.6 | 1.25kHz | type B | 15 | 30 | 30 | AWGN | 3000 | 1.05us | -13.48 |
| Format 0 | 1T2R | 3.6 | 1.25kHz | type B | 15 | 30 | 30 | AWGN | 2334 | 1.06us | -15.67 |
| Format 0 | 1T2R | 3.6 | 1.25kHz | type B | 15 | 30 | 30 | TDLC300-100 low | 400 | 2.55us | -6.79 |

From Table 2-1, for 1.25 kHz format 0 at restricted set type B at 2.1GHz, the SNR@ missed detection 99% at AWGN with 1944Hz, 1875Hz, and 1750Hz are -13.25dB, -13.07dB and -13.61dB, which are similar. So maximum 1944Hz for AWGN for 1.25 kHz format 0 at restricted set type B at 2.1GHz is reasonable.

Observation 1: maximum 1944Hz for AWGN for 1.25 kHz format 0 at restricted set type B at 2.1GHz is reasonable.

From Table 2-1, for 1.25 kHz format 0 at restricted set type B at 3.6GHz, the SNR@ missed detection 99% at AWGN with 3334Hz is -7.65dB, which is greater and worse than SNR@ missed detection 99% at AWGN with 3000Hz(-13.48dB). As a result, 1.25 kHz format 0 at restricted set type B at 3.6GHz is not applicable at AWGN with 3334Hz, and maximum 3000Hz for AWGN for 1.25 kHz format 0 at restricted set type B at3.6 GHz is reasonable.

Observation 2: maximum 3000Hz for AWGN for 1.25 kHz format 0 at restricted set type B at3.6 GHz is reasonable.

From Table 2-1, the evaluation results show the maximum frequency offset for 1.25 kHz format 0 at restricted set type B at2.1GHz and 3.6 GHz are reasonable.

Observation 3: maximum 1340Hz for AWGN for 1.25 kHz format 0 at restricted set type A at 2.1GHz is reasonable.

Observation 4: maximum 2334Hz for AWGN for 1.25 kHz format 0 at restricted set type A at3.6 GHz is reasonable.

Based on above discussion, we have the following proposals.

Proposal 1: the following PRACH format and maximum frequency offset are approved by RAN4.

| Format | frequency(GHz) | SCS | restricted set type | Ncs | v | Logical sequence index | Propagation condition | Maximum Frequency offset (Hz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Format 0 | 2.1 | 1.25kHz | type A | 15 | 0 | 384 | AWGN | 1340 |
| Format 0 | 2.1 | 1.25kHz | type A | 15 | 0 | 384 | TDLC300-100 low | 400 |
| Format 0 | 3.6 | 1.25kHz | type A | 15 | 0 | 384 | AWGN | 2334 |
| Format 0 | 3.6 | 1.25kHz | type A | 15 | 0 | 384 | TDLC300-100 low | 400 |
| Format 0 | 2.1 | 1.25kHz | type B | 15 | 30 | 30 | AWGN | 1944 |
| Format 0 | 2.1 | 1.25kHz | type B | 15 | 30 | 30 | TDLC300-100 low | 400 |
| Format 0 | 3.6 | 1.25kHz | type B | 15 | 30 | 30 | AWGN | 3000 |
| Format 0 | 3.6 | 1.25kHz | type B | 15 | 30 | 30 | TDLC300-100 low | 400 |

Table 2-2 Evaluation results of Format A2,A3, B4 and C2 for 500km/h

| Format | Tx/Rx | frequency(GHz) | SCS | restricted set type | Ncs | v | Logical sequence index | Propagation condition | Frequency offset (Hz) | Proposed time estimation error for AWGN/TDL | SNR@ missed detection: 99% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Format A2 | 1T2R | 2.1 | 15kHz | unrestricted | 23 | 0 | 0 | AWGN | 1944 | 0.52us | -10.19 |
| Format A2 | 1T2R | 2.1 | 15kHz | unrestricted | 23 | 0 | 0 | AWGN | 1750 | 0.53us | -10.95 |
| Format A3 | 1T2R | 2.1 | 15kHz | unrestricted | 23 | 0 | 0 | AWGN | 1944 | 0.54us | -8.46 |
| Format A3 | 1T2R | 2.1 | 15kHz | unrestricted | 23 | 0 | 0 | AWGN | 1750 | 0.55us | -8.03 |
| Format B4 | 1T2R | 2.1 | 15kHz | unrestricted | 23 | 0 | 0 | AWGN | 1944 | 0.56us | -7.06 |
| Format B4 | 1T2R | 2.1 | 15kHz | unrestricted | 23 | 0 | 0 | AWGN | 1750 | 0.57us | -10.11 |
| Format C2 | 1T2R | 2.1 | 15kHz | unrestricted | 23 | 0 | 0 | AWGN | 1944 | 0.58us | -10.21 |
| Format C2 | 1T2R | 2.1 | 15kHz | unrestricted | 23 | 0 | 0 | AWGN | 1750 | 0.59us | -11.49 |
| Format A2 | 1T2R | 3.6 | 30kHz | unrestricted | 46 | 0 | 0 | AWGN | 3334 | 0.26us | -11.98 |
| Format A2 | 1T2R | 3.6 | 30kHz | unrestricted | 46 | 0 | 0 | AWGN | 3000 | 0.27us | -12.1 |
| Format A3 | 1T2R | 3.6 | 30kHz | unrestricted | 46 | 0 | 0 | AWGN | 3334 | 0.28us | -8.26 |
| Format A3 | 1T2R | 3.6 | 30kHz | unrestricted | 46 | 0 | 0 | AWGN | 3000 | 0.29us | -10.16 |
| Format B4 | 1T2R | 3.6 | 30kHz | unrestricted | 46 | 0 | 0 | AWGN | 3334 | 0.30us | -10.55 |
| Format B4 | 1T2R | 3.6 | 30kHz | unrestricted | 46 | 0 | 0 | AWGN | 3000 | 0.31us | -12.32 |
| Format C2 | 1T2R | 3.6 | 30kHz | unrestricted | 46 | 0 | 0 | AWGN | 3334 | 0.32us | -11.78 |
| Format C2 | 1T2R | 3.6 | 30kHz | unrestricted | 46 | 0 | 0 | AWGN | 3000 | 0.33us | -12.1 |

From Table 2-2, for 15 kHz SCS at AWGN with 1944Hz, SNR@ missed detection 99% of format A3 and B4 are -8.46dB and -7.06dB respectively, greater than -10.19dB and -10.21dB that are SNR@ missed detection 99% of format A2 and C2 respectively. So for 15 kHz SCS, format A2 and C2 are more applicable.  For 15kHz SCS, SNR@ missed detection 99% of format A2 at AWGN with 1944Hz is -10.19dB, which is reasonable and slightly greater and worse than SNR@ missed detection 99% of format A2 at AWGN with 1750Hz(-10.95dB). So 1944Hz for 15 kHz SCS for 500km/h is reasonable.

Observation 5: 1944Hz for 15 kHz SCS for 500km/h is reasonable, and for 15 kHz SCS, format A2 and C2 are more applicable.

From Table 2-2, for 30kHz SCS at AWGN with 3334Hz, SNR@missed detection 99% of format A3 is -8.26dB is greater than -11.98dB, -10.55dB, and -11.78dB, which are NR@missed detection 99% of format A2, B4 and C2 respectively. So for 30kHz SCS, format A2, B4 and C2 are more applicable. For 30 kHz SCS, SNR@ missed detection 99% of format A2 at AWGN with 3334Hz is -11.98dB, which is reasonable and slightlygreater and worse than SNR@ missed detection 99% of format A2 at AWGN with 3000Hz(-12.1dB). So 3334Hz for 30 kHz SCS for 500km/h is reasonable.

Observation 6: 3334Hz for 30 kHz SCS for 500km/h is reasonable, and for 30 kHz SCS, format A2, B4 and C2 are more applicable.

Based on above discussion, we have the following proposals.

Proposal 2: the following PRACH format and maximum frequency offset are approved by RAN4.

| Format | frequency(GHz) | SCS | Ncs | v | Logical sequence index | Propagation condition | Maximum Frequency offset (Hz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Format A2,C2 | 2.1 | 15kHz | 23 | 0 | 0 | AWGN | 1944 |
| Format A2,B4,C2 | 3.6 | 30kHz | 46 | 0 | 0 | AWGN | 3334 |

- Conclusion

This paper provided evaluation results for HST. Based on our evaluation results for NR HST PRACH, we have the following observations and proposals.

Observation 1: maximum 1944Hz for AWGN for 1.25 kHz format 0 at restricted set type B at 2.1GHz is reasonable.

Observation 2: maximum 3000Hz for AWGN for 1.25 kHz format 0 at restricted set type B at3.6 GHz is reasonable.

Observation 3: maximum 1340Hz for AWGN for 1.25 kHz format 0 at restricted set type A at 2.1GHz is reasonable.

Observation 4: maximum 2334Hz for AWGN for 1.25 kHz format 0 at restricted set type A at3.6 GHz is reasonable.

Proposal 1: the following PRACH format and maximum frequency offset are approved by RAN4.

| Format | frequency(GHz) | SCS | restricted set type | Ncs | v | Logical sequence index | Propagation condition | Maximum Frequency offset (Hz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Format 0 | 2.1 | 1.25kHz | type A | 15 | 0 | 384 | AWGN | 1340 |
| Format 0 | 2.1 | 1.25kHz | type A | 15 | 0 | 384 | TDLC300-100 low | 400 |
| Format 0 | 3.6 | 1.25kHz | type A | 15 | 0 | 384 | AWGN | 2334 |
| Format 0 | 3.6 | 1.25kHz | type A | 15 | 0 | 384 | TDLC300-100 low | 400 |
| Format 0 | 2.1 | 1.25kHz | type B | 15 | 30 | 30 | AWGN | 1944 |
| Format 0 | 2.1 | 1.25kHz | type B | 15 | 30 | 30 | TDLC300-100 low | 400 |
| Format 0 | 3.6 | 1.25kHz | type B | 15 | 30 | 30 | AWGN | 3000 |
| Format 0 | 3.6 | 1.25kHz | type B | 15 | 30 | 30 | TDLC300-100 low | 400 |

Observation 5: 1944Hz for 15 kHz SCS for 500km/h is reasonable, and for 15 kHz SCS, format A2 and C2 are more applicable.

Observation 6: 3334Hz for 30 kHz SCS for 500km/h is reasonable, and for 30 kHz SCS, format A2, B4 and C2 are more applicable.

Proposal 2: the following PRACH format and maximum frequency offset are approved by RAN4.

| Format | frequency(GHz) | SCS | Ncs | v | Logical sequence index | Propagation condition | Maximum Frequency offset (Hz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Format A2,C2 | 2.1 | 15kHz | 23 | 0 | 0 | AWGN | 1944 |
| Format A2,B4,C2 | 3.6 | 30kHz | 46 | 0 | 0 | AWGN | 3334 |

- References

- R4-1910128,Way forward on NR HST BS demodulation requirements,Huawei, HiSilicon,RAN4#92

- R4-1910983, Discussion on HST PUSCH demodulation requirement, CATT, RAN4#92bis

- Annex

Table 5-1 Test Parameters for HST PACH

| Parameter | PRACH formats with long sequence length | PRACH formats with short sequence length |
| --- | --- | --- |
|  | value | value |
|  | FR1 | FR1 |
| Number of Tx | 1 | 1 |
| Number of Rx | 2 | 2 |
| Preamble format | Format 0, | Format A2, A3, B4 and C2 |
| restricted set type | restricted set type A, restricted set type B | restricted set type A, restricted set type B |
| SCS | 1.25kHz (only for Format 0) | 15kHz, 30kHz |
| Ncs | for restricted set type A: 15, for restricted set type B: 15 | 23(only for 15kHz) 46(only for 30kHz) |
| v | for restricted set type A: 0, for restricted set type B: 30 | 0 |
| Logical sequence index | for restricted set type A: 384, for restricted set type B: 30 | 0 |
| Proposed time estimation error for AWGN/TDL | 1.04usfor AWGN 2.55usforTDLC300ns-100Hz | 0.52us at AWGN for 15kHz, 0.26us at AWGN for 30kHz, 2.03us at TDLC300ns-100Hz for 15kHz, 1.77us at TDLC300ns-100Hz for 30kHz, |
| Propagation condition | AWGN, TDLC300-100， | AWGN |
| Frequency offset | - For AWGN -for 2.1GHz for restricted set type A: - 0Hz - 1340Hz for restricted set type B: - 1944Hz - 1875Hz - 1750Hz -for 3.6GHz for restricted set type A: - 0Hz - 2334Hz for restricted set type B: - 2334Hz - 3334Hz - 3000Hz 2) For fading Channel 400Hz for any fading channel for restricted set type A and restricted set type B | - For AWGN - 15kHz SCS(for 2.1GHz) - 1944Hz - 1750Hz - 30KHz SCS(for 3.6GHz) - 3334Hz - 3000Hz |
| Test metric | False alarm probability: 0.1% missed detection: 99% | False alarm probability: 0.1% missed detection: 99% |
| Carrier frequency (GHz) | 2.1GHz 3.6GHz | 2.1GHz 3.6GHz |