3GPP TSG-RAN WG4 Meeting #92bis	R4-1910995

# Chongqing, China, 14th – 18th October 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | xxxx | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for Applicability rules |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Different applicability rules are defined differently - Applicability rules for interworking requiremnets are not defined |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated column with test list if all tables to have unified format - Table 7.1.1.4-1 and 8.1.1.4-1: Removed capability on “Supported maximum number of ports across all configured NZP-CSI-RS resources per CC” because all FR2 tests are configured with 2 ports CSI-RS, which is minimum capability value for this feature. - Table 7.1.1.4-1: Added capability on “Supported maximum number of configured NZP-CSI-RS resources per CC” - Added reference to applicability rules specified in Sections 5, 6, 7 and 8 for interworking requiremnets in Sections 9 and 10. - Added placeholder for NE-DC and NR-DC capabilities in Section 9.1.1.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Applicability rules are incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.3, 5.1.1.4, 6.1.1.4, 7.1.1.3, 7.1.1.4, 8.1.1.4, 9.1.1.1, 9.1.1.2, 10.1.1.1, 10.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 5.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 5.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 5.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] |  | Test type |  |  | Test list |  |  | Applicability notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver |  | FR1 FDD | PDSCH |  | Clause 5.2.2.1.1 (Test 3-1) Clause 5.2.3.1.1 (Test 5-1) |  |  |
|  |  | FR1 TDD | PDSCH |  | Clause 5.2.2.2.1 (Test 3-1) Clause 5.2.3.2.1 (Test 5-1) |  |  |
| Alternative additional DMRS position for co-existence with LTE CRS (additionalDMRS-DL-Alt) |  | FR1 FDD | PDSCH |  | Clause 5.2.2.1.4 (Test 1-2) Clause 5.2.3.1.4 (Test 1-2) |  |  |
| Basic DL NR-NR CA operation (supportedBandCombinationList) |  | NR CA | SDR |  | Clause 5.5A.1 | 1)Up to 16 DL carriers 2)Same numerology across carrier for data/control channel at a given time |  |

#### 5.1.1.4	Applicability of requirements for mandatory UE features with capability signalling

The performance requirements in Table 5.1.1.4-1 shall apply for UEs which support mandatory UE features with capability signalling only

Table 5.1.1.4-1: Requirements applicability for mandatory features with UE capability signalling

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| 256QAM modulation scheme for PDSCH for FR1 (pdsch-256QAM-FR1) | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 1-3) Clause 5.2.3.1.1 (Test 1-3) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 1-3) Clause 5.2.3.2.1 (Test 1-3) |  |
| PDSCH mapping type B (pdsch-MappingTypeB) | FR1 FDD | PDSCH | Clause 5.2.2.1.3 Clause 5.2.3.1.3 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.3 Clause 5.2.3.2.3 |  |
| Rate-matching around LTE CRS (rateMatchingLTE-CRS) | FR1 FDD | PDSCH | Clause 5.2.2.1.4 |  |
|  | FR1 FDD | PDSCH | Clause 5.2.3.1.4 |  |
| Supported maximum number of ports across all configured NZP-CSI-RS resources per CC (maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC) | FR1 FDD | PDSCH | Clause 5.2.2.1.4 (Tests 1-1, 1-2) Clause 5.2.3.1.1 (Tests 3-1, 4-1, 5-1) Clause 5.2.3.1.4 (Tests 1-1, 1-2) | The requirements apply only in case the number of NZP-CSI-RS ports in the test case satisfies UE capability on maximum number of NZP-CSI-RS ports |
|  | FR1 TDD | PDSCH | Clause 5.2.3.2.1 (Test 3-1, 4-1, 5-1) |  |
| Supported maximum number of PDSCH MIMO layers (maxNumberMIMO-LayersPDSCH) | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Tests 2-1, 2-2, 3-1) Clause 5.2.2.1.2 Clause 5.2.3.1.1 (Tests 2-1, 2-2, 3-1, 4-1, 5-1) Clause 5.2.3.1.2 | The requirements apply only in case the PDSCH MIMO rank in the test case does not exceed UE PDSCH MIMO layers capability |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Tests 2-1, 2-2, 3-1) Clause 5.2.2.2.2 Clause 5.2.3.2.1 (Tests 2-1, 2-2, 3-1, 4-1, 5-1) Clause 5.2.3.2.2 |  |

<SKIP UNCHANGED PART>

#### 6.1.1.4	Applicability of requirements for mandatory UE features with capability signalling

The performance requirements in Table 6.1.1.4-1 shall apply for UEs which support mandatory UE features with capability signalling only.

Table 6.1.1.4-1: Requirements applicability for mandatory features with UE capability signalling

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Supported maximum number of PDSCH MIMO layers (maxNumberMIMO-LayersPDSCH) | FR1 FDD | CQI | Clause 6.2.3.1.1 .1 | The requirements apply only in case the PDSCH MIMO rank in the test case does not exceed UE PDSCH MIMO layers capability |
|  |  | PMI | Clause 6.3.3.1.2 |  |
|  |  | RI | Clause 6.4.2.1 Clause 6.4.3.1 |  |
|  | FR1 TDD | CQI | Clause 6.2.3.2.1.1 |  |
|  |  | PMI | Clause 6.3.3.2.2 |  |
|  |  | RI | Clause 6.4.2.2 Clause 6.4.3.2 |  |
| Supported maximum number of ports across all configured NZP-CSI-RS resources per CC (maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC) | FR1 FDD | PMI | Clause 6.3.2.1.1 Clause 6.3.2.1.2 Clause 6.3.3.1.1 Clause 6.3.3.1.2 | The requirements apply only in case the number of NZP-CSI-RS ports in the test case satisfies UE capability on maximum number of NZP-CSI-RS ports |
|  |  | RI | Clause 6.4.3.1 (Test 4) |  |
|  | FR1 TDD | PMI | Clause 6.3.2.2.1 Clause 6.3.2.2.2 Clause 6.3.3.2.1 Clause 6.3.3.2.2 |  |
|  |  | RI | Clause 6.4.3.2 (Test 4) |  |

<SKIP UNCHANGED PART>

#### 7.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 7.1.1.3-1 shall apply for UEs which support optional UE features only..

Table 7.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver | FR2 TDD | PDSCH | Clause 7.2.2.2.1 (Test 3-1) |  |
| Basic DL NR-NR CA operation (supportedBandCombinationList) | NR CA | SDR | Clause 7.5A.1 | 1)Up to 16 DL carriers 2)Same numerology across carrier for data/control channel at a given time |

#### 7.1.1.4	Applicability of requirements for mandatory UE features with capability signalling

The performance requirements in Table 7.1.1.4-1 shall apply for UEs which support mandatory UE features with capability signalling only.

.

Table 7.1.1.4-1: Requirements applicability for mandatory features with UE capability signalling

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Supported maximum number of PDSCH MIMO layers (maxNumberMIMO-LayersPDSCH) | FR2 TDD | PDSCH | Clause 7.2.2.2.1 (Tests from 2-1 to 2-6) | The requirements apply only in case the PDSCH MIMO rank in the test case does not exceed UE PDSCH MIMO layers capability |
| Support of PT-RS with one antenna port for DL reception (onePortsPTRS) | FR2 TDD | PDSCH | Clause 7.2 |  |
|  |  | SDR | Clause 7.5.1 Clause 7.5A.1 |  |
| PCell operation on FR2 (pCell-FR2) | FR2 TDD | SDR | Clause 7.5A.1 |  |
| Supported maximum number of configured NZP-CSI-RS resources per CC (maxConfigNumberNZP-CSI-RS-PerCC) | FR2 TDD | PDSCH | Clause 7.2 | The requirements apply only in case the number of NZP-CSI-RS resources in the test case satisfies UE capability on maximum number of NZP-CSI-RS resources |
|  |  | SDR | Clause 7.5.1 Clause 7.5A.1 |  |

<SKIP UNCHANGED PART>

#### 8.1.1.4	Applicability of requirements for mandatory UE features with capability signalling

The performance requirements in Table 8.1.1.4-1 shall apply for UEs which support mandatory UE features with capability signalling only.

.

Table 8.1.1.4-1: Requirements applicability for mandatory features with UE capability signalling

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Supported maximum number of PDSCH MIMO layers (maxNumberMIMO-LayersPDSCH) | FR2 TDD | CQI | Clause 8.2.2.2.1.1 | The requirements apply only in case the PDSCH MIMO rank in the test case does not exceed UE PDSCH MIMO layers capability |
|  |  | RI | Clause 8.4.2.2 |  |
| Support of 1 port PTRS (onePortsPTRS) | FR2 TDD | CQI | Clause 8.2 |  |
|  |  | PMI | Clause 8.3 |  |
|  |  | RI | Clause 8.4 |  |
| Supported maximum number of configured NZP-CSI-RS resources per CC (maxConfigNumberNZP-CSI-RS-PerCC) | FR2 TDD | CQI | Clause 8.2 | The requirements apply only in case the number of configured NZP-CSI-RS resource in the test case satisfies UE capability on maximum number of NZP-CSI-RS resource per CC |
|  |  | PMI | Clause 8.3 |  |
|  |  | RI | Clause 8.4 |  |

<SKIP UNCHANGED PART>

#### 9.1.1.1	Applicability of requirements for optional UE features

The performance requirements in Table 9.1.1.1-1 shall apply for UEs which support optional UE features with capability signalling only.

Table 9.1.1.1-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Support of NE-DC (ne-DC, ne- DC-BC) | TBD | TBD | TBD |  |
| Support of NR-DC (ca-ParametersNRDC) | TBD | TBD | TBD |  |

The applicability rule defined in Section 5.1.1.3 shall be applied for performance requirements in Sections 9.2B.1.1 and 9.4B.1.1.

The applicability rule defined in Section 7.1.1.3 shall be applied for performance requirements in Sections 9.2B.1.2, 9.4A.1, 9.4B.1.2 and 9.4B.1.3.

#### 9.1.1.2	Applicability of requirements for mandatory UE features with capability signalling

The applicability rule defined in Section 5.1.1.4 shall be applied for performance requirements in Sections 9.2B.1.1 and 9.4B.1.1.

The applicability rule defined in Section 7.1.1.4 shall be applied for performance requirements in Sections 9.2B.1.2, 9.4A.1, 9.4B.1.2 and 9.4B.1.3.

<SKIP UNCHANGED PART>

#### 10.1.1.1	Applicability of requirements for optional UE features

#### 10.1.1.2	Applicability of requirements for mandatory UE features with capability signalling

The applicability rule defined in Section 6.1.1.4 shall be applied for performance requirements in Sections 10.2B.1.1, 10.3B.1.1 and 10.4B.1.1.

The applicability rule defined in Section 8.1.1.4 shall be applied for performance requirements in Sections 10.2B.1.2, 10.3B.1.2 and 10.4B.1.2.

END OF CHANGE