3GPP TSG-RAN WG4 Meeting #92bis	R4-1910996

# Chongqing, China, 14th – 18th October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | xxxx | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections for PDSCH RMC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Typos in multiple tables with PDSCH RMC |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Table A.3.2.1.1-7: Corrected number of “Allocated slots per 2 frames” for LTE-NR coexistance tests from 19 to 16. - Table A.3.2.1.2-1: Corrected slot indexing for parameter “Transport block CRC per Slot” - Tables A.3.2.1.1-2, A.3.2.2.2-1, A.3.2.2.2-2, A.3.2.2.2-4, A.3.2.2.5-7, A.3.2.2.5-8: Removed or added spaces and removed redundant brackets - Table A.3.2.2.2-5, A.3.2.2.4-1, A.3.2.2.5-2: Corrected slot indexing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDSCH RMC configuration is incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.2.1.1, A.3.2.1.2, A.3.2.2.2, A.3.2.2.4, A.3.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

## A.3.2	Reference measurement channels for PDSCH performance requirements

For PDSCH reference channels if more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit).

### A.3.2.1	FDD

#### A.3.2.1.1	Reference measurement channels for SCS 15 kHz FR1

<SKIP UNCHANGED PART>

Table A.3.2.1.1-2: PDSCH Reference Channel for FDD (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-2.1 FDD | R.PDSCH.1-2.2 FDD | R.PDSCH.1-2.3 FDD | R.PDSCH.1-2.4 FDD |  |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |  |
| Number of allocated resource blocks | PRBs | 52 | 52 | 52 | 52 |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 |  |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 13 | 13 | 13 | 13 |  |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 |  |
| Number of MIMO layers |  | 1 | 2 | 3 | 4 |  |
| Number of DMRS REs |  | 12 | 12 | 24 | 24 |  |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | Bits | 13064 | 26120 | 35856 | 48168 |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | CBs | 2 | 4 | 5 | 6 |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 10, 11 | Bits | 26208 | 52416 | 71136 | 94848 |  |
| For Slots i = 1,…, 9, 12, …, 19 | Bits | 27456 | 54912 | 74880 | 99840 |  |
| Max. Throughput averaged over 2 frames | Mbps | 12.411 | 24.814 | 34.063 | 45.760 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

<SKIP UNCHANGED PART>

Table A.3.2.1.1-7: PDSCH Reference Channel for FDD LTE-NR coexistence scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-7.1 FDD | R.PDSCH.1-7.2 FDD |  |  |  |
| Channel bandwidth | MHz | 10 | 10 |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |
| Number of allocated resource blocks | PRBs | 52 | 52 |  |  |  |
| Number of consecutive PDSCH symbols |  | 9 | 11 |  |  |  |
| Allocated slots per 2 frames | Slots | 16 | 16 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 4 | 4 |  |  |  |
| Modulation |  | QPSK | QPSK |  |  |  |
| Target Coding Rate |  | 0.30 | 0.30 |  |  |  |
| Number of MIMO layers |  | 1 | 1 |  |  |  |
| Number of DMRS REs |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 18 | 18 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,10,15 | Bits | N/A | N/A |  |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} for i from {0,…,19} | Bits | 2472 | 3240 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,10,15 | Bits | N/A | N/A |  |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} for i from {0,…,19} | Bits | 16 | 16 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,10,15 | CBs | N/A | N/A |  |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} for i from {0,…,19} | CBs | 1 | 1 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,10,15 | Bits | N/A | N/A |  |  |  |
| For Slots i = 11 | Bits | 7760 | 10256 |  |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} for i from {1,…, 9, 12, …, 19} | Bits | 8384 | 10880 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 1.978 | 2.592 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: No user data is scheduled on slots with LTE PBCH/PSS/SSS |  |  |  |  |  |  |

<SKIP UNCHANGED PART>

#### A.3.2.1.2	Reference measurement channels for SCS 30 kHz FR1

Table A.3.2.1.2-1: PDSCH Reference Channel for FDD (64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-1.1 FDD |  |  |  |  |
| Channel bandwidth | MHz | 20 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Number of allocated resource blocks | PRBs | 51 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |
| Allocated slots per 2 frames | Slots | 39 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 19 |  |  |  |  |
| Modulation |  | 64QAM |  |  |  |  |
| Target Coding Rate |  | 0.51 |  |  |  |  |
| Number of MIMO layers |  | 2 |  |  |  |  |
| Number of DMRS REs |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For Slots i = 1,…, 39 | Bits | 40976 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For Slots i = 1,…, 39 | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A |  |  |  |  |
| For Slots i = 1,…, 39 | CBs | 5 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For Slots i = 20, 21 | Bits | 77112 |  |  |  |  |
| For Slots i = 1,…, 19, 22, …, 39 | Bits | 80784 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 79.903 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

<SKIP UNCHANGED PART>

### A.3.2.2	TDD

#### A.3.2.2.1	Reference measurement channels for SCS 15 kHz FR1

#### A.3.2.2.2	Reference measurement channels for SCS 30 kHz FR1

Table A.3.2.2.2-1: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 and FR1.30-1A (QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-1.1 TDD | R.PDSCH.2-1.2 TDD | R.PDSCH.2-1.3 TDD |  |  |
| Channel bandwidth | MHz | 40 | 40 | 40 |  |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 |  |  |
| Allocated resource blocks | PRBs | 106 | 6 | 106 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | N/A |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 7 |  |  |
| Allocated slots per 2 frames |  | 31 | 31 | 27 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 4 | 4 | 4 |  |  |
| Modulation |  | QPSK | QPSK | QPSK |  |  |
| Target Coding Rate |  | 0.30 | 0.30 | 0.30 |  |  |
| Number of MIMO layers |  | 1 | 1 | 1 |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 | N/A |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 18 | 12 | 12 |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 2664 | 144 | N/A |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 8064 | 480 | 4608 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 16 | 16 | N/A |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 | 16 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 1 | 1 | N/A |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 1 | 1 | 1 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 20, 21 | Bits | 25440 | 1512 | 13992 |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 8904 | 504 | N/A |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 26712 | 1584 | 15264 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 11.419 | 0.677 | 6.221 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-2: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-2.1 TDD | R.PDSCH.2-2.2 TDD | R.PDSCH.2-2.3 TDD | R.PDSCH.2-2.4 TDD |  |
| Channel bandwidth | MHz | 40 | 40 | 40 | 40 |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |  |
| Allocated resource blocks | PRBs | 106 | 106 | 106 | 106 |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | 4 | 4 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 |  |
| Allocated slots per 2 frames |  | 31 | 31 | 31 | 31 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 13 | 13 | 13 | 13 |  |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 |  |
| Number of MIMO layers |  | 1 | 2 | 3 | 4 |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 | 12 | 12 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 24 | 24 |  |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 8456 | 16896 | 22032 | 29192 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 26632 | 53288 | 73776 | 98376 |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | 24 | 24 | 24 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6}for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 2 | 3 | 3 | 4 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 4 | 7 | 9 | 12 |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 20, 21 | Bits | 53424 | 106848 | 144008 | 193344 |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 17808 | 35616 | 45792 | 61056 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 55968 | 111936 | 152640 | 203520 |  |
| Max. Throughput averaged over 2 frames | Mbps | 37.644 | 75.318 | 104.719 | 138.646 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

<SKIP UNCHANGED PART>

Table A.3.2.2.2-4: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (256QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-4.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 31 |  |  |  |  |
| MCS table |  | 256QAM |  |  |  |  |
| MCS index |  | 24 |  |  |  |  |
| Modulation |  | 256QAM |  |  |  |  |
| Target Coding Rate |  | 0.82 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Maximum number of HARQ transmissions |  | 4 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 29192 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 92200 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 4 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 11 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slots i = 20, 21 | Bits | 106848 |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 35616 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 111936 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 130.308 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-5: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-2

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-5.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,39} |  | 8 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 31 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |
| Target Coding Rate |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,39} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,39} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,39} | Bits | 5376 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,39} | Bits | 8456 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,39} | CBs | 1 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,39} | CBs | 2 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i = 20, 21 | Bits | 26712 |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,39} | Bits | 17808 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19,22,…,39} | Bits | 27984 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 11.875 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

<SKIP UNCHANGED PART>

#### A.3.2.2.3	Reference measurement channels for SCS 60 kHz FR1

#### A.3.2.2.4	Reference measurement channels for SCS 60 kHz FR2

Table A.3.2.2.4-1: PDSCH Reference Channel for TDD UL-DL pattern FR2.60-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.4-1.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 50 |  |  |  |  |
| Subcarrier spacing | kHz | 60 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 79} |  | 10 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,79} |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  | 59 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 2 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 79} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,79} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,79} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 79} | Bits | 25608 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,79} | Bits | 34816 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,79} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 79} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,79} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,79} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 79} | CBs | 4 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,79} | CBs | 5 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,79} | Bits | N/A |  |  |  |  |
| For Slot i = 40, 41 | Bits | 69960 |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {4,…, 79} | Bits | 54912 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,39,42,…,79} | Bits | 73128 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 93.499 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

#### A.3.2.2.5	Reference measurement channels for SCS 120 kHz FR2

<SKIP UNCHANGED PART>

Table A.3.2.2.5-2: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-2.1 TDD | R.PDSCH.5-2.2 TDD | R.PDSCH.5-2.3 TDD |  |  |
| Channel bandwidth | MHz | 100 | 100 | 200 |  |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 |  |  |
| Allocated resource blocks | PRBs | 66 | 66 | 132 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 | 9 | 9 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 13 | 13 | 13 |  |  |
| Allocated slots per 2 frames |  | 127 | 127 | 127 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 13 | 13 | 13 |  |  |
| Modulation |  | 16QAM | 16QAM | 16QAM |  |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 |  |  |
| Number of MIMO layers |  | 1 | 2 | 2 |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 12 | 12 | 12 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 12 | 12 | 12 |  |  |
| Overhead for TBS determination |  | 6 | 6 | 6 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 11272 | 22536 | 45096 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 17424 | 34816 | 69672 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24 | 24 | 24 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 2 | 3 | 6 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | CBs | 3 | 5 | 9 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 80, 81 | Bits | 36564 | 69960 | 139920 |  |  |
| For Slots i = 82, 83 | Bits | 34980 | 73128 | 146256 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24420 | 48840 | 97680 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79,84,…,159} | Bits | 36564 | 73128 | 146256 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 100.799 | 201.434 | 403.096 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

<SKIP UNCHANGED PART>

Table A.3.2.2.5-7: PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR2.120-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-7.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 63 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs (Note 3) |  | 624 |  |  |  |  |
| Overhead for TBS determination |  | 46 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i = 80 | Bits | 14344 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 14344 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i = 80 | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | CBs | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | CBs | N/A |  |  |  |  |
| For Slot i = 80 | CBs | 2 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | CBs | 2 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i = 80 | Bits | 28776 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 30360 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 45.1836 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |

Table A.3.2.2.5-8: PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR2.120-2 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-8.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 59 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs (Note 3) |  | 624 |  |  |  |  |
| Overhead for TBS determination |  | 46 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,8) =1 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i = 80 | Bits | 14344 |  |  |  |  |
| For Slot i, if mod(i, 8) = {0,4,5} for i from {1,…,79,82,…,159} | Bits | 14344 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,8) =1 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i = 80 | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 8) = {0,4,5} for i from {1,…,79,82,…,159} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | CBs | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,8) =1 for i from {0,…,159} | CBs | N/A |  |  |  |  |
| For Slot i = 80 | CBs | 2 |  |  |  |  |
| For Slot i, if mod(i, 8) = {0,4,5} for i from {1,…,79,82,…,159} | CBs | 2 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,8) =1 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i = 80 | Bits | 28776 |  |  |  |  |
| For Slot i, if mod(i, 8) = {0,4,5} for i from {1,…,79,82,…,159} | Bits | 30360 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 42.3148 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |

END OF CHANGE