3GPP TSG-RAN WG4 Meeting #92bis                                                                     R4-1911022

Chongqing, China, 14th – 18th October, 2019

Agenda Item:	8.16.1

Source:	Intel Corporation

Title:	Simulation results for CSI-RS L3 measurement for FR1

Document for:	Discussion

# Introduction

In this contribution, we provide the simulation results for CSI-RS L3 measurement accuracy for FR1.

# Simulation assumption

The draft simulation assumption for CSI-RS based L3 measurement is agreed as follows, where the red character is our simulation parameter.

Table 1: General parameters

| Simulation parameters | Comments/values |  |
| --- | --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 4 GHz | 30 GHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Yes / Yes | Yes / Yes |
| DRX | No | No |
| BS transmit antennas for SS blocks | 1 tx or single layer transmissions | 1 tx or single layer transmissions |
| UE receive antennas | Case A: 2 rx (uncorrelated with equal gain, no rx beamforming); Case B: 2 rx with rx beamforming Note1 | Case A: 2 rx (uncorrelated with equal gain, no rx beamforming); Case B: 2 rx with rx beamforming Note1 |
| Measurement period (in number of samlpes) | 1, 3, 5, 10; other number of samples may also be studied upon a need | 1, 3, 5, 10; other numbers of samples may also be studied upon a need |
| NOTE1: For Case B, companies are encouraged to state their assumptions on beamforming and rx antenna configuration together with the provided simulation results |  |  |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| SNR (in Case B, SNR includes beamforming gain) | dB | 6 dB | 1 dB |
| Es/IoT (calculated from SNRs) | dB | 2.46 dB | -5.97 dB |
| Propagation conditions | - | For 4 GHz: AWGN, EPA5, ETU30 (for 15 kHz SCS), ETU70 (for 15 kHz SCS); For 30 GHz:, optional TDL-A/ /C Note (delay spread: 30 ns; UE speed: 3 km/h and 30 km/h) |  |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned |  |  |  |

Table 3: CSI-RS configuration parameters

| Parameter | 4 GHz | 30 GHz |
| --- | --- | --- |
| CSI-RS bandwidth | 24 PRBs, 96 PRBs; optional: 48 PRBs, 192 PRBs (not for 60 kHz), 264 PRBs (not for 60 kHz) | 24 PRBs, 96 PRBs; optional: 48 PRBs, 192 PRBs, and 264 PRBs |
| CSI-RS SCS | 15 kHz, 30 kHz, 60 kHz | 60 kHz; 120 kHz |
| Periodicity | 5 ms (optional); 40 ms | 5 ms (optional); 40 ms |
| CSI-RS configuration: <X,D,N,(Y,Z),CDM>, where: X=number of CSI-RS ports, D=density [RE/RB/port], N=number of OFDM symbols in the same slot, Y=adjacent in frequency REs in PRB, Z=adjacent in time REs in PRB | <1,1,1,N/A,noCDM> <1,3,1,N/A,noCDM> | <1,1,1,N/A,noCDM> <1,3,1,N/A,noCDM> |

# Simulation results of measurement accuracy for FR1

In this stage, we provide our RSRP delta for FR1 for EPA and ETU channel for 24RB and 96 RB based on 1/3/5/10 samples respectively.

Table 4: RSRP delta for EPA5 channel (dB)

|  | 1 sample | 3 sample | 5 sample | 10 samples |
| --- | --- | --- | --- | --- |
| 24RB, D=1 | 5.91 | 4.13 | 3.25 | 2.81 |
| 24RB, D=3 | 4.18 | 2.8723 | 2.41 | 1.86 |
| 96RB, D=1 | 3.86 | 2.5795 | 2.12 | 1.36 |
| 96RB, D=3 | 2.59 | 2.10 | 1.35 | 0.95 |

Table 5: RSRP delta for ETU30 channel(dB)

|  | 1 sample | 3 sample | 5 sample | 10 samples |
| --- | --- | --- | --- | --- |
| 24RB, D=1 | 6.34 | 4.16 | 3.49 | 2.98 |
| 24RB, D=3 | 4.46 | 2.88 | 2.31 | 1.99 |
| 96RB, D=1 | 5.21 | 3.54 | 2.55 | 2.13 |
| 96RB, D=3 | 2.92 | 2.15 | 1.56 | 1.55 |

To define the CSI-RS based measurement performance accuracy test, the CSI-RS configuration may need to be chosen that CSI-RS based measurement performance will not be worse than that of SSB based measurement. For example, there are 4 scenarios:

  - Measurement delay of CSI-RS based RRM is similar as SSB based RRM, but higher accuracy is achieved

  - Measurement accuracy of CSI-RS based RRM is similar as SSB based RRM, but short measurement period is achieved

  - Measurement accuracy and delay of CSI-RS are both similar as SSB

  - Measurement accuracy and delay of CSI-RS are both better than SSB

If CSI-RS based RRM performance can’t be similar or better than that of SSB, there is no need to define the requirement.

Proposal 1: CSI-RS configuration to define the RRM performance requirement needs to be researched, e.g. density, bandwidth, evaluation time.

From the simulation results, it is shown that for 24RB, D=1 case, the accuracy delta are 3.49dB and 2.98dB for 5 samples and 10 samples respectively. When RF margin is considered, the total accuracy delta are 5.49dB and 4.98dB which are worse than that of SSB. Therefore, no measurement requirement is defined for 24RB with D=1 case.

Proposal 2: no measurement requirement is defined for 24RB with D=1 case.

# Conclusion

In this contribution, we provide our simulation results for L1-RSRP measurement accuracy, the following conclusion can be drawn:

Proposal 1: CSI-RS configuration to define the RRM performance requirement needs to be researched, e.g. density, bandwidth, evaluation time.

Proposal 2: no measurement requirement is defined for 24RB with D=1 case.

# References