3GPP TSG-RAN WG4 Meeting #92Bis	R4-1911025

Chongqing, China, 14 – 18 October 2019

Source:	Intel Corporation

Title:	On NR-U Channel Raster

Agenda item:	8.1.1.3

Document for:	Approval

# Background

In RAN4#92, there was WF [1] where general down selection principles were agreed. However, the major decision is that down selection is based on 15 kHz granularity and no further principles were not discussed. In this paper, we share our view on the down selection principles based on the properties channel raster entries need to be met to improve network and UE performance.

# Discussion

Non-coexisting scenario

The WF says

- Down selection Nref is based on R15 NR channel raster of 15 kHz granularity (Table 5.4.2.1-1 in 38.104 for frequency range 3000 – 24250 MHz)

- Further confirmation on possibility of the guarantee of scenario of non-coexistence with WiFi

- Further confirmation with RAN1 with common understanding on WID scope of considering non-coexisting scenario with WiFi

We don’t have a strong view on the current agreement now. However, RAN1 specification, i.e., TS 38.213, allows only limited number of SSB CORESET0 offsets, i.e., 4-bit in pdcch-ConfigSIB1, and even all 16 indices might not be usable because some of indices are reserved. If RAN4 tries to introduce more than 16 offset values, it would huge impact on RAN1 specification. Considering there are three meeting left before close NR-U WI, it is proposed to check with RAN1.

Observation #1: RAN1 specification, i.e., TS 38.213, allows only limited number of SSB CORESET0 offsets, i.e., 4-bit in pdcch-ConfigSIB1, and some of indices are reserved.

Proposal #1: It is proposed to check with RAN1 for advice or suggestion about the SSB CORSET0 offsets and RAN1 specification impact.

Coexisting scenario

In WF, general down selection principles were agreed for applicable single carrier channel bandwidths. However, the principles are quite general. In this paper, we discuss other aspects need to be considered for NR-U channel raster down selection.

- Implementation complexity and coexistence performance perspective

In case of CA operation, PRB grid alignment plays an important role in terms of implementation complexity and coexistence perspective along with a wideband carrier operation. For example, let’s consider the scenario that gNB1 is serving CA by 2 x 20 MHz and gNB2 is serving one wideband carrier by 1 x 40 MHz. The illustration of the scenario is in figure 1.

Figure 1. Illustration of PRB grid alignment and mis-alignment between 2CC CA and a single wideband carrier

For gNB1 where CA is being serviced by 2 x 20 MHz and let’s assume the PRB grid of 1st CC and 2nd CC is not aligned each other, i.e., solid line in the 1st CC and arrow-head line in the 2nd CC. The dash lines of the 2nd CC in gNB1 indicate the PRB grid aligned with the 1st CC’s PRB grid indicated by solid lines. The arrow-head lines in the 2nd CC in gNB1 indicate its PRB grid which are deviated from the PRB grid aligned for 1st CC. So there is no PRB grid alignment between the 1st CC and 2nd CC.

Now let’s assume there is a neighbour cell served by gNB2 where 40 MHz single wideband carrier is being serviced where the PRB grid of this wideband carrier is aligned with the 1st CC in gNB1. Since PRB grid of the 2nd CC in gNB1 is not aligned with the 1st CC in gNB1, and the PRB grid of the wideband carrier, there would be coexistence between 2nd CC in gNB1 and the 2nd half of the wideband carrier. This situation is marked by red-box in the figure 1.

The coexistence issue can be resolved by PRB grid alignment between CA and a wideband carrier. Therefore it is important to consider both CA and a wideband carrier scenario when down select channel raster entry. Another benefit of PRB grid alignment is it simplifies UE implementation, i.e., a single FFT operation.

Observation #2: There is coexistence issue between CA and a wideband carrier if channel raster is not PRB grid aligned.

Proposal #2: Channel raster should allow PRB grid alignment between CCs in contiguous CA as well as wideband carrier (>20 MHz) and narrow band (20 MHz) CC.

- Spectrum efficiency perspective

The subcarrier offset (kSSB) represents an offset from subcarrier 0 of the Common Resource Block identified by offsetToPointA to subcarrier #0 of the SS/PBCH. In the case of FR1, kSSB is based on a 15 kHz subcarrier spacing while the SS/PBCH and subCarrierSpacingCommon can use subcarrier spacing of 30 kHz unless there is indication by higher layers (RAN1 agreements are in appendix). The kSSB requires a range from 0 to 23.

In case kSSB is not equal to 0, there will be non-usable PRBs and this is resulting 1 PRB less that can be used for the system when SSBs are transmitted. By careful selection of entry points, this situation can be completely avoidable. Therefore spectrum efficiency is maximized by selecting channel raster with kSSB to be 0. The figure 1 illustrates the situation.

Figure 1. Illustration of subcarrier offset (kSSB) and usable PRBs.

There is another advantage of having kSSB to be 0. To indicate all 24 offsets, 5-bit has been assigned in PBCH payload. In case kSSB index is already known to UE, i.e., kSSB = 0, then PBCH decoding performance is improved by 5-bit.

Observation #3: When kSSB is not equal to 0, there will be non-usable PRBs which decreases spectrum efficiency of the system. However, this can be completely avoided by careful selection of entry points by choosing kSSB to be 0, and spectral efficiency is maximized.

Observation #4: Channel raster entries having the same kSSB improves PBCH decoding performance for UE by 5-bit.

Proposal #3: It is proposed that channel raster to be down selected with kSSB = 0 for each entry.

- Channel raster entry with different numerology

In TS 38.104 subclause 5.4.2.2 specifies NR channel raster to resource element mapping and there is a sentence that “The mapping must apply to at least one numerology supported by the BS.” The excerpt is below.

#### 5.4.2.2 Channel raster to resource element mapping The mapping between the RF reference frequency on the channel raster and the corresponding resource element is given in table 5.4.2.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the channel and applies to both UL and DL. The mapping must apply to at least one numerology supported by the BS. Table 5.4.2.2-1: Channel Raster to Resource Element Mapping Resource element index; 0; 6. Physical resource block number. , , are as defined in TS 38.211 [9].

The current specification allows BS that supports multiple numerologies to satisfy ARFCN requirement with just one or more numerology. In case of unlicensed bands, multiple operators may use different numerology as reference and create non-uniform channel placement which is causing inter-operability issues.

For example, 106 RB is defined with 15 kHz SCS and 51 RB with 30 kHz SCS for 20 MHz CBW. Based on the channel raster to resource element mapping given in Table 5.4.2.2-1, the center frequency with 15 kHz SCS is located in subcarrier #0 in the 53rd RB, while the center frequency with 30 kHz SCS is located in the subcarrier #6 in the 26th RB. Figure 2 illustrates this example.

Figure 2. Illustration of channel raster position with 15 and 30 kHz SCS for 20 MHz CBW (not scaled)

This example shows that channel raster entry for 15 kHz and 30 kHz could not be aligned for a given BW and care should be taken when channel raster points are being selected to avoid non-uniform channel placement causing inter-operability issues.

Observation #5: Different channel raster points could be possible for different subcarrier spacing, i.e., 15 and 30 kHz SCS, for a given CBW, i.e., 20 MHz, and this creates non-uniform channel placement causing inter-operability issues.

Proposal #4: Channel raster entry should be chosen carefully so that 15 and 30 kHz SCS has the same entry point for a given CBW.

In appendix, we provide channel raster to meet all the properties in proposals #2 – #4.

# Summary

In this paper we made the following observations and proposals:

Observation #1: RAN1 specification, i.e., TS 38.213, allows only limited number of SSB CORESET0 offsets, i.e., 4-bit in pdcch-ConfigSIB1, and some of indices are reserved.

Observation #2: There is coexistence issue between CA and a wideband carrier if channel raster is not PRB grid aligned.

Observation #3: When kSSB is not equal to 0, there will be non-usable PRBs which decreases spectrum efficiency of the system. However, this can be completely avoided by careful selection of entry points by choosing kSSB to be 0, and spectral efficiency is maximized.

Observation #4: Channel raster entries having the same kSSB improves PBCH decoding performance for UE by 5-bit.

Observation #5: Different channel raster points could be possible for different subcarrier spacing, i.e., 15 and 30 kHz SCS, for a given CBW, i.e., 20 MHz, and this creates non-uniform channel placement causing inter-operability issues.

Proposal #1: It is proposed to check with RAN1 for advice or suggestion about the SSB CORSET0 offsets and RAN1 specification impact.

Proposal #2: Channel raster should allow PRB grid alignment between CCs in contiguous CA as well as wideband carrier (>20 MHz) and narrow band (20 MHz) CC.

Proposal #3: It is proposed that channel raster to be down selected with kSSB = 0 for each entry.

Proposal #4: Channel raster entry should be chosen carefully so that 15 and 30 kHz SCS has the same entry point for a given CBW.

# Reference

[1]	R4-1910593, “WF for NR-U channel raster”, Ericsson, Ljubljana, August 2019

# Appendix

RAN1 agreements:

RAN1 #AH1901 (21st January – 25th January, 2019) Agreement: - UE assumes 30 KHz SCS for SS/PBCH block for 5GHz band and 6GHz band if the SCS is not indicated by higher layers. - Support configuration by higher layers of 15 KHz or 30 KHz SCS for SS/PBCH block. - Include this agreement in a LS to RAN4 (cc RAN2) for inclusion in specs managed by RAN4. RAN1 #96 (8th – 12th April, 2019) Agreement: - The SCS for all SSBs and Coreset #0 on a carrier is always the same for operation of NR in unlicensed spectrum. - CORESET #0 frequency domain resource configuration should be 48 RBs for 30KHz SCS and 96 RBs for 15KHz SCS.

Proposed channel raster entry:

| LTE Chn Raster [kHz] | BW [MHz | Data SCS [kHz] | NR Chn Raster [kHz] | NR-ARFCN | N_PRB | k_SSB | RB Offset (Left) | RB Offset (Right) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5160000 | 20/60/100 | 15 | 5160000 | 744000 | 104 | 0 | 0 | 64 |
| 5160000 | 20/60/100 | 30 | 5160000 | 744000 | 50 | 0 | 3 | 27 |
| 5170000 | 40/80 | 15 | 5169900 | 744660 | 214 | 0 | 0 | 174 |
| 5170000 | 40/80 | 30 | 5169900 | 744660 | 105 | 0 | 3 | 82 |
| 5180000 | 20/60/100 | 15 | 5180160 | 745344 | 104 | 0 | 0 | 64 |
| 5180000 | 20/60/100 | 30 | 5180160 | 745344 | 50 | 0 | 3 | 27 |
| 5190000 | 40/80 | 15 | 5190060 | 746004 | 214 | 0 | 0 | 174 |
| 5190000 | 40/80 | 30 | 5190060 | 746004 | 105 | 0 | 3 | 82 |
| 5200000 | 20/60/100 | 15 | 5199960 | 746664 | 104 | 0 | 2 | 62 |
| 5200000 | 20/60/100 | 30 | 5199960 | 746664 | 50 | 0 | 0 | 30 |
| 5210000 | 40/80 | 15 | 5209860 | 747324 | 214 | 0 | 2 | 172 |
| 5210000 | 40/80 | 30 | 5209860 | 747324 | 105 | 0 | 0 | 85 |
| 5220000 | 20/60/100 | 15 | 5220120 | 748008 | 104 | 0 | 2 | 62 |
| 5220000 | 20/60/100 | 30 | 5220120 | 748008 | 50 | 0 | 0 | 30 |
| 5230000 | 40/80 | 15 | 5230020 | 748668 | 214 | 0 | 2 | 172 |
| 5230000 | 40/80 | 30 | 5230020 | 748668 | 105 | 0 | 0 | 85 |
| 5240000 | 20/60/100 | 15 | 5239920 | 749328 | 104 | 0 | 4 | 60 |
| 5240000 | 20/60/100 | 30 | 5239920 | 749328 | 50 | 0 | 1 | 29 |
| 5250000 | 40/80 | 15 | 5250000 | 750000 | 216 | 0 | 4 | 172 |
| 5250000 | 40/80 | 30 | 5250000 | 750000 | 106 | 0 | 1 | 85 |
| 5260000 | 20/60/100 | 15 | 5260080 | 750672 | 104 | 0 | 4 | 60 |
| 5260000 | 20/60/100 | 30 | 5260080 | 750672 | 50 | 0 | 1 | 29 |
| 5270000 | 40/80 | 15 | 5269980 | 751332 | 214 | 0 | 4 | 170 |
| 5270000 | 40/80 | 30 | 5269980 | 751332 | 105 | 0 | 1 | 84 |
| 5280000 | 20/60/100 | 15 | 5279880 | 751992 | 104 | 0 | 6 | 58 |
| 5280000 | 20/60/100 | 30 | 5279880 | 751992 | 50 | 0 | 2 | 28 |
| 5290000 | 40/80 | 15 | 5290140 | 752676 | 214 | 0 | 4 | 170 |
| 5290000 | 40/80 | 30 | 5290140 | 752676 | 105 | 0 | 1 | 84 |
| 5300000 | 20/60/100 | 15 | 5300040 | 753336 | 104 | 0 | 6 | 58 |
| 5300000 | 20/60/100 | 30 | 5300040 | 753336 | 50 | 0 | 2 | 28 |
| 5310000 | 40/80 | 15 | 5309940 | 753996 | 214 | 0 | 6 | 168 |
| 5310000 | 40/80 | 30 | 5309940 | 753996 | 105 | 0 | 2 | 83 |
| 5320000 | 20/60/100 | 15 | 5319840 | 754656 | 104 | 0 | 0 | 64 |
| 5320000 | 20/60/100 | 30 | 5319840 | 754656 | 50 | 0 | 3 | 27 |
| 5330000 | 40/80 | 15 | 5330100 | 755340 | 214 | 0 | 6 | 168 |
| 5330000 | 40/80 | 30 | 5330100 | 755340 | 105 | 0 | 2 | 83 |
| 5340000 | 20/60/100 | 15 | 5340000 | 756000 | 104 | 0 | 0 | 64 |
| 5340000 | 20/60/100 | 30 | 5340000 | 756000 | 50 | 0 | 3 | 27 |
| 5480000 | 20/60/100 | 15 | 5480040 | 765336 | 104 | 0 | 6 | 58 |
| 5480000 | 20/60/100 | 30 | 5480040 | 765336 | 50 | 0 | 2 | 28 |
| 5490000 | 40/80 | 15 | 5489940 | 765996 | 214 | 0 | 6 | 168 |
| 5490000 | 40/80 | 30 | 5489940 | 765996 | 105 | 0 | 2 | 83 |
| 5500000 | 20/60/100 | 15 | 5499840 | 766656 | 104 | 0 | 0 | 64 |
| 5500000 | 20/60/100 | 30 | 5499840 | 766656 | 50 | 0 | 3 | 27 |
| 5510000 | 40/80 | 15 | 5510100 | 767340 | 214 | 0 | 6 | 168 |
| 5510000 | 40/80 | 30 | 5510100 | 767340 | 105 | 0 | 2 | 83 |
| 5520000 | 20/60/100 | 15 | 5520000 | 768000 | 104 | 0 | 0 | 64 |
| 5520000 | 20/60/100 | 30 | 5520000 | 768000 | 50 | 0 | 3 | 27 |
| 5530000 | 40/80 | 15 | 5529900 | 768660 | 214 | 0 | 0 | 174 |
| 5530000 | 40/80 | 30 | 5529900 | 768660 | 105 | 0 | 3 | 82 |
| 5540000 | 20/60/100 | 15 | 5540160 | 769344 | 104 | 0 | 0 | 64 |
| 5540000 | 20/60/100 | 30 | 5540160 | 769344 | 50 | 0 | 3 | 27 |
| 5550000 | 40/80 | 15 | 5550060 | 770004 | 214 | 0 | 0 | 174 |
| 5550000 | 40/80 | 30 | 5550060 | 770004 | 105 | 0 | 3 | 82 |
| 5560000 | 20/60/100 | 15 | 5559960 | 770664 | 104 | 0 | 2 | 62 |
| 5560000 | 20/60/100 | 30 | 5559960 | 770664 | 50 | 0 | 0 | 30 |
| 5570000 | 40/80 | 15 | 5569860 | 771324 | 214 | 0 | 2 | 172 |
| 5570000 | 40/80 | 30 | 5569860 | 771324 | 105 | 0 | 0 | 85 |
| 5580000 | 20/60/100 | 15 | 5580120 | 772008 | 104 | 0 | 2 | 62 |
| 5580000 | 20/60/100 | 30 | 5580120 | 772008 | 50 | 0 | 0 | 30 |
| 5590000 | 40/80 | 15 | 5590020 | 772668 | 214 | 0 | 2 | 172 |
| 5590000 | 40/80 | 30 | 5590020 | 772668 | 105 | 0 | 0 | 85 |
| 5600000 | 20/60/100 | 15 | 5599920 | 773328 | 104 | 0 | 4 | 60 |
| 5600000 | 20/60/100 | 30 | 5599920 | 773328 | 50 | 0 | 1 | 29 |
| 5610000 | 40/80 | 15 | 5610000 | 774000 | 216 | 0 | 4 | 172 |
| 5610000 | 40/80 | 30 | 5610000 | 774000 | 106 | 0 | 1 | 85 |
| 5620000 | 20/60/100 | 15 | 5620080 | 774672 | 104 | 0 | 4 | 60 |
| 5620000 | 20/60/100 | 30 | 5620080 | 774672 | 50 | 0 | 1 | 29 |
| 5630000 | 40/80 | 15 | 5629980 | 775332 | 214 | 0 | 4 | 170 |
| 5630000 | 40/80 | 30 | 5629980 | 775332 | 105 | 0 | 1 | 84 |
| 5640000 | 20/60/100 | 15 | 5639880 | 775992 | 104 | 0 | 6 | 58 |
| 5640000 | 20/60/100 | 30 | 5639880 | 775992 | 50 | 0 | 2 | 28 |
| 5650000 | 40/80 | 15 | 5650140 | 776676 | 214 | 0 | 4 | 170 |
| 5650000 | 40/80 | 30 | 5650140 | 776676 | 105 | 0 | 1 | 84 |
| 5660000 | 20/60/100 | 15 | 5660040 | 777336 | 104 | 0 | 6 | 58 |
| 5660000 | 20/60/100 | 30 | 5660040 | 777336 | 50 | 0 | 2 | 28 |
| 5670000 | 40/80 | 15 | 5669940 | 777996 | 214 | 0 | 6 | 168 |
| 5670000 | 40/80 | 30 | 5669940 | 777996 | 105 | 0 | 2 | 83 |
| 5680000 | 20/60/100 | 15 | 5679840 | 778656 | 104 | 0 | 0 | 64 |
| 5680000 | 20/60/100 | 30 | 5679840 | 778656 | 50 | 0 | 3 | 27 |
| 5690000 | 40/80 | 15 | 5690100 | 779340 | 214 | 0 | 6 | 168 |
| 5690000 | 40/80 | 30 | 5690100 | 779340 | 105 | 0 | 2 | 83 |
| 5700000 | 20/60/100 | 15 | 5700000 | 780000 | 104 | 0 | 0 | 64 |
| 5700000 | 20/60/100 | 30 | 5700000 | 780000 | 50 | 0 | 3 | 27 |
| 5710000 | 40/80 | 15 | 5709900 | 780660 | 214 | 0 | 0 | 174 |
| 5710000 | 40/80 | 30 | 5709900 | 780660 | 105 | 0 | 3 | 82 |
| 5720000 | 20/60/100 | 15 | 5720160 | 781344 | 104 | 0 | 0 | 64 |
| 5720000 | 20/60/100 | 30 | 5720160 | 781344 | 50 | 0 | 3 | 27 |
| 5745000 | 20/60/100 | 15 | 5745000 | 783000 | 104 | 0 | 6 | 58 |
| 5745000 | 20/60/100 | 30 | 5745000 | 783000 | 50 | 0 | 2 | 28 |
| 5755000 | 40/80 | 15 | 5754900 | 783660 | 214 | 0 | 6 | 168 |
| 5755000 | 40/80 | 30 | 5754900 | 783660 | 105 | 0 | 2 | 83 |
| 5765000 | 20/60/100 | 15 | 5765160 | 784344 | 104 | 0 | 6 | 58 |
| 5765000 | 20/60/100 | 30 | 5765160 | 784344 | 50 | 0 | 2 | 28 |
| 5775000 | 40/80 | 15 | 5775060 | 785004 | 214 | 0 | 6 | 168 |
| 5775000 | 40/80 | 30 | 5775060 | 785004 | 105 | 0 | 2 | 83 |
| 5785000 | 20/60/100 | 15 | 5784960 | 785664 | 104 | 0 | 0 | 64 |
| 5785000 | 20/60/100 | 30 | 5784960 | 785664 | 50 | 0 | 3 | 27 |
| 5795000 | 40/80 | 15 | 5794860 | 786324 | 214 | 0 | 0 | 174 |
| 5795000 | 40/80 | 30 | 5794860 | 786324 | 105 | 0 | 3 | 82 |
| 5805000 | 20/60/100 | 15 | 5805120 | 787008 | 104 | 0 | 0 | 64 |
| 5805000 | 20/60/100 | 30 | 5805120 | 787008 | 50 | 0 | 3 | 27 |
| 5815000 | 40/80 | 15 | 5815020 | 787668 | 214 | 0 | 0 | 174 |
| 5815000 | 40/80 | 30 | 5815020 | 787668 | 105 | 0 | 3 | 82 |
| 5825000 | 20/60/100 | 15 | 5824920 | 788328 | 104 | 0 | 2 | 62 |
| 5825000 | 20/60/100 | 30 | 5824920 | 788328 | 50 | 0 | 0 | 30 |
| 5835000 | 40/80 | 15 | 5835000 | 789000 | 216 | 0 | 2 | 174 |
| 5835000 | 40/80 | 30 | 5835000 | 789000 | 106 | 0 | 0 | 86 |
| 5845000 | 20/60/100 | 15 | 5845080 | 789672 | 104 | 0 | 2 | 62 |
| 5845000 | 20/60/100 | 30 | 5845080 | 789672 | 50 | 0 | 0 | 30 |
| 5855000 | 40/80 | 15 | 5854980 | 790332 | 214 | 0 | 2 | 172 |
| 5855000 | 40/80 | 30 | 5854980 | 790332 | 105 | 0 | 0 | 85 |
| 5865000 | 20/60/100 | 15 | 5864880 | 790992 | 104 | 0 | 4 | 60 |
| 5865000 | 20/60/100 | 30 | 5864880 | 790992 | 50 | 0 | 1 | 29 |
| 5875000 | 40/80 | 15 | 5875140 | 791676 | 214 | 0 | 2 | 172 |
| 5875000 | 40/80 | 30 | 5875140 | 791676 | 105 | 0 | 0 | 85 |
| 5885000 | 20/60/100 | 15 | 5885040 | 792336 | 104 | 0 | 4 | 60 |
| 5885000 | 20/60/100 | 30 | 5885040 | 792336 | 50 | 0 | 1 | 29 |
| 5895000 | 40/80 | 15 | 5894940 | 792996 | 214 | 0 | 4 | 170 |
| 5895000 | 40/80 | 30 | 5894940 | 792996 | 105 | 0 | 1 | 84 |
| 5905000 | 20/60/100 | 15 | 5904840 | 793656 | 104 | 0 | 6 | 58 |
| 5905000 | 20/60/100 | 30 | 5904840 | 793656 | 50 | 0 | 2 | 28 |