3GPP TSG-RAN WG4 Meeting #92-Bis						R4-1911028

Chongqing, China, 14th – 18th October, 2019

Agenda item:	6.5.4.2

Source:	Intel Corporation

Title:	A-MPR for n39

Document for:	Approval

# Introduction

In [1], it was agreed to define A-MPR for channel BWs > 20MHz and new NS signalling value for NR band n39 by RAN4 92-Bis. In this paper, we discuss A-MPR needed for n39 to meet the additional emission requirements.

# A-MPR for n39

In RAN4#92, it was identified that A-MPR is needed for NR band n39 for CBW > 20MHz to protect frequency range 1805-1855MHz with emission requirement of -40dBm/MHz and frequency range 1855-1880MHz with emission requirement of -15.5dBm/5MHz. In [1], it was agreed to define A-MPR for the requirements shown in Table 1 for CBWs > 20MHz and new NS signalling value for NR band n39 by RAN4 92-Bis. In this paper, we discuss A-MPR needed for n39 to meet additional emission requirements.

Table 1: Additional requirements for n39 NS_XX

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 1805 | - | 1855 | -40 | 1 | 1 |
| Frequency range | 1855 | - | 1880 | -15.5 | 5 | 1, 2, 3 |
| NOTE 1: This requirement is applicable for carriers with aggregated channel bandwidths confined in 1885-1920 MHz. NOTE 2: The requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 3: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

## A-MPR Regions

The IMD3 may fall in the frequency range 1855-1880MHz while IMD5 may fall in the frequency range 1805-1855MHz. Our A-MPR simulation results show that the boundaries of A-MPR regions can be derived based on the conditions of having IMD3 in the frequency range 1855-1880MHz. The A-MPR values are calculated to meet the additional emission requirements in both frequency ranges 1805-1855MHz and 1855-1880MHz. If IMD3 is not within the frequency range 1855-1880MHz, MPR is sufficient to meet the additional emission requirements in both frequency ranges 1805-1855MHz and 1855-1880MHz.

There are two regions for IMD3 to fall in the frequency range 1855-1880MHz:

- Region 1: IMD3 due to contiguous RB allocation

- In this region, IMD3 falls in the frequency range 1855-1880MHz due to contiguous RB allocation as shown in Figure 1.

Figure 1: IMD3 due to contiguous RB allocation

- Conditions on RBstart and LCRB to have this IMD3 in the frequency range 1855-1880MHz:

RBstart≤LCRB-5MHz12*SCS

- Region 2: IMD3 due to small contiguous RB allocation and the image

- In this region, IMD3 falls in the frequency range 1855-1880MHz due to small contiguous RB allocation and the image as shown in Figure 2. This region is for small LCRB contiguous allocation close to channel edge.

Figure 2: IMD3 due to small contiguous RB allocation and the image

- Conditions on RBstart to have this IMD3 in frequency range 1855-1880MHz:

RBstart≤BW-5MHz3*12*SCS

## Simulation Results:

Simulation assumptions:

- PC3 reference waveform (BW = 20MHz SCS = 15KHz DFT-s-OFDM QPSK 100RB0) is with 0dB MPR at 23dBm maximum output power

- LO leakage = -28dBc

- Image = -28dBc

- CIMD3 = -60dBc

- CIMD5 = -65dBc

Figures 3-4 shows A-MPR regions needed to protect frequency range 1805-1855MHz with emission requirement of -40dBm/ MHz and frequency range 1855-1880MHz with emission requirement of -15.5dBm/5MHz. As expected,  there are two regions. For any undefined region, MPR is sufficient. The complete set of simulation results are available in Appendix.

Figure 3

Figure 4

Figure 5

Figure 6

## A-MPR for for n39 for CBW > 20MHz Proposal:

Based of our analysis and simulation results, we propose the following A-MPR in tables 1 and 2 for n39 for CBW > 20MHz to protect frequency range 1805-1855MHz with emission requirement of -40dBm/MHz and frequency range 1855-1880MHz with emission requirement of                               -15.5dBm/5MHz.

| Table 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BW | Region 1 |  | Region 2 |  |  |
|  | RBstart | A-MPR | RBstart | LCRB | A-MPR |
| 25MHz | ≤LCRB - 5MHz/12/SCS | A1 | ≤20MHz/3/12/SCS | ≤1.44 MHz/12/SCS | A2 |
|  |  |  |  | ≤3.6 MHz/12/SCS | A3 |
| 30MHz | ≤LCRB - 5MHz/12/SCS | A4 | ≤25MHz/3/12/SCS | ≤1.44 MHz/12/SCS | A5 |
|  |  |  |  | ≤3.6 MHz/12/SCS | A6 |

Proposal: Define the A-MPR in tables 1 and 2 for n39 for CBW > 20MHz to protect frequency range 1805-1855MHz with emission requirement of -40dBm/MHz and frequency range 1855-1880MHz with emission requirement of -15.5dBm/5MHz.

| Table 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A-MPR (dB) |  |  |  |  |  |
|  |  | A1 | A2 | A3 | A4 | A5 | A6 |
| DFT-s-OFDM | BPSK | 2.5 | 2 | N/A | 3.5 | 2 | N/A |
|  | QPSK | 3.5 | 2 | N/A | 4 | 2 | N/A |
|  | 16QAM | 4 | 2.5 | N/A | 4.5 | 2.5 | N/A |
|  | 64QAM | 4 | 2.5 | N/A | 4.5 | 2.5 | N/A |
|  | 256QAM | 4 | 2.5 | N/A | 4.5 | 2.5 | N/A |
| CP-OFDM | QPSK | 5.5 | N/A | 3 | 6 | N/A | 3 |
|  | 16QAM | 5.5 | N/A | 3 | 6 | N/A | 3 |
|  | 64QAM | 5.5 | N/A | 3 | 6 | N/A | 3 |
|  | 256QAM | 5.5 | N/A | 3 | 6 | N/A | 3 |
| NOTE 1: The backoff applied is max (MPR, A-MPR) where MPR is defined in Table 6.2.2-1. |  |  |  |  |  |  |  |

# Conclusion

# In this paper, we studied A-MPR needed for n39 for CBW > 20MHz to protect frequency range 1805-1855MHz with emission requirement of -40dBm/MHz and frequency range 1855-1880MHz with emission requirement of -15.5dBm/5MHz.

Proposal: Define the A-MPR in tables 1 and 2 for n39 for CBW > 20MHz to protect frequency range 1805-1855MHz with emission requirement of -40dBm/MHz and frequency range 1855-1880MHz with emission requirement of -15.5dBm/5MHz.

| Table 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BW | Region 1 |  | Region 2 |  |  |
|  | RBstart | A-MPR | RBstart | LCRB | A-MPR |
| 25MHz | ≤LCRB - 5MHz/12/SCS | A1 | ≤20MHz/3/12/SCS | ≤1.44 MHz/12/SCS | A2 |
|  |  |  |  | ≤3.6 MHz/12/SCS | A3 |
| 30MHz | ≤LCRB - 5MHz/12/SCS | A4 | ≤25MHz/3/12/SCS | ≤1.44 MHz/12/SCS | A5 |
|  |  |  |  | ≤3.6 MHz/12/SCS | A6 |

| Table 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A-MPR (dB) |  |  |  |  |  |
|  |  | A1 | A2 | A3 | A4 | A5 | A6 |
| DFT-s-OFDM | BPSK | 2.5 | 2 | N/A | 3.5 | 2 | N/A |
|  | QPSK | 3.5 | 2 | N/A | 4 | 2 | N/A |
|  | 16QAM | 4 | 2.5 | N/A | 4.5 | 2.5 | N/A |
|  | 64QAM | 4 | 2.5 | N/A | 4.5 | 2.5 | N/A |
|  | 256QAM | 4 | 2.5 | N/A | 4.5 | 2.5 | N/A |
| CP-OFDM | QPSK | 5.5 | N/A | 3 | 6 | N/A | 3 |
|  | 16QAM | 5.5 | N/A | 3 | 6 | N/A | 3 |
|  | 64QAM | 5.5 | N/A | 3 | 6 | N/A | 3 |
|  | 256QAM | 5.5 | N/A | 3 | 6 | N/A | 3 |
| NOTE 1: The backoff applied is max (MPR, A-MPR) where MPR is defined in Table 6.2.2-1. |  |  |  |  |  |  |  |

# References

- R4-1909960, n39 Coexistence Issue and Missing AMPR, RAN4#92, August, 2019.

# Appendix: n39 A-MPR results