3GPP TSG-RAN WG4 Meeting #92-bis	R4-1911053

# Chongqing, China, 14th– 18th Oct, 2019

Source: 	ZTE

Title:	TP to TR 38.820 OBUE requirements for 7-24GHz

Agenda Item:	10.4.8.3

Document for:	Approval

Introduction

The OBUE requirement has been discussed in[1]. It is proposed the following text to be captured in the TR 38.820

Discussion

Part of OBUE requirement has been discussed. Although the regulatory requirements have not been defined yet, we still capture some consideration about the emission mask.

Conclusion

It is proposed the following text proposals to be captured in the TR 38.820.

- References

R4-191XXXX On OBUE requirement for 7-24GHz, zte

--------------Start of text proposal-------------

    - 7.4.1	Receiver requirements

      - 7.4.1.1	Tx requirements overview

      - 7.4.1.X	OBUE Requirement

7.4.1.X.1 OBUE requirement for NR FR1 and FR2

The out of band unwanted emission requirement for NR has been defined for FR1 and FR2 separately. For FR1, the requirements are based on LTE requirements as a set of band-centric masks has been provided. It is agreed that “The emission levels for the UEM will be the same as for LTE (5 MHz and larger channel BW) in the full frequency range (FDL_low – ΔfUEM) ~ (FDL_high + ΔfUEM)” according to TR 38.817-02[1]. Also the following variations for the Unwanted Emissions Mask (UEM) tables should be used for sub-6 GHz:

–	BS classes

–	Category of emission (A/B)

–	Frequency range (above/below 1 GHz)

–	Power levels (for some BS classes)

Hence the OBUE requirements for FR1 is varied from three different classes, two different categories and different rated output TRP (only for MR Base station) in TS 38.104[2].

However, for FR2, we don’t have the legacy LTE requirements to refer. Basically, the requirements are fully influenced by the regulatory requirements. For Cat A limit, it is based on FCC part 30.203 as -13dBm/MHz applies for both spurious domain and the out of band domain. For Cat B, however, the requirement is based on the ECC regulation and it is scaled by the rated TRP and linked to the total transmission bandwidth. Also the submitted LS[3] to ITU-R WP5D for sharing study has the baseline requirements for FR2.

As both FR1 and FR2 emission mask are band-centric, the mask for 7-24GHz can be also assumed as band-centric.

7.4.1.X.2 OBUE Requirements for range 7-24GHz

It can be observed form current NR OBUE requirement, BS classes and especially BS rated output TRP are specified. During the discussion of 7-24GHz SI, the deployment scenario has already been captured in chapter 5.6. As long as the BS classes are similar between FR1 and FR2, it is reasonable to assume similar BS classes should applied for 7-24GHz range. The criteria as minimum distance or MCL that used for distinguishing the BS classes can be further studied.

As there is no legacy LTE requirement for 7-24GHz range, we should define the mask by the same way of FR2. Different regulations should be considered as FCC stands for region 1 and ECC stands for region 2. Hence category A/B should also applied.

The following variations for the Unwanted Emissions Mask (UEM) tables should be used for 7-24 GHz:

–	BS classes

–	Category of emission (A/B)