| 3GPP TR 38.883 V0.1.0 (2019-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on support of NR downlink 256 Quadrature Amplitude Modulation (QAM) for frequency range 2 (FR2) (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Background	7

5	Feasibility study for DL 256QAM	7

5.1	General	7

5.2	Simulation based feasibility study	7

5.2.1	Simulation assumptions	7

5.2.2	Simulation results	7

5.2.3	Conclusion	7

5.3	Implementation based feasibility study	8

5.4	Conclusion	8

6	Specification impact for DL 256QAM	8

6.1	BS part	8

6.2	UE part	8

Annex <A> (informative): Change history	9

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document captures finding of the study on support of NR DL 256QAM for FR2. The purpose of this TR is to study the feasibility and performance benefits for NR DL 256QAM for FR2 as defined in Work Item “Add support of NR DL 256QAM for FR2” [2].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-190760, “Add support of NR DL 256QAM for FR2”, China Telecom

[3]	3GPP TR 38.803: "Study on new radio access technology: RF and co-existence aspects".

[4]	3GPP TS 38.306: "User Equipment (UE) radio access capabilities".

[5]	R4-1909091, “Link level simulation results for the feasibility study of FR2 DL 256QAM”, China Telecom

[6]	R4-1909269, “Initial FR2 DL 256QAM link level simulation results”, Nokia

[7]	R4-1908660, “Feasibility evaluation for NR FR2 DL256QAM”, NTT DOCOMO

[8]	R4-1909186, “Simulation results on DL 256QAM”, Huawei

[9]	R4-1909401, “Initial Link Simulation Results on DL 256 QAM FR2”, Ericsson

[10]	R4-1908391, “Discussion on 256QAM for FR2”, CATT

[11] 	R4-1908211, “Discussion on feasibility of DL 256QAM in FR2 scenarios”, Intel

[12] 	R4-1908144, “Views on feasibility of 256QAM for FR2”, Qualcomm

[13]	Staffan Ek et al., A 28-nm FD-SOI 115-fs Jitter PLL-Based LO System for 24-30-GHz Sliding-IF 5G Transceivers,  IEEE Journal of Solid-State Circuits ( Volume: 53 , Issue: 7 , July 2018 )

[14]	R4-1909403, “Feasibility of UE demodulation testing”, Ericsson

[5.2.2.1-1] 	R4-1713125, “Evaluation on BS Tx EVM for mmWave”, Huawei, HiSilicon

[5.2.2.2-2] 	R4-1906014, “System level simulation for 256QAM DL in FR2”, Nokia, Nokia Shanghai Bell

[5.2.2.2-3]	T. Tuominen, N. Tervo, A.Pärssinen; Analyzing  5G  RF  System  Performance  and Relation to Link Budget for Directive MIMO,  IEEE Transactions on Antennas and Propagation ( Volume: 65 , Issue: 12 , Dec. 2017 )

[5.2.2.3-4]	R4-1908211, “Discussion on feasibility of DL 256QAM in FR2 scenarios”, Intel

[A-1]	R4-1711777, WF on simulation assumption for NR BS EVM requirement, Huawei, HiSilicon

[A-2]	R4-1709666, Simulation assumptions on UE Tx EVM for mmWave, Huawei, HiSilicon

[A-3]	3GPP Radio Access Network Working Group. 2016. “E-UTRA and UTRA Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS) (Release 13)”. 3GPP TR 37.842 V13.0.0.

[A-4]	Du Jinfeng, Chizhik Dmitry, Rodriguez Rodolfo Feick, Mauricio, Castro Guillermo, Valenzuela Reinaldo. A. 2018. “Suburban Fixed Wireless Access Channel Measurements and Models at 28 GHz for 90% Outdoor Coverage”, arXiv: 1807:03763.

[A-5]	3GPP Radio Access Network Working Group. 2018. “Study on channel model for frequencies from 0.5 to 100 GHz (Release 15)”. 3GPP TR 38.901 V15.0.0.

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ACRONYM>	<Explanation>

4	Background

At the 3GPP RAN #83 meeting, the Work Item on “Add support of NR DL 256QAM for FR2” was approved for Rel-16. The objectives of the core part in the Work Item are as follows:

Phase 1:  Continue and complete the feasibility and performance benefit study to identify applicable scenarios

- Both system and link simulations as well as RF and baseband implementation need to be considered in the Rel-16 evaluation to study the benefits of FR2 DL 256QAM.

- This phase is planned from RAN4#91 and should end by RAN4#93.

Phase 2:  Specify BS and UE requirements for NR DL 256QAM for FR2 if applicable scenarios and feasible requirements are identified

- The requirements for BS include Tx modulation quality, Tx power dynamic range etc.

- The requirements for UE include Rx maximum input power level, FRC etc.

This TR aims to study the Phase 1 in the objectives and further capture the impact on the specification if applicable.

5	Feasibility study for DL 256QAM

Editor’s note: This clause collects the feasibility and performance benefits study for Rel-16. If any summarization or retrieve for Rel-15 study, new agenda can be assigned.

5.1	General

The feasibility study for FR2 DL 256QAM will be carried out based on two following parts:

- Simulation based feasibility study

- Implementation based feasibility study

5.2	Simulation based feasibility study

For the simulation study, the methods shall be same as the usual ways we have adopted in the earlier times for modulation orders feasibility evaluation in LTE or in NR. Throughput performance is compared among different modulation orders with EVM variable to verify the benefit for higher order modulation and then define the minimum EVM requirement. However, for NR FR2 simulation study, the impact due to phase noise is no longer negligible comparing to FR1, for higher order modulations.  The phase noise impairment is frequency dependent as stated in 6.1.9.5 of [3] PN could increase by 6 dB every time when f0 doubles. Therefore, the phase noise impairment shall be considered and emphasized in the feasibility study for FR2 DL 256QAM.  As in [3] different PN models and impacts were studied as part of the overall system and not just as part of the effect at the transmitter.

The impact of phase noise will induce two main effects which include

- Rotate the phases of constellation points in the transmitted/received signal by a common value as termed as common phase error (CPE)

- Break the orthogonal in the OFDM signal, each subcarrier is interfered by every other adjacent subcarrier as termed as inter-carrier interference (ICI)

The CPE impacts can be compensated based on the phase offset estimates obtained from the dedicated phase tracking reference signals (PTRS). Although the PTRS is mandatory with UE capability signalling [4], but it is necessary for transmitter/receiver to apply PTRS to remove the CPE, which will not only benefit for 256QAM but also for lower order modulation. On the other side, 256QAM is an optional feature for FR2, but it shall be more applicable with PTRS supporting.

5.2.1	Link level simulation

Link level simulation is targeted as mainstream way to evaluate if FR2 256QAM can achieve benefit by comparing to 64QAM. The simulation results from companies are listed as below.

5.2.1.1	Simulation assumptions

The link level simulation assumptions are listed as in table 5.2.1.1-1, based on which, to evaluate the throughput difference between 64QAM and 256QAM. The study aims to identify conditions where DL 256QAM provides performance benefits.

Table 5.2.1.1-1 link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz (n257) and 39 GHz (n260) |
| CBW | 50 MHz, 100MHz |
| SCS | 60kHz, 120 kHz; |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 30ns delay spread, 35Hz Doppler frequency TDL-D 30ns delay spread, 35Hz Doppler frequency Static (AWGN) |
| MCS | 64QAM: MCS 23, 24, 26, 28 in TS 38.214 Table 5.1.3.1-1, and other MCSs are not precluded 256QAM: MCS 21, 23, 25, 27 in TS 38.214 Table 5.1.3.1-2, and other MCSs are not precluded Baseline: fixed MCSs |
| Precoding | Precoding configuration defined in 38.101-4 Section 7.2 for fading channels and Section 7.5 for static channel; follow PMI |
| Symbol type | CP-OFDM |
| HARQ | 8, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 (for D slots) |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol) |
| Phase noise compensation | Practical based on PTRS |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX Option a): example1 (BS) + example1(UE) Option b): example2 (BS) + example2(UE) Option c): example2 (BS) + example2(BS) Option d):example2 (BS) + PN model config1: example1(UE) Option e): Other phase noise models, e.g. ones extracted from commercially available components or published results, are not excluded |
| txEVM + rxEVM excluding phase noise for 256QAM | txEVM: [1.0%-5.0%], rxEVM: [1.0%-5.0%] Option 1: txEVM <= rxEVM; Option2: no restriction |
| Other parameters | follow assumptions in 38.101-4 Section 7.2 for fading channels (e.g., case 2-6) and Section 7.5 for static channels |

The assumptions adopted by each company are shown as following table 5.2.1.1-2 which are down-selected based on the table 5.2.1.1-1.

Table 5.2.1.1-2 link level simulation assumptions down-selected by companies

| Parameter |  |  |  | CTC[5] | Nokia[6] | Docomo[7] | Huawei[8] | Ericsson[9] | CATT[10] | Intel[11] | Qualcomm[12] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  |  | 29 GHz | 29 GHz | 29 GHz | 29 GHz |  | 29 GHz |  |  |
| CBW |  |  |  | 50MHz | 100MHz | 100MHz | 50MHz |  | 50MHz | 50MHz | 100MHz |
| SCS |  |  |  | 120kHz | 60kHz | 120kHz | 120kHz |  | 60kHz | 60kHz | 120kHz |
| Allocated RBs |  |  |  |  |  |  |  |  |  |  |  |
| Propagation |  |  | TDL-A |  |  |  |  |  |  |  |  |
|  |  |  | TDL-D |  |  |  |  |  |  |  |  |
|  |  |  | Static |  |  |  |  |  |  |  |  |
| MCS |  |  | 64QAM | 28 | 26,28 | 24,25,26,28 | 23,24,26,28 | 23,24,26,28 | 23 | 23,24,26,28 | 26,27,28 |
|  |  |  | 256QAM | 27 | 21,23 | 21,23,25,27 | 21,23,25,27 | 21,23,25,27 | 21 | 21,23,25,27 | 20,21,22 |
| Precoding |  |  |  |  |  |  |  |  |  |  |  |
| Symbol type |  |  |  |  |  |  |  |  |  |  |  |
| HARQ |  |  |  | None | None | 8 |  | None | 8 | 8 | 8 |
| Antenna configuration |  | Fading |  | 2x2 for Rank1 | 2x2 for Rank1 |  |  | 1x2 for Rank1 | 2x2 for Rank1 |  | 2x2 for Rank2 |
|  |  | Static |  |  |  | 2x2 for Rank2 |  | 1x2 for Rank1 | 1x2 for Rank1 |  | 2x2 for Rank2 |
| Channel estimation |  |  |  |  |  |  |  |  |  |  |  |
| Receiver type |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH configuration |  |  |  |  |  |  |  |  |  |  |  |
| DMRS configuration |  |  |  |  |  |  |  | No additional |  |  |  |
| PTRS configuration |  |  |  | None |  |  |  |  |  |  |  |
| Phase noise compensation |  |  |  | None |  | Ideal |  |  |  |  |  |
| Phase noise model | Option a) |  |  |  |  |  |  |  |  |  |  |
|  | Option b) |  |  |  |  |  |  |  |  |  |  |
|  | Option c) |  |  |  |  |  |  |  |  |  |  |
|  | Option d) |  |  |  |  |  |  |  |  |  |  |
|  | Option e) |  |  |  |  |  |  |  |  |  | example1BS+example2UE internal PN model |
| txEVM + rxEVM excluding phase noise for 256QAM |  |  |  | Tx+Rx: 3%, 4% | txEVM: 3%, rxEVM: 3% | txEVM: 0%, 3%, rxEVM: 0%, 3% | txEVM: 1%-3%, rxEVM: 1%-3% |  |  | txEVM: 1%-3%, rxEVM: 1%-3% | Tx:3% Rx:internal |
| Other parameters |  |  |  |  |  |  |  |  |  |  |  |
| Note: The symbol of means selecting the parameters corresponding to table 5.2.1.1-1. |  |  |  |  |  |  |  |  |  |  |  |

5.2.1.2	Results from China Telecom [5]

The key parameter of EVM is constructed of two values dependent on different kinds of distortions. One is fixed EVM which reflects the impairment by the component non-linearity attribute contributed from the full transmitter chain and the other one is derived EVM based on the phase noise of transmitter/receiver. So in the simulation, the EVM variable is defined as fixed EVM + explicit derived EVM by PN model, and then we evaluate the SE performance with 0%, 3% and 4% EVM values to find if any performance benefit for 256QAM by comparing to 64 QAM, in which the case for 0% EVM is as baseline for reference.

Figure 5.2.1.2-1 depicts the spectrum efficiency performance by comparing 256QAM to 64QAM.

The curve with red colour represents the performance for 256QAM without PTRS.

The curve with blue colour represents the performance for 64QAM without PTRS.

Figure 5.2.1.2-1: Spectrum efficiency performance by comparing 256QAM to 64QAM

It is worth to note that all the EVM values in the figure have included the impact due to the phase noise which will contribute the -35dBc EVM in typically. Based on the figure above, we can observe that the phase noise will cause the SE performance degradation, the higher modulation order the more severe degradation. Although the PTRS is mandatory with UE capability signalling, but it is necessary for transmitter/receiver to apply PTRS to remove the CPE, which will not only benefit for 256QAM but also for lower order modulation. On the other side, 256QAM is an optional feature for FR2, but it shall be more applicable with PTRS supporting.

On the other side, even without PTRS which means no phase noise compensation, 256QAM still can achieve higher spectrum efficiency than 64QAM when SINR is larger than 25dB with the total EVM is less than 4%.

5.2.1.3	Results from Nokia [6]

Simulation results obtained with parameters in table 5.2.1.1-2 are shown in Figure 5.2.1.3-1.

Figure 5.2.1.3-1: Link level simulation results

In figure 5.2.1.3-1 it can be seen that the throughput with 256QAM exceeds throughput with 64QAM approximately at 28 dB SNR.

One important aspect to be taken into account when analysing the results is the used phase noise model. As shown in Figure 5.2.1.3-2, the phase noise performance is significantly worse that what is actually achievable with reasonable silicon area and power consumption [13]. On average the phase noise model is 6.5 dB worse than performance in [13]. Therefore with realistic phase noise assumptions more gains are expected.

One should also note that it may be overly optimistic to compare 64QAM and 256QAM with the same baseline EVM on Tx side, as 64QAM EVM budget is more relaxed and this can be utilized by heavier crest factor reduction.

Figure 5.2.1.3-2: Comparison of the used phase noise model and published results

In addition, the results in Figure 5.2.1.3-1 are obtained using 3% Tx EVM, excluding the EVM impact from the phase noise. It should be noted the gain is observed with total EVM of the Tx chain of approximately 5.3 %, excluding the benefits from PT-RS based equalization. Further gains would be observed using EVM contribution which keeps the total EVM similar to FR1 requirement.

Based on the results and analysis the following observation is made.

Observation 1: Even with the used pessimistic phase noise and EVM assumptions throughput gains over 64QAM can be observed with 256QAM.

Observation 2: TR 38.803 config 1 PN model is too pessimistic compared to currently achievable performance. Therefore a [6.5] dB downscaling of the corresponding PN model should be considered.

5.2.1.4	Results from DoCoMo [7]

Figure 5.2.1.4-1 shows link level simulation results compared between 64QAM and 256QAM. On static channel with antenna configuration 2x2 and Rank 2, even with  3% Tx EVM and 3% Rx EVM, the performance gain compared to 64QAM modulation is obtained over 21dB SNR. This SNR is a realistic value that can be achieved in the expected deployment (e.g., Small cell scenario).

Observation 1: From the evaluation results, FR2 DL 256QAM modulation has a better performance than 64QAM modulation with realistic SNR.

a) Tx EVM 0% and Rx EVM 0%						 b) Tx EVM 3% and Rx EVM 3%

Figure 5.2.1.4-1: Simulation results on static channel

From above evaluations, we can conclude that DL 256QAM in FR2 can provide certain system performance gain in realistic network. Therefore, we propose to introduce the requirement of FR2 DL 256QAM.

Regarding Tx EVM, our simulation assumed 3% considering the feasible value. As a requirement for BS Tx EVM, it is sufficient to define 3.5% which is the same requirement as FR1. Therefore, it is proposed to define 3.5% as a requirement for BS Tx EVM for 256QAM modulation.

5.2.1.5	Results from Huawei [8]

Option d): PN model config2: example2 (BS) + PN model config1: example1(UE)

Figure 5.2.1.5-1: simulation results (Option d PN model)

Option a): PN model config1: example1 (BS) + example1(UE)

Figure 5.2.1.5-2: simulation results (Option a PN model)

From the simulation results, it is shown that support 256 QAM can provide significant performance gain over 64QAM where the UE is in good propagation condition. It is also found that the performance is more sensitive to RF impairment for 256 QAM

Observation 1: Support 256QAM can provide significant performance gain over 64QAM where the UE is in good propagation condition.

5.2.1.6	Results from Ericsson [9]

As part of the study a companion paper [14] discusses further the complications on testability for this feature when it comes to receiver demodulation and the required SINR.  Looking at the parameters, the higher order MCS (256 QAM MCS 25 and 27) are not presented here as initial results yielded little to no throughput.  Although to keep ease in simulation time, HARQ was not applied and it may be possible to see more tangible throughput numbers.

The following results show the throughput performance at 256 QAM.

Figure 5.2.1.6-1: SCS 60 kHz, MCS 23

Comparably, when looking at 64 QAM throughput performance is better than 256 QAM when the expected EVM at transmitter and receiver is 5%.  The overall performance of 256 QAM at low SNR is rather sensitive to any added receiver and/or transmitter noise.  The fading channel conditions also provide some aspects to the results below.  Further simulations using HARQ could help this aspect; no link adaptation was simulated for this scenario.

Figure 5.2.1.6-2: 256 QAM and 64 QAM throughput performance comparison

5.2.1.7	Results from CATT [10]

From the Figure 5.2.1.7-1(a) and 5.2.1.7-1(b), the achievable throughput for 256QAM is worse than 64QAM at 3.5% TX EVM and 3.5% RX EVM at SNR 40dB under TDL-A and TDL-D fading channel. The results for TDL-A and TDL-D fading channel indicate that the FR2 256QAM is hard to be deployed in scenarios include Homes, Roof-above or indoor, Commercial centre or official building. From the Figure 5.2.1.7-1(c), the achievable throughput for 256QAM is better than 64QAM at 3.5% TX EVM and 3.5% RX EVM in the SNR range from 30dB to 40dB in static channel. However there may be very limited scenarios with static channel condition in practice.

| a) TDL-A | b) TDL-D |
| --- | --- |
| c) Static channel |  |

Figure 5.2.1.7-1: performance comparison of 64QAM and 256QAM in TDL-A in option b phase noise model

5.2.1.8	Results from Intel [11]

In Table 5.2.1.8-1 andTable 5.2.1.8-2 we provide summary of simulation results and compare performance of 64QAM and 256QAM for 25, 30 and 35 dB SNR point. The table shows the relative throughput improvement in case of using 256QAM comparing to 64QAM.

Table 5.2.1.8-1: Performance improvement of 256QAM over 64QAM for CF 29 GHz

| CF, GHz | Rank configuration | Channel model | Tx/ Rx EVM | Phase noise model C |  |  | Phase noise model D |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 25 dB | 30 dB | 35 dB | 25 dB | 30 dB | 35 dB |
| 29 GHz | Rank 1 | Static | 1% | 19% | 21% | 24% | 19% | 19% | 21% |
|  |  |  | 2% | 15% | 19% | 20% | 11% | 19% | 20% |
|  |  |  | 3% | 11% | 19% | 19% | 11% | 18% | 19% |
|  |  | TDL-A | 1% | 5% | 11% | 19% | 5% | 10% | 18% |
|  |  |  | 2% | 5% | 9% | 16% | 4% | 9% | 13% |
|  |  |  | 3% | 3% | 7% | 11% | 1% | 5% | 10% |
|  |  | TDL-D | 1% | 4% | 13% | 17% | 2% | 9% | 16% |
|  |  |  | 2% | 2% | 9% | 15% | 2% | 8% | 12% |
|  |  |  | 3% | 1% | 7% | 10% | 2% | 5% | 9% |
|  | Rank 2 | Static | 1% | 10% | 19% | 19% | 10% | 19% | 19% |
|  |  |  | 2% | 8% | 15% | 19% | 4% | 12% | 19% |
|  |  |  | 3% | 0% | 11% | 17% | 0% | 11% | 15% |
|  |  | TDL-A | 1% | -2% | -9% | 5% | -2% | -10% | 0% |
|  |  |  | 2% | -1% | -8% | 0% | -1% | -6% | -10% |
|  |  |  | 3% | 0% | -6% | -11% | 0% | -5% | -14% |
|  |  | TDL-D | 1% | -7% | 3% | 9% | -13% | 3% | 6% |
|  |  |  | 2% | -12% | 2% | 6% | -12% | 0% | 2% |
|  |  |  | 3% | -12% | -3% | 3% | -12% | -9% | 2% |
|  | Adaptive Rank | Static | 1% | 10% | 19% | 19% | 10% | 19% | 19% |
|  |  |  | 2% | 8% | 15% | 19% | 4% | 12% | 19% |
|  |  |  | 3% | 0% | 11% | 17% | 0% | 11% | 15% |
|  |  | TDL-A | 1% | -2% | -9% | 5% | -2% | -10% | 0% |
|  |  |  | 2% | -1% | -8% | 0% | -1% | -6% | -10% |
|  |  |  | 3% | 0% | -6% | -11% | 0% | -5% | -14% |
|  |  | TDL-D | 1% | -7% | 3% | 9% | -13% | 3% | 6% |
|  |  |  | 2% | -12% | 2% | 6% | -12% | 0% | 2% |
|  |  |  | 3% | -12% | -3% | 3% | -12% | -9% | 2% |

Table 5.2.1.8-2: Performance improvement of 256QAM over 64QAM for CF 39 GHz

| CF, GHz | Rank configuration | Channel model | Tx/ Rx EVM | Phase noise model C |  |  | Phase noise model D |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 25 dB | 30 dB | 35 dB | 25 dB | 30 dB | 35 dB |
| 39 GHz | Rank 1 | Static | 1% | 11% | 16% | 16% | 11% | 11% | 11% |
|  |  |  | 2% | 11% | 11% | 14% | 10% | 11% | 11% |
|  |  |  | 3% | 9% | 11% | 11% | 1% | 11% | 11% |
|  |  | TDL-A | 1% | 5% | 9% | 11% | 3% | 6% | 11% |
|  |  |  | 2% | 3% | 6% | 10% | 1% | 6% | 9% |
|  |  |  | 3% | 0% | 2% | 8% | -4% | 3% | 4% |
|  |  | TDL-D | 1% | 1% | 8% | 10% | 2% | 6% | 9% |
|  |  |  | 2% | 1% | 2% | 9% | 2% | 1% | 7% |
|  |  |  | 3% | 1% | 2% | 7% | -2% | 1% | 2% |
|  | Rank 2 | Static | 1% | 0% | 10% | 10% | 0% | 10% | 10% |
|  |  |  | 2% | 0% | 10% | 10% | 0% | 9% | 10% |
|  |  |  | 3% | 0% | 4% | 8% | 0% | 0% | 9% |
|  |  | TDL-A | 1% | -3% | -12% | -2% | -1% | -7% | -18% |
|  |  |  | 2% | -1% | -7% | -10% | 0% | -5% | -14% |
|  |  |  | 3% | 0% | -6% | -16% | 0% | -6% | -8% |
|  |  | TDL-D | 1% | -14% | 1% | 2% | -12% | -6% | 1% |
|  |  |  | 2% | -12% | -3% | 2% | -12% | -16% | -2% |
|  |  |  | 3% | -11% | -14% | 1% | -11% | -15% | -14% |
|  | Adaptive Rank | Static | 1% | 0% | 10% | 10% | 0% | 10% | 10% |
|  |  |  | 2% | 0% | 10% | 10% | 0% | 9% | 10% |
|  |  |  | 3% | 0% | 4% | 8% | 0% | 0% | 9% |
|  |  | TDL-A | 1% | -3% | -12% | -2% | -1% | -7% | -18% |
|  |  |  | 2% | -1% | -7% | -10% | 0% | -5% | -14% |
|  |  |  | 3% | 0% | -6% | -16% | 0% | -6% | -8% |
|  |  | TDL-D | 1% | -14% | 1% | 2% | -12% | -6% | 1% |
|  |  |  | 2% | -12% | -3% | 2% | -12% | -16% | -2% |
|  |  |  | 3% | -11% | -14% | 1% | -11% | -15% | -14% |

Observations: From link level results we can conclude

- Static channel model

  - 29 GHz carrier frequency:

    - Sufficient performance improvement of 256QAM over 64QAM (> 5%) is observed for MIMO Rank 1 and 2 transmissions and all considered SNR operating points for most of considered scenarios.

  - 39 GHz carrier frequency

    - MIMO rank 1: sufficient performance improvement is observed for almost all considered SNR points.

    - MIMO rank 2: sufficient performance improvement is observed for SNR > 30 dB only

- Fading channel models

  - Sufficient performance improvement of 256QAM over 64QAM (> 5%) is observed for scenarios with Rank 1 transmission and high SNR conditions (i.e. ≥ 30dB)

  - Limited or no performance improvement of 256QAM over 64QAM is observed for Rank 2 transmission

  - For phase noise model (i.e. model D), significant performance improvement of 256QAM over 64QAM (> 10%) is observed only for scenarios with 29 GHz carrier frequency, Rank 1 transmission and 35dB SNR

5.2.1.9	Results from Qualcomm [12]

Table 5.2.1.9-1 compares the SNR points at 90% of peak throughput for two cases under AWGN conditions.

Table 5.2.1.9-1: Comparison of RAN4 and Internal IPN models under AWGN channel condition

| Test Cases | SNR (dB) at 90% of peak throughput using RAN4 IPN model | SNR (dB) at 90% of peak throughput using internal IPN model | Peak Throughput (Mbps) |
| --- | --- | --- | --- |
| 64QAM, MCS 26, 2x2, Rank2 | 20.51 | 19.30 | 700.72 |
| 64QAM, MCS 27, 2x2, Rank2 | 21.93 | 20.27 | 731.60 |
| 64QAM, MCS 28, 2x2, Rank2 | 24.10 | 21.30 | 762.95 |
| 256QAM, MCS 21, 2x2, Rank2 | 23.38 | 20.94 | 762.95 |
| 256QAM, MCS 22, 2x2, Rank2 | 27.03 | 22.27 | 809.97 |

Based on above results, we have following observations:

Observation 1: Peak Throughput for 64QAM MCS28 is exactly equal to that for 256QAM MCS21.

Observation 2: RAN4 IPN models are very pessimistic. SNR needed to achieve 256QAM regime is very high.

Observation 3: SNR needed to achieve 90% of peak throughput for 64QAM MCS28 is slightly higher than that for 256QAM MCS21 under AWGN conditions.

As RAN4 IPN model is very pessimistic, we now focus on our internal IPN model for the rest of the simulations.

Simulation Results without EVM

In this section, we compare the 64QAM and 256QAM performance under different channel conditions and carrier frequencies to determine whether 256QAM can provide gains over 64QAM under FR2. Here, we look at 70% and 90% of peak throughput since most of the RAN4 fixed MCS requirements are defined at 70% of peak throughput. For all simulations, we assumed our internal IPN model.

Table 2 and Table 3 list the SNRs required to achieve 70% and 90% of peak throughput under different channel conditions with carrier frequency of 29GHz and 39GHz, respectively without considering any Tx/Rx EVM.

Table 5.2.1.9-2: SNR required to achieve 70% and 90% of peak throughput without EVM, carrier frequency = 29GHz

| Test Cases | AWGN SNR (dB) |  | TDL-D 30ns 35Hz SNR (dB) |  | TDL-A 30ns 35Hz SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| % of Peak Throughput | 70% | 90% | 70% | 90% | 70% | 90% |
| 64QAM, MCS 26, 2x2, Rank2 | 18.90 | 19.30 | 20.72 | 22.15 | 25.49 | 28.03 |
| 64QAM, MCS 27, 2x2, Rank2 | 19.81 | 20.27 | 21.53 | 23.18 | 27.00 | 29.60 |
| 64QAM, MCS 28, 2x2, Rank2 | 20.90 | 21.30 | 22.90 | 24.28 | 28.93 | 32.15 |
| 256QAM, MCS 20, 2x2, Rank2 | 19.89 | 20.30 | 21.56 | 23.25 | 26.30 | 28.59 |
| 256QAM, MCS 21, 2x2, Rank2 | 20.12 | 20.94 | 22.34 | 24.00 | 27.22 | 29.72 |
| 256QAM, MCS 22, 2x2, Rank2 | 21.78 | 22.27 | 23.72 | 25.58 | 28.77 | 31.50 |

Table 5.2.1.9-3: SNR required to achieve 70% and 90% of peak throughput without EVM, carrier frequency = 39GHz

| Test Cases | AWGN SNR (dB) |  | TDL-D 30ns 35Hz SNR (dB) |  | TDL-A 30ns 35Hz SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| % of Peak Throughput | 70% | 90% | 70% | 90% | 70% | 90% |
| 64QAM, MCS 26, 2x2, Rank2 | 18.90 | 19.30 | 21.01 | 22.34 | 26.01 | 28.59 |
| 64QAM, MCS 27, 2x2, Rank2 | 19.90 | 20.30 | 21.80 | 23.62 | 27.55 | 30.33 |
| 64QAM, MCS 28, 2x2, Rank2 | 20.94 | 21.37 | 23.32 | 24.91 | 29.87 | - |
| 256QAM, MCS 20, 2x2, Rank2 | 19.91 | 20.32 | 21.80 | 23.65 | 26.88 | 29.42 |
| 256QAM, MCS 21, 2x2, Rank2 | 20.87 | 21.31 | 22.87 | 24.29 | 27.87 | 30.43 |
| 256QAM, MCS 22, 2x2, Rank2 | 22.00 | 22.53 | 24.41 | 26.18 | 29.79 | - |

Based on above results, we have following observations:

Observation 5: SNR needed to achieve high throughput regime using 64QAM or 256QAM is very high for TDL-A channel model.

Observation 6: For AWGN, 256QAM shows gains for SNR > ~20dB and for TDL-D, 256QAM shows gains for SNR > ~22dB over 64QAM without considering EVM.

Observation 7: There is < 0.5dB degradation in performance when going from carrier frequency of 29GHz to 39GHz for lower MCS for 256QAM regime under AWGN and TDL-D channel conditions without considering EVM.

Simulation Results with EVM

In previous sections, we focused on best case scenarios to determine the upper limit of performance. In this section, we present simulation results with EVM since that will be more practical scenario.

As shown in above, very high SNR is needed to achieve high throughput regime under TDL-A condition. Therefore, we will only focus on AWGN and TDL-D channels in this section. We assume Tx EVM of 3% (current RAN4 assumption for 256QAM) for both 64QAM and 256QAM. Rx EVM is assumed as per our internal UE implementation.

Table 5.2.1.9-4 and Table 5.2.1.9-5 list the SNRs required to achieve 70% and 90% of peak throughput under different channel conditions with carrier frequency of 29GHz and 39GHz, respectively with Tx/Rx EVM.

Table 5.2.1.9-4: SNR required to achieve 70% and 90% of peak throughput with EVM, carrier frequency = 29GHz

| Test Cases | AWGN SNR (dB) |  | TDL-D 30ns 35Hz SNR (dB) |  |
| --- | --- | --- | --- | --- |
| % of Peak Throughput | 70% | 90% | 70% | 90% |
| 64QAM, MCS 26, 2x2, Rank2 | 18.90 | 19.30 | 21.18 | 22.50 |
| 64QAM, MCS 27, 2x2, Rank2 | 19.90 | 20.30 | 22.02 | 23.84 |
| 64QAM, MCS 28, 2x2, Rank2 | 20.95 | 21.40 | 23.54 | 25.35 |
| 256QAM, MCS 20, 2x2, Rank2 | 19.91 | 20.32 | 22.06 | 23.89 |
| 256QAM, MCS 21, 2x2, Rank2 | 20.91 | 21.33 | 23.09 | 24.58 |
| 256QAM, MCS 22, 2x2, Rank2 | 21.99 | 22.48 | 24.78 | 26.40 |

Table 5.2.1.9-5: SNR required to achieve 70% and 90% of peak throughput with EVM, carrier frequency = 39GHz

| Test Cases | AWGN SNR (dB) |  | TDL-D 30ns 35Hz SNR (dB) |  |
| --- | --- | --- | --- | --- |
| % of Peak Throughput | 70% | 90% | 70% | 90% |
| 64QAM, MCS 26, 2x2, Rank2 | 18.95 | 19.39 | 21.42 | 23.12 |
| 64QAM, MCS 27, 2x2, Rank2 | 19.98 | 20.46 | 22.54 | 24.14 |
| 64QAM, MCS 28, 2x2, Rank2 | 21.68 | 22.25 | 24.06 | 25.96 |
| 256QAM, MCS 20, 2x2, Rank2 | 20.09 | 20.87 | 22.55 | 24.21 |
| 256QAM, MCS 21, 2x2, Rank2 | 21.02 | 21.63 | 23.55 | 25.47 |
| 256QAM, MCS 22, 2x2, Rank2 | 22.85 | 23.38 | 25.60 | 27.79 |

Based on above results, we have following observations:

Observation 8: For AWGN, 256QAM shows gains for SNR > ~21dB and for TDL-D, 256QAM shows gains for SNR > ~23dB over 64QAM with EVM consideration.

5.2.1.10	Conclusion

Editor Note: this conclusion is an initial conclusion based on the initial results provided in RAN4#91meeting.Further update will be made based on the results updated in the next meeting.

Based on the simulation results and observations provided above, the following table summarizes the SNR ranges in which 256QAM shows benefit by comparing to 64QAM below in table 5.2.1.10-1.

Table 5.2.1.10-1: SNR required to achieve gains for 256QAM

| Contributor | AWGN SNR (dB) | TDL-D SNR (dB) | TDL-A SNR (dB) |
| --- | --- | --- | --- |
| China Telecom |  |  | > 25dB |
| Nokia |  | > 28dB |  |
| DoCoMo | > 21dB |  |  |
| Huawei |  | > 24dB |  |
| Ericsson |  |  | No benefit |
| CATT | > 21dB | > 27dB | > 31dB |
| Intel | > 25dB | > 30dB | > 35dB |
| Qualcomm | > 21dB | > 23dB |  |
| Average | > 21dB | > 26.4dB |  |

5.2.2	System level simulation

System simulation is targeted as supplementary for link level simulation to further confirm the scenario that FR2 256QAM is applicable. The simulation results from companies are listed as below and corresponding assumptions are captured in Annex A.

5.2.2.1	Results from Huawei

The simulation results and proposals are based on contribution [5.2.2.1-1]

Figure 5.2.2.1-1 to 5.2.2.1-4 show the SINR CDF by a range of tx EVM for different modulation orders. On the right of SINR CDF are the corresponding curves for throughput loss versus tx EVM values.

Figure 5.2.2.1-1: SINR CDF and throughput loss versus BS tx EVM for QPSK

Figure 5.2.2.1-2: SINR CDF and throughput loss versus BS tx EVM for 16QAM

Figure 5.2.2.1-3: SINR CDF and throughput loss versus BS tx EVM for 64QAM

Figure 5.2.2.1-4: SINR CDF and throughput loss versus BS tx EVM for 256QAM

It can be observed from the SINR CDF that the receiving SINR will be improved when the tx EVM requirement becomes more tighter. However it seems difficult to find a trade-off Tx EVM value since the SINR improvement looks like being averaged across the tx EVM values. Thus we map the SINR CDF to throughput to quantize the improvement to SINR for different tx EVM values. It is worth to note that there is little difference for the throughput loss baseline when EVM is lowest value in the certain given range or 0%.

For the series of modulation orders, the figures show more stringent requirement is needed to meet the 5% throughput loss threshold, however by considering the feasibility and LTE requirement as baseline, and also a little margin for degradation due to phase noise, it is proposed to reuse LTE UE BS EVM requirement for QPSK, 16QAM, 64QAM and 256QAM modulation orders for NR.

5.2.2.2	Results from Nokia

The simulation results and proposals are based on contribution [5.2.2.2-2]

Figure 5.2.2.2-1 presents CDF of DL SINR results for two configurations:

- BS with 16x8 antenna array and total Tx power of 31dBm; UE with 1x1 antenna (blue curve),

- BS with 16x8 antenna array and total Tx power of 31dBm; UE with 2x2 antenna array (red curve).

Figure 5.2.2.2-1: CDF of DL SINR for 256QAM

It can be observed that for the first configuration (UE 1x1) the SINR equal or higher than 25dB is obtained for 15% of the best DL links, whereas SINR equal or higher than 28dB is obtained for 10% of the best DL links. In case of second configuration (UE 2x2) the same levels of SINR are accessible for 24% and 16% of the best DL links, respectively.

According to studies presented in [5.2.2.2-3] the SINR of 25dB at the receiver is required to ensure sufficent link quality for 256QAM modulation, the results of presented simulation study indicate that up to 25 % of users in assumed scenario will benefit from higher DL data rate, given that other Tx and Rx impairments have limited impact.

It was observed that in fixed wireless access scenario up to 25% of the users are in propagation conditions which enable benefits from 256 QAM.

5.2.2.3	Results from Intel

The simulation results and proposals are based on contribution [5.2.2.3-4]

Figure 5.2.2.3-1 shows large scale SINR distribution for considered scenarios.

| Figure 5.2.2.3-1: System-level large scale SINR distribution |  |
| --- | --- |

Observations: From system-level simulation results we can observe that:

- Indoor office scenario: 5% of users have SINR  ≥  25 dB

- Urban micro scenario: 20% of users have SINR ≥ 25 dB

5.2.2.4	Conclusion

Editor Note: this conclusion is an initial conclusion based on the initial results collected in RAN4#91meeting. Further update will be made based on the results updated in the next meeting.

Based on the simulation results and observations provided above, it can be concluded that from system level simulation point, up to 25% users operate in SINR >25dB region required for work of 256QAM for scenarios such as fixed wireless access, backhaul, indoor office and urban micro scenarios in LOS.

5.3	Implementation based feasibility study

Editor’s note: This clause captures implementation based feasibility study for both BS and UE sides.

5.4	Conclusion

6	Specification impact for DL 256QAM

Editor’s note: This clause will depend on the conclusion of the clause for feasibility study

6.1	BS part

6.2	UE part

7	Demod test challenge for DL 256QAM

Editor’s note: This clause will capture the study for highlighting demod test challenge which will have no impact to define the core requirement or start the normative work.

Annex <A> (informative):
System level simulation assumptions

A.1 Assumptions from Huawei

According to the approved WF in [A-1], the system simulation assumptions for mmWave were agreed as following

- The assumptions agreed for UE in [A-2] can be used as a reference for urban macro scenario

The simulation methodology for EVM model and simulation metrics are as follows.

EVM model

- The tx EVM is modeled corresponding to the MCS by adding a certain white noise to the transmitter, and the white noise follows the distribution as.

Simulation metrics

The simulation results are for SINR statistics at the UE side, of which the interference is consisted by own system co-channel interference. Considering the co-channel interference variation with tx EVM, the tx EVM requirement shall be determined by comparing SINR degradation across a range of EVM values. Further, the SINR degradation for the certain EVM can be quantized by mapping to the corresponding throughput loss to evaluate the system performance more directly.

A.2 Assumptions from Nokia

Figure A.2-1 presents simulation layout in suburban area with single-storey or double-storey buildings and vegetation with area of 700m x 600m, consist of 16 blocks with 20 buildings each. For each block two sectors BS is used, with one sector per 10 buildings. There is assumption that 90% of buildings have outside CPEs, and 10% of buildings closest to BS use inside CPEs. Geometry and details are presented in figure A.2-1. Table A.2-1 provides parameters used in simulations for BS and CPE.

Figure A.2-1: Simulation layout

Table A.2-1: Simulation parameters

| BS |  |
| --- | --- |
| Carrier frequency | 28 GHz |
| Channel Bandwidth | 800 MHz |
| Antenna pattern | According to [A-3] |
| Gain of antenna element | 6 dBi |
| Antenna array (H × V) | 16x8 |
| Tx power (without loss) / polarization | 31 dBm |
| Height of antenna | 8 m |
| CPE |  |
| UE density | 1 UE / sector |
| Antenna pattern | According to [A-3] |
| Gain of antenna element | 6 dBi |
| Antenna array (H × V) | 1x1 / 2x2 |
| Height of antenna | 1,5 m |
| Orientation in horizontal plane | Towards BS |
| Orientation in vertical plane | Towards BS |
| Noise Factor | 9 dB |

As 3GPP TR 38.901 does not specify channel model for suburban area, the 3GPP UMi Street Canyon model has been used, with modification of path loss and angular spread characteristics according to measurement campaign performed in real suburban campaign in 28 GHz frequency band [A-4].

Propagation conditions that are assumed in simulations are described in table A.2-2. Path loss channel models used in simulations are described in table A.2-3.

| BS – CPE separation in 2D | Conditions | Note |
| --- | --- | --- |
| Up to 20 m | LOS/Indoor | BS and CPE on the same street. All LOS CPEs are Indoor |
| Beyond 20 m | VLOS/Outdoor | BS and CPE on the same street. LOS with Vegetation (VLOS) [A-4] |
| Different street | NLOS/Outdoor | BS and CPE on different streets. |

Table A.2-2: Assumed propagation conditions

Table A.2-3: Path loss channel model according to [A-4]

| Propagation conditions | Path loss [dB] (d [m]: distance between BS and CPE) | Standard deviation [dB] |
| --- | --- | --- |
| LOS | 61,4 + 24,0 · log10(d) | 4,2 |
| VLOS | 45,1 + 40,6 · log10(d) | 6,4 |
| NLOS | 80,3 + 31,3 · log10(d) | 4,8 |
| O2I | 15,1 | 2,5 |

Angular spread models for departure and arrival (Azimuth Spread of Departure (ASD); Zenith Spread of Departure (ZSD); Azimuth Spread of Arrival (ASA); Zenith Spread of Arrival (ZSA)) used in simulations are presented in table A.2-4.

Table A.2-4: Angular spread model for departure and arrival

| Propagation conditions | log10 (ASD/1º) | log10 (ZSD/1º) | log10 (ASA/1º) | log10 (ZSA/1º) |
| --- | --- | --- | --- | --- |
| LOS | 3GPP UMi SC [A-5] | 3GPP UMi SC [A-5] |   [A-4] | 3GPP UMi SC [A-5] |
| VLOS |   [A-4] | 3GPP UMi SC [A-5] |   [A-4] | 3GPP UMi SC [A-5] |
| NLOS |   [A-4] | 3GPP UMi SC [A-5] |   [A-4] | 3GPP UMi SC [A-5] |

A.3 Assumptions from Intel

The simulation assumptions are from TR 38.802 and TR 38.855. Table A.3-1 provides information on the main simulation assumptions.

Table A.3-1: System level simulation assumptions

|  |  | Indoor Hotspot | Urban Micro |
| --- | --- | --- | --- |
| Layout |  | Indoor floor: (12BSs per 120m x 50m), TRP number per floor:12, Inter-gNB distance = 20m | Hexagonal grid, 19 macro sites, 3 sectors per site, ISD = 200m |
| UE drop procedure |  | 100% indoors, uniformly distributed | 100% outdoors, uniformly distributed |
| Channel model |  | Indoor open office from TR 38.901 | UMi Street Canyon from TR 38.901 |
| Traffic model |  | Full buffer |  |
| gNB parameters | Antenna configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1) dH=dV=0.5λ | (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2) dH=dV=0.5λ |
|  | Antenna radiation pattern | TR 38.802 Table A.2.1-7 | TR 38.802 Table A.2.1-6 |
|  | Number of beams | 8 | 16 |
|  | Beam selection | Optimal beam for Serving cell, random for neighboring cells |  |
|  | Antenna height | 3 m | 10 m |
|  | TX power | 24dBm | 37dBm (per panel) |
| UE parameters (FR2 PC3 UE , Handheld) | Antenna configuration | 4 antenna elements, 2 panels |  |
|  | Antenna radiation pattern | Omni, 0dBi |  |
|  | Antenna height | 1.5 m |  |
|  | Avg. element gain | 5 dBi |  |
|  | Implementation loss | 10 dB |  |
|  | Noise figure | 10 dB |  |

Annex <B> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-08 | RAN4#92 | R4-1910478 |  |  |  | TR skeleton: Study on support of NR downlink 256 Quadrature Amplitude Modulation (QAM) for frequency range 2 (FR2) | 0.0.1 |