3GPP TSG RAN WG4 Meeting #92-Bis	R4-1911144

Chongqing, CN, 14-18 Oct. 2019

Agenda item:	9.8.2

Source:	SoftBank Corp.

Title:	TP for TR 37.716-21-21: EN-DC_1_n3-n28

Document for:	Approval

1	Introduction

EN-DC of 1B LTE and 2B NR of DC_1_n3-n28 was approved in RAN#85 [1]. This TP is to capture the basic aspects for the EN-DC. REFSENS exception results for 5th IMD are needed to complete the combo.

2	Text Proposal

[Unchanged Parts Skipped]

6.X	DC_1_n3-n28

6.X.1	Operating bands for DC

Table 6.X.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n3-n28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

6.X.2	Channel bandwidths per operating band for DC

Table 6.X.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1_n3-n28 | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 70 |
|  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.X.3	Co-existence studies

Based on the co-existence studies of DC_1A-n3A and DC_1A-n28A captured in TR 37.716-11-11 and 37.863-01-01, 5th order IMD generated by dual uplink of Band 1 + Band n28 may also fall into own Rx of band n3.

6.X.4	∆TIB and ∆RIB values

For DC_1_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n28 | 1 | 0.3 |
|  | n3 | 0.3 |
|  | n28 | 0.6 |

Table 6.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n3-n28 | 1 | 0 |
|  | n3 | 0 |
|  | n28 | 0.2 |

6.X.5	MSD

As mentioned in 6.X.3, IMD5 of B1 and n28 to Band n3 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-n3A_n28A | 1 | TBD | TBD | TBD | TBD | N/A | FDD | N/A |  |
|  | n28 | TBD | TBD | TBD | TBD | N/A | TDD | N/A |  |
|  | n3 | TBD | TBD | TBD | TBD | TBD | FDD | IMD5 |  |
| DC_1A-n3A_n28A | 8 | TBD | TBD | TBD | TBD | N/A | FDD | N/A |  |
|  | n28 | TBD | TBD | TBD | TBD | N/A | TDD | N/A |  |
|  | n3 | TBD | TBD | TBD | TBD | TBD | FDD | IMD5 |  |

[Unchanged Parts Skipped]

7.  Reference

[1]  RP-191744  Revised WID on EN-DC of x bands (x=1,2,3,4) LTE and NR 2 bands, LG Electronics