3GPP TSG RAN WG4 Meeting #92-Bis	R4-1911149

Chongqing, CN, 14-18 Oct. 2019

Agenda item:	9.4.2

Source:	SoftBank Corp.

Title:	TP for TR 37.716-21-11: EN-DC_1-8_n28

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 1B NR of DC_1-8_n28 was approved in RAN#85[1]. This TP is to capture the basic aspects for the EN-DC.

2	Combination proposed

Combinations proposed in [1] are as below.

Table 1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-8_n28 | CA_1-8 | n28 | No |

Table 2: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-8A_n28A | DC_1A_n28A DC_8A_n28A | CA_1A-8A | n28A |

3.  Co-existence

When Uplink EN-DC configuration is DC_1_n28, 5th IMD will fall into Rx band of Band 8. When Uplink EN-DC configuration is DC_8_n28, 5th IMD will fall into Rx band of Band 1.

4.  Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows. EN-DC values are picked.

Table 3: ΔTIB of constituent 2 band combo.

| ΔTIB | B1+B8 | B1+n28 | B8+n28 | Proposed Value |
| --- | --- | --- | --- | --- |
| B1 | 0.3 | 0.3 |  | 0.3 |
| B8 | 0.3 |  | 0.6 | 0.6 |
| n28 |  | 0.6 | 0.5 | 0.6 |

Table 4: ΔRIB of constituent 2 band combo.

| ΔRIB | B1+B8 | B1+n28 | B8+n28 | Proposed Value |
| --- | --- | --- | --- | --- |
| B1 | 0 | 0 |  | 0 |
| B8 | 0 |  | 0.2 | 0.2 |
| n28 |  | 0.2 | 0.1 | 0.2 |

Then the proposal for this 2LTE-1NR EN-DC is:

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n28 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n28 | 1 | 0 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |

5.  REFSENS

As mentioned in section 3, REFSENS exceptions due to 5th IMD are expected. TP includes skeleton so that the actual values are filled when evaluation results come up.

Table 5: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-8A_n28A | 1 | TBD | TBD | TBD | TBD | N/A | FDD | N/A |  |
|  | n28 | TBD | TBD | TBD | TBD | N/A | TDD | N/A |  |
|  | 8 | TBD | TBD | TBD | TBD | TBD | FDD | IMD5 |  |
| DC_1A-8A_n28A | 8 | TBD | TBD | TBD | TBD | N/A | FDD | N/A |  |
|  | n28 | TBD | TBD | TBD | TBD | N/A | TDD | N/A |  |
|  | 1 | TBD | TBD | TBD | TBD | TBD | FDD | IMD5 |  |

6.  Conclusion

This paper proposes study results of EN-DC 1-8_n28. REFSENS exception results for 5th IMD are needed to complete the combo.

TP is shown in Annex.

7.  Reference

[1]  RP-192087  WID revision: Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL); rapporteur: Huawei

Annex.   Proposed text for TR37.716-21-11:

[Unchanged Parts Skipped]

5.1.x	DC_1-8_n28

### 5.1.x.1	Operating bands for DC_1-8_n28

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-8_n28 | CA_1-8 | n28 | No |

### 5.1.x.2	Configurations for DC_1-8_n28

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-8A_n28A | DC_1A_n28A DC_8A_n28A | CA_1A-8A | n28A |

### 5.1.x.3	Co-existence studies

### When Uplink EN-DC configuration is DC_1_n28, 5th IMD will fall into Rx band of Band 8. When Uplink EN-DC configuration is DC_8_n28, 5th IMD will fall into Rx band of Band 1.

### 5.1.x.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.x.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n28 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |

Table 5.1.x.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n28 | 1 | 0 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |

### 5.1.x.5	REFSENS requirements

As mentioned in 5.1.x.3, IMD5 of B1 and n28 to Band 8 Rx and IMD5 of B8 and n28 to Band 1 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-8A_n28A | 1 | TBD | TBD | TBD | TBD | N/A | FDD | N/A |  |
|  | n28 | TBD | TBD | TBD | TBD | N/A | TDD | N/A |  |
|  | 8 | TBD | TBD | TBD | TBD | TBD | FDD | IMD5 |  |
| DC_1A-8A_n28A | 8 | TBD | TBD | TBD | TBD | N/A | FDD | N/A |  |
|  | n28 | TBD | TBD | TBD | TBD | N/A | TDD | N/A |  |
|  | 1 | TBD | TBD | TBD | TBD | TBD | FDD | IMD5 |  |

[Unchanged Parts Skipped]