3GPP TSG RAN WG4 Meeting #92-Bis	R4-1911151

Chongqing, CN, 14-18 Oct. 2019

Agenda item:	9.5.2

Source:	SoftBank Corp.

Title:	TP for TR 37.716-31-11: EN-DC_1-8-42_n77

Document for:	Approval

1	Introduction

EN-DC of 3B LTE and 1B NR of DC_1-8-42_n77 was approved in RAN#85[1]. This TP is to capture the basic aspects for the EN-DC.

2	Combination proposed

Combinations proposed in [1] are as below.

Table 1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-8-42_n77 | CA_1-8-42 | n77 | DC_1_n77 |

Table 2: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-8A-42A_n77A | DC_1A_n77A DC_8A_n77A | CA_1A-8A-42A | n77A |
| DC_1A-8A-42C_n77A | DC_1A_n77A DC_8A_n77A | CA_1A-8A-42C | n77A |

3.  Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows.

Table 3: ΔTIB of constituent 4 band combo.

| ΔTIB | B1+B8+B42 | B1+B8+n77 | B1+B42+n77 | B8+B42+n77 | Proposed Value |
| --- | --- | --- | --- | --- | --- |
| B1 | 0.3 | 0.3 | 0.6 |  | 0.6 |
| B8 | 0.6 | 0.6 |  | 0.6 | 0.6 |
| B42 | 0.8 |  | 0.8 | 0.8 | 0.8 |
| n77 |  | 0.8 | 0.8 | 0.8 | 0.8 |

Table 4: ΔRIB of constituent 4 band combo.

| ΔRIB | B1+B8+B42 | B1+B8+n77 | B1+B42+n77 | B8+B42+n77 | Proposed Value |
| --- | --- | --- | --- | --- | --- |
| B1 | 0 | 0 | 0.2 |  | 0.2 |
| B8 | 0.2 | 0.2 |  | 0.2 | 0.2 |
| B42 | 0.5 |  | 0.5 | 0.5 | 0.5 |
| n77 |  | 0.5 | 0.5 | 0.5 | 0.5 |

Then the proposal for this 3LTE-1NR EN-DC is:

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-8-42_n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8-42_n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

5.  REFSENS

MSD of this 4DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01, TR 37.716-11-11, TR37.863-02-01 and TR37.716-21-11. Therefore, no additional MSD requirement is needed.

6.  Conclusion

This paper proposes study results of EN-DC 1-8-42_n77.

TP is shown in Annex.

7.  Reference

[1]  RP-191813  Revised WID: Basket WI Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL), Ericsson

Annex.   Proposed text for TR37.716-31-11: LTE 3B -NR 1B DC

[Unchanged Parts Skipped]

5.1	Inter-band EN-DC within FR1

## 5.1.xx	DC_1-8-42_n77

### 5.1.xx.1	Operating bands for EN-DC

Table 5.1.xx.1-1: Band combinations EN-DC (four bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-8-42_n77 | CA_1-8-42 | n77 | DC_1_n77 |

### 5.1.xx.2	Configurations for EN-DC

Table 5.1.xx.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-8A-42A_n77A | DC_1A_n77A DC_8A_n77A | CA_1A-8A-42A | n77A |
| DC_1A-8A-42C_n77A | DC_1A_n77A DC_8A_n77A | CA_1A-8A-42C | n77A |

### 5.1.xx.3	∆TIB and ∆RIB values

For DC_1-8-42_n77, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.xx.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-42_n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 5.1.xx.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-8-42_n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

### 5.1.xx.4	REFSENS requirements

Co-existence study for DC_1-8-42_n77 was covered by the studies for the fallback modes of DC_1-8_n77, DC_1-42_n77 and DC_8-42_n77.

No additional MSD requirement need to be defined for this dual connectivity configuration.

[Unchanged Parts Skipped]