3GPP TSG-RAN WG4 Meeting #92                                                 R4-1911165

Ljubljana, Slovenia, 20th to 26th Aug. 2019

Agenda Item:	6.5.5.1

Source: 	MediaTek Inc., Murata

Title: 	DC_11_n77 and DC_21_n77 OOBB relaxation

Document for:	Approval

- 1. Introduction

RAN4#92 discussed OOBB exceptions[1][2] and agreed to further analyse DC_B11_n77 and DC_B21_n77 OOBB exception due to 2nd order inter-modulation by B11/B21 and CW of out of band blocking in range 3. This contribution provide analysis for the combinations.

- 2. Discussion

The OOBB exception mechanism was discussed in [3]. A comparison table in [2] and filter data collected from a few component vendors are copied below in Table 2-1 and Table 2-2 respectively:

|  |  | [1] | [2] | DOCOMO |  |
| --- | --- | --- | --- | --- | --- |
| a | Low band carrier UL power at antenna port | 23dBm | 19dBm | 19dBm |  |
| b | Low band attenuation w diplexer + n78 filter | 64(30+32+2)dB | 52(20+32) | 76.2(23.9+52.3) IL is 1.15 for diplexer | 87.3(35+52.3) IL is 2.0 for diplexer |
| c | IIP2 | 10dBm | 7dBm | 10dBm |  |
| d | Blocker attenuation at region 3 w n78filter  applied IIP2 to both low band output and blocker, respectively | 20(18+2) or 27(25+2)dB | Not applied | FFS |  |
| e | Blocker level | -20dBm | -20dBm | -20dBm |  |
| f | LNA selectivity | 10dB | Not applied | FFS |  |
|  | Power level of 2nd Inter-modulation (PIMD2 =a - b - c - d +e –f) | -101 or -108dBm | -60dBm | -87.2dBm | -98.3 |
|  | Reference sensitivity (REFSENS n78 10MHz SCS=15kHz) | -95.8dBm(REFSENS) | -89.8dBm (REFSENS + 6dB) | -89.8dBm (REFSENS + 6dB) |  |
|  | Required improvement (Delta between IMD2 and Reference sensitivity) | -5.2 or -12.2dB | 29.8dB | 2.6 | -8.5 |

Table 2-1: comparison between three contributions

| n77 Filter | IL (dB) | Rejection (dB) in MHz Frequency Range |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | < 2700 | 2700 - 3050 | 4450 - 4900 | 4900 - 5925 | 5150 - 5850 |
| Vendor A | 1.25 (typ) | 44 (typ) | 5.4 (typ) | 2.9 (typ) | 39.7 (typ) | N/A |
| Vendor B | 1.90 (max) | 40 (min) | N/A | N/A | N/A | 20 (min) |
| Vendor C | 2.6 (max) | >30 (typ) | 12 (typ) | 12 (typ) | 10 (typ) | 12 (typ) |
| Vendor D-1 | 1.2 (typ) | 33 (typ) | 2.9@3050 (typ) 13@2800 (min) | 2.3@4450 (typ) 5@4700 (min) | 15 (typ) | 25 (typ) |
| Vendor D-2 | 1.3 (typ) 2.0 (max) | 43 (typ) | 12.3@3050 (typ) 30@2800 (min) | 5.0@4450 (typ) 13@4700 (min) | 30 (typ) | 30 (typ) |

Table 2-2 Collective n77 filter data from component vendors

The B11 uplink frequency range is 1427.9~1447.9MHz, B21 uplink frequency range is 1447.9~1462.9MHz. Owing to the wide frequency range in the band n77, the OBB near the band edges can be separated from the wanted signal by a frequency gap which equals to the pairing band (B11/21) UL carrier frequency. As a result, the 2nd order intermodulation product generated from the B11/21 UL and the blocker would fall onto the wanted signal to cause sensitivity degradation. In Table 2-2, it can be seen that for Vendor A ~D, the filters virtually do not provide any rejection or provide poor rejection even to up 4900 MHz and down to 2700 MHz. It is not sufficient to pass range 3 OOBB requirements where the blocker level is specified at -15dBm. In table 2-1, it shows about 20dB or more shortage for filter rejection to pass range 3 OOBB requirements.

Observation 1: n77 filters provide poor out of band rejection for frequency range up to 4900MHz and down to 2700MHz

Observation 2: Even with 13dB OOBB rejection, it is still not sufficient to pass range 3 OOBB requirements where the blocker level is specified at -15dBm.

Based on the above assessment, we think there is sufficient technical justification to preserve this exception in the spec. We also think it would not be reasonable to force a UE to use a higher cost or more filter components to simply pass the artificially engineered OBB requirement without exceptions where the blocking condition may not exist in real field and yet such filters may render higher insertion loss. Further, indirect path such as PCB isolation shall also be considered. Even the direct path can reach enough rejection with higher cost or more filters, transmitter power may still reach receiver LNA port thru coupling path.

Proposal 1: The following EN-DC combinations should be included in the OBB exception band combination list in Rel-15 technical specifications.

DC_11_n77

DC_21_n77

Reference:

- R4-1909059 OBB exception for n28_n78, NTT Docomo, Inc. RAN4#92

- R4-1910317 Draft CR for TS 38.101-3: Additional out-of-band blocking exceptions for inter-band EN-DC, RAN4#92

- R4-1906155, “OBB issue for n77/n78/n79 pairing with low-band in CA/SUL/EN-DC”, MediaTek Inc., RAN4#91

- R4-1911164 Draft CR for OOBB relaxation, RAN4#92-bis