3GPP TSG RAN WG4 Meeting #92-Bis	R4-1911200

Chongqing, China, 14th – 18th Oct, 2019

3GPP TSG RAN WG1 Meeting #98						R1-1909910

Prague, CZ, August 26th – 30th, 2019

Title:	LS on NR V2X synchronization procedures and priority

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN1

To:	RAN2, RAN4

Cc:

Contact Person:

#### Name:	Teng Ma

#### E-mail Address:	mateng@catt.cn

Attachments:	None

1. Overall Description:

In NR V2X synchronization discussion, RAN1 reached the following agreements by email discussion [98-NR-09]:

- For confirmation of the working assumption of synchronization priority rules, eNB/gNB should be included into the priority order of GNSS-based synchronization. GNSS-based synchronization; gNB/eNB-based synchronization. • P0: GNSS • P1: UE directly synchronized to GNSS • P2: UE indirectly synchronized to GNSS • P3: gNB/eNB • P4: UE directly synchronized to gNB/eNB • P5: UE indirectly synchronized to gNB/eNB • P6: the remaining UEs have the lowest priority.; • P0’: gNB/eNB • P1’: UE directly synchronized to gNB/eNB • P2’: UE indirectly synchronized to gNB/eNB • P3’: GNSS • P4’: UE directly synchronized to GNSS • P5’: UE indirectly synchronized to GNSS • P6’: the remaining UEs have the lowest priority.. - When GNSS-based synchronization is (pre-)configured, it is also supported to disable the use of P3, P4, P5 by (pre-)configuration (i.e., a UE skips P3, P4, P5 in synchronization reference selection procedure). Details are up to RAN2.

2. Actions:

To RAN WG2:

RAN1 respectfully requests RAN2 to implement the above agreement in their specification work.

To RAN WG4:

RAN1 respectfully requests RAN4 to take the above agreement into consideration in their future work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #98bis                 October 14 – 20, 2019    		Chongqing, China

TSG-RAN WG1 Meeting #99                      November 18 – 22, 2019    		Reno, USA