3GPP TSG-RAN WG4 Meeting # 92bis	R4-1911212

# Chongqing, China, 14 – 18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | <CR#> | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.817-02: Clarification on interference level of receiver dynamic range requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is not clear that the lowest SCS defined for each BS channel bandwidth and the corresponding transmission bandwidth configuration are used to calculate the AWGN interference level of the other SCS in the receiver dynamic range requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the lowest SCS defined for each BS channel bandwidth and the corresponding transmission bandwidth configuration are used to calculate the AWGN interference level of the other SCS in the receiver dynamic range requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity remains and would lead to different interpretations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

## 7.3	Dynamic range

Dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal inside the received BS channel bandwidth. In this condition a throughput requirement shall be met for a specified reference measurement channel.

FRC

NR supports multiple numerologies as well as increased number of BS channel bandwidths, e.g. up to 100MHz for frequency range 1. To better balance the spec complexity and test permutation, it was agreed to define the following FRCs for dynamic range requirement. The SNR @95% maximum throughput point for each FRC will be used for the wanted signal calculation.

7.3-1: Fixed reference channel and SNR points for dynamic range

| FRC | BS channel bandwidth [MHz] | Subcarrier spacing [kHz] |
| --- | --- | --- |
| G-FR1-A2-1 | 5 | 15 |
| G-FR1-A2-2 | 5 | 30 |
| G-FR1-A2-3 | 10 | 60 |
| G-FR1-A2-4 | 20 | 15 |
| G-FR1-A2-5 | 20 | 30 |
| G-FR1-A2-6 | 20 | 60 |

Interference level

Receiver dynamic range requirement is to ensure that the base station can receive high throughput also in the presence of increased interference levels. It measures the effects of base station receiver impairments. While measuring these effects, uncertainty due to the receiver’s own thermal noise floor should be minimized. So the interference level should be increased by a certain amount of margin to mask the receiver’s own noise floor. From a scenario of view, it is reasonable to adopt the similar amount of interference signal margin for NR as for E-UTRA. The mean power of interfering signal is defined in the following method:

PIntf = -174dBm/Hz+10*log10(NRB*SCS*12) + NF + 20

Where:

-	NRB is the transmission bandwidth configuration NRB of the lowest SCS defined in TS38.104 Section 5.3.2 for each BS channel bandwidth;

-	NF is noise figure of NR BS receiver in dB, where 5dB is assumed for WA, 10dB for MR ,13dB for LA;

Wanted signal

The mean power of the wanted signal is defined in the following method:

Pwanted= = -174dBm/Hz+10*log10(NRB*SCS*12) + NF + 20+SNR+IM

Where

-	NF is noise figure of NR BS receiver in dB, where 5dB is assumed for WA, 10dB for MR ,13dB for LA;

-	IM is implementation margin reserved for BS manufacturer in dB; For high MCS level, IM is assumed to be 2.5dB;

-	NRB depends on the FRCs defined for dynamic range requirement;

-	SNR is the value to satisfy the 95% throughput of measurement channel;

<End of change>