3GPP TSG-RAN WG4 Meeting # 92bis	R4-1911219

# Chongqing, China, 14 – 18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | <CR#> | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.817-02: Clarification on interfering signal frequency offsets of receiver intermodulation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The following omissions make it unclear how the interfering signal frequency offsets of receiver intermodulation requirements are obtained: 1) The agreement that the subcarrier spacing of the interfering signal shall be 30 kHz for the case of wanted signal subcarrier spacing 60 kHz and BS channel bandwidth <=20MHz has not been recorded. 2) The agreement that the BS channel bandwidth of the modulated interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap has not been recorded. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing agreements to clarify how the interfering signal frequency offsets of receiver intermodulation requirements are obtained. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity remains and would lead to different interpretations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

## 7.7	Receiver intermodulation

The following have been agreed for conducted receiver intermodulation requirements in FR1:

-	To specify the below 6GHz NR BS receiver intermodulation conducted requirements for each NR BS class with an interfering signal power equal to that for the corresponding E-UTRA BS class, and the wanted signal level calculated as the BS reference sensitivity plus 6 dB.

-	The SNR for the BS reference sensitivity can be obtained at 95% relative throughput from link level simulations.

-	The bandwidth of the modulated interfering signal should be defined as the same as that of the interfering signal for the ACS requirement, with SCS is the same as that of the wanted signal to ensure that any peaks of the IM product align with the SCS of the wanted signal, except the subcarrier spacing of the interfering signal shall be 30 kHz for the case of wanted signal subcarrier spacing 60 kHz and BS channel bandwidth <=20MHz.

-	The offsets between the CW and modulated interfering signal centre frequency and the nominal band edge of the wanted carrier specified to ensure the intermodulation products fall (almost) on the edge resource blocks of an operating BS channel bandwidth and avoid orthogonality between sub-carriers of wanted and interfering signal. The BS channel bandwidth of the modulated interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap.

-	Requirements are set for all possible SCS of the wanted signal, but only the lowest SCS supported by the BS needs to be tested.

Based on above agreements, the receiver intermodulation requirement is specified in Table 7.7.2-1 and Table 7.7.2-2 of 3GPP TS 38.104 [3] for general intermodulation and Table 7.7.2-3 and Table 7.7.2-4 of 3GPP TS 38.104 [3] for narrowband intermodulation.

<End of change>