3GPP TSG-RAN4  #92-Bis																	     	R4-1911238

Chongqing, CN, 14-18 Oct. 2019

Source: 	Qualcomm Incorporated

Title: 	n259 peak EIRP and REFSENS Budget

Agenda item:	9.16.1

Work item:	NR_n259-Core

Document for:	Discussion

# Introduction

We previously shared our view on n259 REFSENS and peak EIRP in [1]. The WF from last meeting [2] solicited input from more companies. In this contribution we elaborate further on our proposals for n259 (39.5 to 43.5 GHz).

# Discussion

## Peak EIRP

In [1] we shared our proposal for minimum peak EIRP in n259. Our proposal is based on rel. 15 assumptions for EIRP metric is per subsection 5.3.2.2 of TR38.810. Further details are included in table 2.1-1.

| Parameter | Worst Case Value | units | Notes |
| --- | --- | --- | --- |
| # of elements in array | 4 |  |  |
| Pout per element | 10.0 | dBm | PVT corners |
| Gain per element | 3.5 | dBi |  |
| # of chains | 2 |  | Arranged to transmit in orthogonal polarizations |
| Implementation Losses | 4.3 | dB |  |
| Packaging Loss | 6.7 | dB | Conservative estimate |
| Peak EIRP | 17.5 | dBm |  |

Table 2.1-1: n259 min peak EIRP budget

## REFSENS

In [1] we had shared a proposal for REFSENS in n259. We have refined our budget relative to [1], which allows us to present a more aggressive estimate for REFSENS. Our proposal continues to be based on rel. 15 assumptions for REFSENS. These details are captured in [3], for convenient reference. Parameter details are included in table 2.2-1.

| Parameter | Worst Case Value | units | Notes |
| --- | --- | --- | --- |
| # of elements in array | 4 |  |  |
| Element NF | 12.0 | dB | PVT corners |
| Gain per element | 3.5 | dBi |  |
| # of chains | 2 |  | Arranged to receive orthogonal polarizations, MRC |
| Implementation Losses | 3.0 | dB |  |
| Packaging Loss | 6.7 | dB | Conservative estimate |
| REFSENS | -82.6 | dBm | SNR Target = -1dB, 100MHz channel |

Table 2.2-1: n259 REFSENS budget

Our revised REFSENS estimate for n259 is almost equivalent to the requirement for n260 (-82.7dBm), despite possibly worse NF and antenna gain. While this may seem odd, it is useful to look at [4], where our estimate for REFSENS for n260 was actually -85.7dBm. The n260 requirement was adjusted upward from our estimate after inclusion of proposals from other contributors. Our REFSENS estimate for n259 shows about 3dB degradation from our n260 estimate.

Note that REFSENS estimate is agnostic of misalignment between UE and TE axes. This flexibility comes from the assumptions on receiver topology made for rel. 15.

## A note on packaging loss

Both, peak EIRP and REFSENS estimates include a conservative estimate of packaging loss (‘form factor integration loss’), made on behalf of UE vendors. We however believe lower loss packaging is implementable and are willing to use lower parameter value in the budgets, depending on RAN4 consensus.

# Conclusion

We presented updated min peak EIRP and REFSENS estimates for n259, based on rel. 15 assumptions:

- Min. peak EIRP = 17.5 dBm

- REFSENS @ -1dB SNR and 100M = -82.6 dBm

We also believe our estimates for packaging loss are perhaps pessimistic, so we are willing to modify our estimates based on future RAN4 consensus on this parameter.

# Reference

- R4-1909970, ‘n259 RF parameters’, Qualcomm, RAN4#92, Ljubljana, Sl, Aug 2019

- R4-1910511, ‘WF on link budget parameters for Tx/Rx of n259’, Huawei et al, RAN4#92, Ljubljana, Sl, Aug 2019

- R4-1900671, ‘On FR2 EIS’, Qualcomm, RAN4#90, Athens, GR, Feb-Mar 2019

- R4-1804589, ‘mmw UE EIS’, Qualcomm, RAN4#86, Melbourne, AUS, Apr 2018