3GPP TSG-RAN WG4 Meeting #92-Bis                                       R4-1911321

Chongqing, China, 14th – 18th Oct, 2019

Agenda Item:	8.1.4.3

Source:	MediaTek Inc.

Title:	Discussion on RRC re-establishment requirement for NR-U

Document for:		Discussion

- Introduction

In RAN4-92 meeting, the RRC re-establishment requirements for NR-U have been provided in the WF [1]. In this paper, it further discusses the requirements.

- Discussion

The NR UE re-establishment delay requirement is provided [2] as

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

where the Tidentify_intra_NR and Tidentify_inter_NR,i are provided per target cell and extended according to the number of DMTC cycles not available at the UE.

Observation 1: the number of DMTC cycles not available at the UE is cell-specific, and the Tidentify_intra_NR and Tidentify_inter_NR,i are also cell-specific.

For re-establishment, UE would search for cells on a given carrier, and it is up to UE to choose which cell to camp on. Therefore, since the target cell on a carrier is selected by UE, the delay on a carrier will be up to UE, and the eventual re-establishment delay will be also up to UE, as long as the delay is still within the upper limit.

Observation 2: Re-establishment delay will depend on the target cell selected by the UE.

Proposal 1: When the re-establishment delay is within the upper limit, it is up to UE to select which cell to camp on.

- Summary

In this paper, the RRC re-establishment requirements for NR-U has been discussed. We have the following observations and proposals:

Observation 1: the number of DMTC cycles not available at the UE is cell-specific, and the Tidentify_intra_NR and Tidentify_inter_NR,i are also cell-specific.

Observation 2: Re-establishment delay will depend on the target cell selected by the UE.

Proposal 1: When the re-establishment delay is within the upper limit, it is up to UE to select which cell to camp on.

References

[1] R4-1909988, WF on RRM Requirements for NR-U, Ericsson.

[2] TS38.133, NR; Requirements for support of radio resource management, V15.7.0.