3GPP TSG-RAN WG4 Meeting #92-Bis                                       R4-1911323

Chongqing, China, 14th – 18th Oct, 2019

Agenda Item:	8.1.4.7

Source:	MediaTek Inc.

Title:	Discussion on active BWP switching requirement for NR-U

Document for:		Discussion

- Introduction

In RAN4-92 meeting, it has been discussed the requirements for active BWP switching requirement for NR-U [1]. In this paper, it further discussed the impact of LBT failures on PDSCH reception or PUSCH transmission after the BWP switching.

- Discussion

UE shall complete the switch of active DL and/or UL BWP within the delay defined in section 8.6 in [2]. From the core requirement perspective, the LBT failure would not have impact, since the delay requirement is not defined based on channel availability or other reference signals. After the switch delay, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP.

Observation 1: The current active BWP switch delay is defined as the time when UE is ready, instead of when the channel is available.

Observation 2: The LBT failure would not have impact on the core requirement of active BWP switch delay for NR-U.

However, the ACK transmission was used in the test cases as the confirmation of completion of BWP switching. Therefore, it would have impact on BWP test cases of BWP.

Proposal 1: Capture the delayed ACK transmission due to UL LBT in BWP test cases, if the test cases are introduced.

- Summary

In this paper, the requirements for active BWP switching have been discussed. We have the following observations and proposals:

Observation 1: The current active BWP switch delay is defined as the time when UE is ready, instead of when the channel is available.

Observation 2: The LBT failure would not have impact on the core requirement of active BWP switch delay for NR-U.

Proposal 1: Capture the delayed ACK transmission due to UL LBT in BWP test cases, if the test cases are introduced.

References

[1] R4-1909988, WF on RRM Requirements for NR-U, Ericsson.

[2] TS38.133, NR; Requirements for support of radio resource management, V15.7.0.