3GPP TSG-RAN WG4 Meeting #92	R4-1911327

Chongqing, China, 14th– 18th October 2019

Agenda item:	8.18.1.5

Source: 		NTT DOCOMO, INC.

Title: 	Further study on power imbalance requirement for intra-band EN-DC/NR CA

Document for:	Approval

1. Introduction

At the last RAN4 meeting, RAN4 agreed to introduce power imbalance requirement for intra-band contiguous NR CA with 2CC scenario, but whether to introduce the requirement for intra-band EN-DC in FR1 is still FFS [1]. In this contribution, we discuss the detailed test parameters for intra-band NR CA scenario and necessity of power imbalance requirement for intra-band EN-DC scenario in FR1.

2. Discussion

2.1. Intra-band contiguous NR CA

For this scenario, 2 contiguous CC with 6 dB power imbalance was agreed in [1]. In this section, we discuss the other test parameters. In order to reduce the work load for this requirement, firstly, the existing test conditions in LTE specification (i.e. section 8.2.1.7 in [2]) and NR specifications for single carrier normal PDSCH test in [3] should be reused as much as possible.

Proposal 1: For power imbalance test for intra-band NR-CA, the following test parameters are applied:

| Parameters | Value |
| --- | --- |
| Duplex mode | Case #1: FDD+FDD CA w/ 15kHz SCS Case #2: TDD+TDD CA w/ 30kHz SCS (TDD pattern: 7DS2U) |
| RB allocation | Full allocation |
| PDSCH configurations | Mapping type: Type A K0: 0 PRB bundling size: WB |
| PDSCH DMRS configurations | DMRS type: Type 1 Number of additional DMRS: 1 (i.e. 1+1) |
| Modulation order | 64QAM (Code rate is FFS) |
| Transmission rank | Rank 1 |
| Max number of HARQ transmission | 1 (RV = {0}) |
| PDCCH allocation | Symbol #0 |
| MIMO configuration | 2x2 / 2x4 MIMO |
| Propagation condition | Static propagation condition No external noise sources are applied |
| Precoding configuration | SP Type I, Random per slot with PRB bundling granularity |

Proposal 2: For power imbalance test for intra-band NR-CA, the following test methodology is applied:

  - Reference testing point: 85%ile throughput

  - Measurement cell: Weaker cell only

For channel bandwidth combination (CA configuration hereafter) for intra-band NR CA with contiguous 2CCs, the following CA configurations were specified in the current Rel-15 UE RF requirement [7]. As pointed out in the last RAN4 meeting, UE is allowed to signal any “maximum supported” channel bandwidth of each CC in CA scenario as UE capability signalling [5, 6]. Therefore, to ensure the power imbalance requirement is surely tested for any UE capabilities, all of below CA configurations or a CA configuration with minimum aggregated channel bandwidth should be specified as CA configuration for power imbalance requirement. To minimize the work load, the latter approach would be preferable.

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n41C | - | 40 | 80, 100 |  |  |  | 180 | 0 |
|  |  | 50, 60, 80 | 60, 80, 100 |  |  |  |  |  |
| CA_n77C CA_n78C CA_n79C | - | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |

Proposal 3: As CA configuration for power imbalance requirement, specify the following CA configurations to ensure the requirement is surely tested for any UE capability of channel bandwidth in the Rel.15 core spec.

- {40+80 MHz, 50+60MHz}

- Further discuss after Rel-16 core spec is finalized.

2.2. Intra-band EN-DC in FR1

For intra-band FR1 EN-DC, whether to introduce power imbalance requirement is FFS [1]. In this section, we describe the necessity of this kind of new requirement.

From Rel-15 UE RF, some band combinations for intra-band EN-DC were also introduced, e.g. DC_3_n3, DC_41_n41, DC_42_n78. Note the DC_42_n78 has different band number for NR and LTE side, but the LTE band 42 (3.4-3.6GHz) is completely contained in the NR band n78 (3.3-3.8GHz), so UE RF requirements of intra-band EN-DC are applied for this band combination. For the intra-band EN-DC cases, only collocated scenario is considered based on the RRM specification, however, reception power of LTE and NR would be different according to deployment scenario. Roughly speaking, the following three deployment scenarios could be considered for intra-band EN-DC (Figure 1):

- Scenario #1: LTE and NR have different antenna/beam pattern, e,g., advanced beamforming technology for NR

- Scenario #2: LTE and NR have same antenna pattern, but the transmission precoding matrix (TPMI) is different.

- Scenario #3: LTE and NR have same antenna pattern and transmission precoding matrix by strict coordination of LTE/NR schedulers at eNB/gNB

The same beam pattern and tight scheduler coordination such as Scenario #3 would be ideal from the system performance perspective, but non-ideal scenarios such as Scenario #1 and #2 should be also allowed in practical. In the Rel-15 specification, NSA scenario was firstly specified as early drop, and this enables that NR cells are deployed into the existing LTE network and smooth migration from LTE to NR in future. Since it is not likely that the NR equipment in the initial deployment are totally renewed when NSA are migrated to SA, for NR side advanced beamforming technology would be applied from the initial deployment of NSA. In addition, from the view point of minimization of LTE development cost, non-scheduler coordination between LTE and NR is one of practical solution.

The reception power of LTE and NR would be the same only in the scenario #3, and it would be different for other scenarios. Especially, the scenario #1 would be the worst case from power imbalance perspective. To analyse the worst-case power imbalance for intra-band EN-DC in FR1, we conducted system level simulation assuming DC_42A_n78A and Dense Urban scenario as an example. The system evaluation assumption including evaluation parameters and beam pattern are described in Annex. Note that in this band combination, RF components would be common between LTE and NR according to UE RF discussion [8], and proper AGC implementation should be verified in this case.

Figure 1: Deployment scenario for intra-band EN-DC

The figure 2 show CDF curve of difference of downlink instantaneous reception power between LTE and NR carriers. From the result, we can observe maximally 25 dB power imbalance between LTE and NR due to different beam patterns. Note that DL SINR distribution (geometry) in NR can be significantly improved thanks to narrower beam pattern (lower inter-cell interference as a result) as shown in Fig. A2 in Annex, even if serious power difference would occur. Hence, this scenario is worth considering.

Figure 2. DL RSRP difference between LTE and NR carriers in Scenario #1

(Note: RSRP means here instantaneous reception power)

Observation 2: For intra-band EN-DC scenario in FR1, UE may suffer significant power imbalance between LTE and NR carriers even in co-located scenario if beam pattern is different between LTE and NR.

- Maximally 25 dB power imbalance is observed in system evaluation.

Proposal 4: Power imbalance requirement should be introduced to ensure correct UE implementation in intra-band EN-DC scenario in FR1.

Proposal 5: For test setup, NR carrier has lower power of 6dB than LTE carrier, and only throughput NR carrier is tested.

3. Conclusion

In this contribution, we discuss the detailed test parameters for intra-band NR CA scenario and necessity of power imbalance requirement for intra-band EN-DC scenario in FR1. Our proposals and observations are summarized below.

Proposal 1: For power imbalance test for intra-band NR-CA, the following test parameters are applied:

| Parameters | Value |
| --- | --- |
| Duplex mode | Case #1: FDD+FDD CA w/ 15kHz SCS Case #2: TDD+TDD CA w/ 30kHz SCS (TDD pattern: 7DS2U) |
| RB allocation | Full allocation |
| PDSCH configurations | Mapping type: Type A K0: 0 PRB bundling size: WB |
| PDSCH DMRS configurations | DMRS type: Type 1 Number of additional DMRS: 1 (i.e. 1+1) |
| Modulation order | 64QAM (Code rate is FFS) |
| Transmission rank | Rank 1 |
| Max number of HARQ transmission | 1 (RV = {0}) |
| PDCCH allocation | Symbol #0 |
| MIMO configuration | 2x2 / 2x4 MIMO |
| Propagation condition | Static propagation condition No external noise sources are applied |
| Precoding configuration | SP Type I, Random per slot with PRB bundling granularity |

Proposal 2: For power imbalance test for intra-band NR-CA, the following test methodology is applied:

  - Reference testing point: 85%ile throughput

  - Measurement cell: Weaker cell only

Proposal 3: As CA configuration for power imbalance requirement, specify the following CA configurations to ensure the requirement is surely tested for any UE capability of channel bandwidth in the Rel.15 core spec.

- {40+80 MHz, 50+60MHz}

- Further discuss after Rel-16 core spec is finalized.

Observation 2: For intra-band EN-DC scenario in FR1, UE may suffer significant power imbalance between LTE and NR carriers even in co-located scenario if beam pattern is different between LTE and NR.

- Maximally 25 dB power imbalance is observed in system evaluation.

Proposal 4: Power imbalance requirement should be introduced to ensure correct UE implementation in intra-band EN-DC scenario in FR1.

Proposal 5: For test setup, NR carrier has lower power of 6dB than LTE carrier, and only throughput NR carrier is tested.

References

- R4-1910052, NTT DOCOMO, WF on UE demodulation requirements with power imbalance, RAN4 #92

- TS36.101 (V16.2.0)

- TS38.101-4 (V15.2.0)

- R4-1910046, Intel Cooperation, Way forward on PDSCH CA normal demodulation requirements, RAN4 #92

- TS38.306 (V15.7.0)

- TS38.331 (V15.7.0)

- TS38.101-1 (V15.6.0)

- R4-1901884, Qualcomm Incorporated, ENDC DL Carrier Imbalance for Co-bands, RAN4 #90

Annex

Table A1. System evaluation assumption

| Parameter | Value |
| --- | --- |
| Deployment | Single macro layer, Hex. Grid |
| ISD | 200m |
| Carrier frequency | B42: 3.5GHz, n78: 3.7GHz |
| CBW | B42: 20MHz (100PRBs), n78: 100MHz (273PRBs) |
| Channel model | TR38.901 UMi-Street canyon |
| BS EIRP | 33 dBm |
| BS antenna config | (M,N,P,Mg,Ng) = (4,4,2,1,1) (dH,dV,dHg,dVg) = (0.5,0.5,NA,NA)λ |
| BS antenna height | 10m |
| UE antenna height | 1.5m (all outdoor) |
| UE receiver NF | 9dB |
| UE distribution | 10 users per TRP |

Figure A1. Beam pattern for LTE and NR

Figure A2. DL SINR comparison between LTE and NR