3GPP TSG-RAN WG4 Meeting #92bis                                                             R4-1911356

Chongqing, China, 14th-18th Oct, 2019

Source:	Samsung, Verizon

Title:	TP for TR 37.716-21-11: DC_13-66_n260

Agenda item:			9.4.3

Document for:	Approval

# 1. Introduction

In RAN#85 meeting, the revised WID on EN-DC for 3 bands DL with 2 bands UL (2 LTE band + 1 NR band) was approved [1], in which the latest EN-DC for 3 bands DL with 2 bands UL (2 LTE band + 1 NR band) was attached. This contribution is a text proposal for TR 37.716-21-11 to include related band combinations for DC_13-66_n260 and DC_13-66-66_n260.

# 2. Reference

- RP-192087, Revised WID on EN-DC for 3bands DL with 2 bands UL (2 LTE band + 1 NR band), Huawei.

# 3. Text Proposal

<Start of Text Proposal>

## 5.2.x	DC_13-66_n260, DC_13-66-66_n260

### 5.2.x.1	Operating bands for DC

Table 5.2.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_13-66_n2601 | CA_13-66 | n260 | No |
| DC_13-66-66_n2601 | CA_13-66-66 | n260 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

### 5.2.x.2	Configurations for DC

Table 5.2.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_13A-66A_n260(2A) DC_13A-66A_n260(3A) DC_13A-66A_n260(4A) DC_13A-66A_n260(5A) DC_13A-66A_n260(6A) DC_13A-66A_n260(2A-2G) DC_13A-66A_n260(2H) DC_13A-66A_n260(A-2G) DC_13A-66A_n260(3A-G) DC_13A-66A_n260(G-H) DC_13A-66A_n260(2G) DC_13A-66A_n260(2A-G) DC_13A-66A_n260(A-H) DC_13A-66A_n260(A-G) | DC_13A_n260A DC_66A_n260A | CA_13A-66A | CA_n260(2A) CA_n260(3A) CA_n260(4A) CA_n260(5A) CA_n260(6A) CA_n260(2A-2G) CA_n260(2H) CA_n260(A-2G) CA_n260(3A-G) CA_n260(G-H) CA_n260(2G) CA_n260(2A-G) CA_n260(A-H) CA_n260(A-G) |
| DC_13A-66A-66A_n260A DC_13A-66A-66A_n260(2A) DC_13A-66A-66A_n260(3A) DC_13A-66A-66A_n260(4A) DC_13A-66A-66A_n260(5A) DC_13A-66A-66A_n260(6A) DC_13A-66A-66A_n260G DC_13A-66A-66A_n260H DC_13A-66A-66A_n260I DC_13A-66A-66A_n260J DC_13A-66A-66A_n260K DC_13A-66A-66A_n260L DC_13A-66A-66A_n260M DC_13A-66A-66A_n260(2A-2G) DC_13A-66A-66A_n260(2H) DC_13A-66A-66A_n260(A-2G) DC_13A-66A-66A_n260(3A-G) DC_13A-66A-66A_n260(G-H) DC_13A-66A-66A_n260(2G) DC_13A-66A-66A_n260(2A-G) DC_13A-66A-66A_n260(A-H) DC_13A-66A-66A_n260(A-G) | DC_13A_n260A DC_66A_n260A | CA_13A-66A-66A | n260A CA_n260(2A) CA_n260(3A) CA_n260(4A) CA_n260(5A) CA_n260(6A) CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M CA_n260(2A-2G) CA_n260(2H) CA_n260(A-2G) CA_n260(3A-G) CA_n260(G-H) CA_n260(2G) CA_n260(2A-G) CA_n260(A-H) CA_n260(A-G) |

### 5.2.x.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_13-66_n260 have been given in TR37.863-02-01 as the below tables.

Table 5.2.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n260 DC_13-66-66_n260 | 13 | 0.3 |
|  | 66 | 0.3 |
|  | n260 | 0 |

Table 5.2.x.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n260 DC_13-66-66_n260 | 13 | 0 |
|  | 66 | 0 |
|  | n260 | 0 |

<End of Text Proposal>