3GPP TSG-RAN WG4 Meeting #92bis    	R4-1911374

Chongqing, China, 14th – 18th Oct， 2019

Agenda item:	6.12.2.1.1

Source:	Samsung

Title:	Updated simulation results for NR BS demodulation performance in Rel-15

Document for:	Discussion

# 1	Introduction.

In this contribution, the updated ideal and impairment simulation results for NR BS demodulation performance are provided for requirements finalization.

# 2	Simulation Results

In this subsection, based on the agreed simulation assumption for NR PUSCH and NR PUCCH [1][2], both ideal simulation results and impairment results are provided in this contribution, as shown in table 1 to table 11, respectively.

Table1: The target SNR performance for NR PUSCH with 1Tx2Rx with CP-OFDM

| FR | SCS | Channel | MCS | DMRS configuration | Target SNR (ideal) | Target SNR(impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 mapping type A | 15KH, 10MHz | TDLB100-400 | MCS2 | 1+1 | -3.5 | -1.5 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.53 | 10.53 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.89 | 12.89 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -3.99 | -1.99 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.5 | 10.5 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.37 | 12.37 |
| FR1 mapping type B | 15KHz, 10KHz | TDLB100-400 | MCS2 | 1+1 | -3.21 | -1.21 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.15 | 10.15 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.52 | 12.52 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -4.23 | -2.23 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 7.93 | 9.93 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.3 | 12.3 |
| FR2 with PTRS | 60KHz,100MHz | TDLA30-300 | MCS16 | 1 | 9.93 | 12.43 |
|  |  |  |  | 1+1 | 8.67 | 11.17 |
|  |  | TDLA30-75 | MCS20 | 1 | 11.73 | 14.23 |
|  |  |  |  | 1+1 | 11.59 | 14.09 |
|  | 120KHz, 100MHz | TDLA30-300 | MCS16 | 1 | 9.50 | 12.00 |
|  |  |  |  | 1+1 | 9.06 | 11.56 |
|  |  | TDLA30-75 | MCS20 | 1 | 11.46 | 13.96 |
|  |  |  |  | 1+1 | 11.51 | 14.01 |
| FR2 without PTRS | 60KHz, 100MHz | TDLA30-300 | MCS2 | 1 | -4.43 | -1.93 |
|  |  |  |  | 1+1 | -4.72 | -2.22 |
|  |  | TDLA30-300 | MCS16 | 1 | 9.41 | 11.91 |
|  |  |  |  | 1+1 | 8.28 | 10.78 |
|  |  | TDLA30-75 | MCS20 | 1 | 11.20 | 13.70 |
|  |  |  |  | 1+1 | 11.24 | 13.74 |
|  | 120KHz, 100MHz | TDLA30-300 | MCS2 | 1 | -4.29 | -1.79 |
|  |  |  |  | 1+1 | -4.36 | -1.86 |
|  |  | TDLA30-300 | MCS16 | 1 | 8.80 | 11.30 |
|  |  |  |  | 1+1 | 8.48 | 10.98 |
|  |  | TDLA30-75 | MCS20 | 1 | 10.80 | 13.30 |
|  |  |  |  | 1+1 | 10.90 | 13.40 |
| FR1 mapping type A | 15KHz, 5MH | TDLB100-400 | MCS2 | 1+1 | -3.21 | -1.01 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.46 | 10.46 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.81 | 12.81 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -3.56 | -1.56 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.4 | 10.4 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.78 | 12.78 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -3.58 | -1.68 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.64 | 10.64 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.45 | 12.45 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -4.51 | -2.51 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.58 | 10.58 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.55 | 12.55 |
|  | 30KHZ, 100MHz | TDLB100-400 | MCS2 | 1+1 | -4.53 | -2.53 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.49 | 10.49 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 11.24 | 13.24 |
| FR1 mapping type B | 15KHz, 5MHz | TDLB100-400 | MCS2 | 1+1 | -3.47 | -0.77 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 7.66 | 9.66 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.44 | 12.44 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -3 | -1 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.03 | 10.3 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.48 | 12.48 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -3.83 | -1.83 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.1 | 10.1 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.4 | 12.4 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -4.16 | -2.16 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.01 | 10.01 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.4 | 12.4 |
|  | 30KHz, 100MHz | TDLB100-400 | MCS2 | 1+1 | -4.11 | -2.11 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 7.89 | 9.89 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 11.08 | 13.08 |
| FR2 with PTRS | 60KHz, 50MHz | TDLA30-300 | MCS16 | 1 | 9.23 | 11.73 |
|  |  |  |  | 1+1 | 8.51 | 11.01 |
|  |  | TDLA30-75 | MCS20 | 1 | 11.31 | 13.81 |
|  |  |  |  | 1+1 | 11.3 | 13.8 |
|  | 120KHz, 50MHz | TDLA30-300 | MCS16 | 1 | 9.45 | 11.95 |
|  |  |  |  | 1+1 | 8.87 | 11.37 |
|  |  | TDLA30-75 | MCS20 | 1 | 11.36 | 13.86 |
|  |  |  |  | 1+1 | 11.2 | 13.7 |
|  | 120KHz, 200MHz | TDLA30-300 | MCS16 | 1 | 9.55 | 12.05 |
|  |  |  |  | 1+1 | 9.05 | 11.55 |
|  |  | TDLA30-75 | MCS20 | 1 | 11.55 | 14.05 |
|  |  |  |  | 1+1 | 11.54 | 14.04 |
| FR2 without PTRS | 60KHz, 50MHz | TDLA30-300 | MCS2 | 1 | -4.44 | -1.94 |
|  |  |  |  | 1+1 | -4.55 | -2.05 |
|  |  | TDLA30-300 | MCS16 | 1 | 8.99 | 11.49 |
|  |  |  |  | 1+1 | 8.15 | 10.65 |
|  |  | TDLA30-75 | MCS20 | 1 | 10.63 | 13.13 |
|  |  |  |  | 1+1 | 10.62 | 13.12 |
|  | 120KHz, 50MHz | TDLA30-300 | MCS2 | 1 | -3.73 | -1.23 |
|  |  |  |  | 1+1 | -3.99 | -1.49 |
|  |  | TDLA30-300 | MCS16 | 1 | 8.73 | 11.23 |
|  |  |  |  | 1+1 | 8.24 | 10.74 |
|  |  | TDLA30-75 | MCS20 | 1 | 10.71 | 13.21 |
|  |  |  |  | 1+1 | 10.63 | 13.13 |
|  | 120KHz, 200MHz | TDLA30-300 | MCS2 | 1 | -4.23 | -1.73 |
|  |  |  |  | 1+1 | -4.51 | -2.01 |
|  |  | TDLA30-300 | MCS16 | 1 | 8.82 | 11.32 |
|  |  |  |  | 1+1 | 8.39 | 10.89 |
|  |  | TDLA30-75 | MCS20 | 1 | 10.78 | 13.28 |
|  |  |  |  | 1+1 | 10.75 | 13.25 |

Table2: The target SNR performance for NR PUSCH for 1Tx4Rx with CP-OFDM

| FR | SCS | Channel | MCS | DMRS configuration | Target SNR (ideal) | Target SNR (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 mapping type A | 15KH, 10MHz | TDLB100-400 | MCS2 | 1+1 | -7.29 | -5.29 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.63 | 6.63 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 7.23 | 9.23 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -7.51 | -5.51 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.56 | 6.56 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 6.75 | 8.75 |
| FR1 mapping type B | 15KHz, 10KHz | TDLB100-400 | MCS2 | 1+1 | -6.83 | -4.53 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.25 | 6.25 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 7.03 | 9.03 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -7.42 | -5.42 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.01 | 6.01 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 6.79 | 8.79 |
| FR1 mapping type A | 15KHz, 5MH | TDLB100-400 | MCS2 | 1+1 | -7.08 | -5.08 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.56 | 6.56 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 7.21 | 9.21 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -7.08 | -5.08 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.52 | 6.52 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 7.09 | 9.09 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -6.8 | -4.8 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.7 | 6.7 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 6.73 | 8.73 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -7.7 | -5.7 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.63 | 6.63 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 6.87 | 8.87 |
|  | 30KHZ, 100MHz | TDLB100-400 | MCS2 | 1+1 | -7.79 | -5.79 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.54 | 6.54 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 7.55 | 9.55 |
| FR1 mapping type B | 15KHz, 5MHz | TDLB100-400 | MCS2 | 1+1 | -6.63 | -4.33 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.2 | 6.2 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 6.94 | 8.94 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -6.16 | -4.16 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.13 | 6.13 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 6.98 | 8.98 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -7.3 | -5.3 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.16 | 6.13 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 6.85 | 8.85 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -7.51 | -5.51 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.1 | 6.1 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 6.92 | 8.92 |
|  | 30KHz, 100MHz | TDLB100-400 | MCS2 | 1+1 | -7.31 | -5.31 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 3.99 | 5.99 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 7.75 | 9.75 |

Table3: The target SNR performance for NR PUSCH with 1 Tx8Rx with CP-OFDM

| FR | SCS | Channel | MCS | DMRS configuration | Target SNR (ideal) | Target SNR (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 mapping type A | 15KH, 10MHz | TDLB100-400 | MCS2 | 1+1 | -9.88 | -7.68 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 1.59 | 3.59 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.91 | 5.91 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -10.25 | -8.25 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 1.51 | 3.51 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.51 | 5.51 |
| FR1 mapping type B | 15KHz, 10KHz | TDLB100-400 | MCS2 | 1+1 | -9.62 | -7.62 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 1.13 | 3.13 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.85 | 5.85 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -10.4 | -8.4 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 0.74 | 2.74 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.77 | 5.77 |
| FR1 mapping type A | 15KHz, 5MH | TDLB100-400 | MCS2 | 1+1 | -9.88 | -7.88 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 1.37 | 3.37 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.92 | 5.92 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -9.67 | -7.5 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 1.33 | 3.33 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.77 | 5.77 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -9.76 | -7.76 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 1.67 | 3.67 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.48 | 5.48 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -10.13 | -8.13 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 1.61 | 3.61 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.58 | 5.58 |
|  | 30KHZ, 100MHz | TDLB100-400 | MCS2 | 1+1 | -10.36 | -8.36 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 1.68 | 3.68 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 4.17 | 6.17 |
| FR1 mapping type B | 15KHz, 5MHz | TDLB100-400 | MCS2 | 1+1 | -9.84 | -7.84 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 0.92 | 2.92 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.81 | 5.81 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -9.61 | -7.21 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 0.94 | 2.94 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.72 | 5.72 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -10.5 | -8.5 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 0.99 | 2.99 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.82 | 5.82 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -10.6 | -8.6 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 0.9 | 2.9 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.85 | 5.85 |
|  | 30KHz, 100MHz | TDLB100-400 | MCS2 | 1+1 | -10.3 | -8.3 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 0.85 | 2.85 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 4.15 | 6.15 |

Table4: The target SNR performance for NR PUSCH with 2 Tx2Rx with CP-OFDM

| FR | SCS | Channel | MCS | DMRS configuration | Target SNR (ideal) | Target SNR (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 mapping type A | 15KH, 10MHz | TDLB100-400 | MCS2 | 1+1 | 0.71 | 3.01 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.60 | 18.60 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -0.45 | 1.65 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 18.09 | 20.09 |
| FR1 mapping type B | 15KHz, 10MMHz | TDLB100-400 | MCS2 | 1+1 | 0.65 | 3.85 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.59 | 18.59 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -0.14 | 1.86 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.28 | 18.28 |
| FR2 with PTRS | 60KHz,100MHz | TDLA30-300 | MCS16 | 1 | NA | NA |
|  |  |  |  | 1+1 | 16.53 | 19.03 |
|  | 120KHz, 100MHz | TDLA30-300 | MCS16 | 1 | 18.71 | 21.51 |
|  |  |  |  | 1+1 | 16.71 | 19.31 |
| FR2 without PTRS | 60KHz, 100MHz | TDLA30-300 | MCS2 | 1 | -0.42 | 2.08 |
|  |  |  |  | 1+1 | -1.08 | 1.42 |
|  |  | TDLA30-300 | MCS16 | 1 | NA | NA |
|  |  |  |  | 1+1 | 16.23 | 18.73 |
|  | 120KHz, 100MHz | TDLA30-300 | MCS2 | 1 | -0.14 | 2.36 |
|  |  |  |  | 1+1 | -0.39 | 2.11 |
|  |  | TDLA30-300 | MCS16 | 1 | 17.63 | 20.63 |
|  |  |  |  | 1+1 | 16.31 | 18.81 |
| FR1 mapping type A | 15KHz, 5MH | TDLB100-400 | MCS2 | 1+1 | -1.2 | 0.8 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.23 | 18.23 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | 0.82 | 2.82 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.49 | 18.49 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | 0.03 | 2.03 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.5 | 18.5 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -0.73 | 1.27 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.5 | 18.5 |
|  | 30KHZ, 100MHz | TDLB100-400 | MCS2 | 1+1 | -1.01 | 0.99 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.9 | 18.9 |
| FR1 mapping type B | 15KHz, 5MHz | TDLB100-400 | MCS2 | 1+1 | 0.22 | 2.82 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.21 | 18.21 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -0.1 | 1.9 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.55 | 18.55 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -0.8 | 1.2 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.38 | 18.38 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -0.64 | 1.36 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.37 | 18.37 |
|  | 30KHz, 100MHz | TDLB100-400 | MCS2 | 1+1 | 0.1 | 2.1 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.84 | 18.84 |
| FR2 with PTRS | 60KHz, 50MHz | TDLA30-300 | MCS16 | 1 | -13.48 | -10.98 |
|  |  |  |  | 1+1 | 16.54 | 19.04 |
|  | 120KHz, 50MHz | TDLA30-300 | MCS16 | 1 | 18.76 | 21.26 |
|  |  |  |  | 1+1 | 16.86 | 19.36 |
|  | 120KHz, 200MHz | TDLA30-300 | MCS16 | 1 | 19 | 21.5 |
|  |  |  |  | 1+1 | 16.65 | 19.15 |
| FR2 without PTRS | 60KHz, 50MHz | TDLA30-300 | MCS2 | 1 | -0.41 | 2.09 |
|  |  |  |  | 1+1 | -1.07 | 1.43 |
|  |  | TDLA30-300 | MCS16 | 1 | NA | NA |
|  |  |  |  | 1+1 | 16.55 | 19.05 |
|  | 120KHz, 50MHz | TDLA30-300 | MCS2 | 1 | -0.3 | 2.2 |
|  |  |  |  | 1+1 | -0.45 | 2.05 |
|  |  | TDLA30-300 | MCS16 | 1 | 17.87 | 20.37 |
|  |  |  |  | 1+1 | 16.29 | 18.79 |
|  | 120KHz, 200MHz | TDLA30-300 | MCS2 | 1 | -0.28 | 2.22 |
|  |  |  |  | 1+1 | -0.48 | 2.02 |
|  |  | TDLA30-300 | MCS16 | 1 | 18.05 | 20.55 |
|  |  |  |  | 1+1 | 16.06 | 18.56 |

Table5: The target SNR performance for NR PUSCH with 2 Tx4Rx with CP-OFDM

| FR | SCS | Channel | MCS | DMRS configuration | Target SNR (ideal) | Target SNR (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 mapping type A | 15KH, 10MHz | TDLB100-400 | MCS2 | 1+1 | -3.4 | -1.2 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.48 | 11.48 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -3.84 | -1.84 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.75 | 11.75 |
| FR1 mapping type B | 15KHz, 10KHz | TDLB100-400 | MCS2 | 1+1 | -4.1 | -2.1 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.46 | 11.46 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -3.92 | -1.92 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.26 | 11.26 |
| FR1 mapping type A | 15KHz, 5MH | TDLB100-400 | MCS2 | 1+1 | -3.66 | -1.26 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.19 | 11.19 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -3.23 | -1.23 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.22 | 11.22 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -3.45 | -1.35 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.86 | 11.86 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -3.83 | -1.83 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.81 | 11.81 |
|  | 30KHZ, 100MHz | TDLB100-400 | MCS2 | 1+1 | -4.53 | -2.53 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 10.03 | 12.03 |
| FR1 mapping type B | 15KHz, 5MHz | TDLB100-400 | MCS2 | 1+1 | -3.63 | -1.23 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.3 | 11.3 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -4 | -2 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.36 | 11.36 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -4.37 | -2.37 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.33 | 11.33 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -4.13 | -2.13 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.31 | 11.31 |
|  | 30KHz, 100MHz | TDLB100-400 | MCS2 | 1+1 | -4.7 | -2.7 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.45 | 11.45 |

Table6: The target SNR performance for NR PUSCH with 2 Tx8Rx with CP-OFDM

| FR | SCS | Channel | MCS | DMRS configuration | Target SNR (ideal) | Target SNR(impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 mapping type A | 15KH, 10MHz | TDLB100-400 | MCS2 | 1+1 | -7.10 | -5.10 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.23 | 7.23 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -6.44 | -4.44 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.26 | 7.26 |
| FR1 mapping type B | 15KHz, 10KHz | TDLB100-400 | MCS2 | 1+1 | -6.87 | -3.47 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.28 | 7.28 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -6.95 | -4.95 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.15 | 7.15 |
| FR1 mapping type A | 15KHz, 5MH | TDLB100-400 | MCS2 | 1+1 | -7.08 | -3.88 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.19 | 7.19 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -6.9 | -4.9 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.16 | 7.16 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -6.65 | -4.15 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.42 | 7.42 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -6.6 | -4.5 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.28 | 7.28 |
|  | 30KHZ, 100MHz | TDLB100-400 | MCS2 | 1+1 | -7.02 | -5.02 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.34 | 7.34 |
| FR1 mapping type B | 15KHz, 5MHz | TDLB100-400 | MCS2 | 1+1 | -7.01 | -4.61 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.16 | 7.16 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -6.9 | -4.9 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.24 | 7.24 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -7.7 | -5.7 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.2 | 7.2 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -7.14 | -5.14 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.18 | 7.18 |
|  | 30KHz, 100MHz | TDLB100-400 | MCS2 | 1+1 | -6.87 | -4.87 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.26 | 7.26 |

Table7: The target SNR performance for NR PUSCH with DFT-s-OFDM

| Number of Antenna | FR | SCS | Chanel | MCS | DMRS configuration | Target SNR (ideal) | Target SNR(impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM 1T2R | FR1 mapping type A | 15KH, 5MHz(25PRB) | TDLB100-400 | MCS2 | 1+1 | -3.82 | -1.82 |
|  |  | 30KHz, 10MHz(24PRB) | TDLB100-400 | MCS2 | 1+1 | -3.77 | -1.77 |
|  | FR1 mapping type B | 15KH, 5MHz(25PRB) | TDLB100-400 | MCS2 | 1+1 | -3.34 | -1.34 |
|  |  | 30KHz, 10MHz(24PRB) | TDLB100-400 | MCS2 | 1+1 | -4.55 | -2.55 |
|  | FR2 without PTRS | 60KHz, 50MHz(30PRB) | TDLA30-300 | MCS2 | 1 | -4.1 | -1.6 |
|  |  |  |  |  | 1+1 | -4.07 | -1.57 |
|  |  | 120KHz, 50MHz(30PRB) | TDLA30-300 | MCS2 | 1 | -3.64 | -1.14 |
|  |  |  |  |  | 1+1 | -3.54 | -1.04 |
| DFT-s-OFDM 1T4R | FR1 mapping type A | 15KHz, 5MH(25PRB) | TDLB100-400 | MCS2 | 1+1 | -6.57 | -4.37 |
|  |  | 30KHZ, 10MHz(24PRB) | TDLB100-400 | MCS2 | 1+1 | -6.73 | -4.73 |
|  | FR1 mapping type B | 15KHz, 5MHz(25PRB) | TDLB100-400 | MCS2 | 1+1 | -7.09 | -5.09 |
|  |  | 30KHz,10MHz(24PRB) | TDLB100-400 | MCS2 | 1+1 | -8.12 | -6.12 |
| DFT-s-OFDM 1T8R | FR1 mapping type A | 15KHz, 5MH(25PRB) | TDLB100-400 | MCS2 | 1+1 | -9.92 | -7.82 |
|  |  | 30KHZ, 10MHz(24PRB) | TDLB100-400 | MCS2 | 1+1 | -9.56 | -7.56 |
|  | FR1 mapping type B | 15KHz, 5MHz(25PRB) | TDLB100-400 | MCS2 | 1+1 | -10.4 | -8.4 |
|  |  | 30KHz,10MHz(24PRB) | TDLB100-400 | MCS2 | 1+1 | -11.3 | -9.3 |

Table 8: Ideal and impairment simulation result for NR PUCCH with 1T2R in FR1

| Format | SCS& BW | Number of bit | Numb of symbols | Number of RBs | Test metric | Ideal result | Impairment result |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Format 0 | 15Khz, 10MHz | 1 | 1 | 1 | ACK miss | 7.9 | 9.9 |
|  |  | 1 | 2 | 1 |  | 2.6 | 4.6 |
|  | 30KHz, 40MHz | 1 | 1 | 1 |  | 8.6 | 10.6 |
|  |  | 1 | 2 | 1 |  | 2.0 | 6.0 |
| Format 1 | 15Khz, 10MHz | 2 | 14 | 1 | ACK miss | -5.7 | -3.7 |
|  |  |  |  |  | NACK to ACK | -5.3 | -3.3 |
|  | 30KHz, 40MHz |  |  |  | ACK miss | -5 | -3 |
|  |  |  |  |  | NACK to ACK | -5.4 | -1.9 |
| Format2 | 15Khz, 10MHz | 4 | 1 | 4 | ACK miss | 3.4 | 5.4 |
|  |  | 22 | 2 | 9 | BLER@1% | -0.5 | 1.5 |
|  | 30KHz, 40MHz | 4 | 1 | 4 | ACK miss | 3.5 | 5.5 |
|  |  | 22 | 2 | 9 | BLER@1% | -0.3 | 1.7 |
| Format3 | 15Khz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -1.4 | 1.1 |
|  |  |  |  |  | With additional DMRS | -1.5 | 1 |
|  |  | 16 | 4 | 3 | Without additional DMRS | 0.5 | 2.5 |
|  | 30KHz, 40MHz | 16 | 14 | 1 | Without additional DMRS | -0.8 | 1.7 |
|  |  |  |  |  | With additional DMRS | -0.8 | 1.7 |
|  |  | 16 | 4 | 3 | Without additional DMRS | 0.3 | 2.3 |
| Format4 | 15Khz, 10MHz | 22 | 14 | 1 | Without additional DMRS | 0.5 | 3.0 |
|  |  |  |  |  | With additional DMRS | 0.5 | 3.0 |
|  | 30KHz, 40MHz |  |  |  | Without additional DMRS | 1.3 | 3.8 |
|  |  |  |  |  | With additional DMRS | 1.4 | 3.9 |
| Format0 | 15Khz, 5MHz | 1 | 1 | 1 | ACK miss | 8.00 | 10.00 |
|  |  | 1 | 2 | 1 |  | 1.70 | 3.70 |
|  | 15Khz, 20MHz | 1 | 1 | 1 |  | 7.80 | 9.80 |
|  |  | 1 | 2 | 1 |  | 1.80 | 3.80 |
|  | 30KHz, 10MHz | 1 | 1 | 1 |  | 8.50 | 10.50 |
|  |  | 1 | 2 | 1 |  | 3.20 | 5.20 |
|  | 30KHz, 20MHz | 1 | 1 | 1 |  | 8.70 | 10.70 |
|  |  | 1 | 2 | 1 |  | 2.10 | 4.10 |
|  | 30KHz, 100MHz | 1 | 1 | 1 |  | 8.6 | 10.6 |
|  |  | 1 | 2 | 1 |  | 2.2 | 5.2 |
| Format1 | 15Khz, 5MHz | 2 | 14 | 1 | ACK miss | -6.40 | -4.40 |
|  |  |  |  |  | NACK to ACK | -5.90 | -3.90 |
|  | 15Khz, 20MHz |  |  |  | ACK miss | -6.40 | -4.40 |
|  |  |  |  |  | NACK to ACK | -5.50 | -3.50 |
|  | 30KHz, 10MHz |  |  |  | ACK miss | -5.30 | -3.30 |
|  |  |  |  |  | NACK to ACK | -4.60 | -2.60 |
|  | 30KHz, 20MHz |  |  |  | ACK miss | -6.10 | -4.10 |
|  |  |  |  |  | NACK to ACK | -5.60 | -3.60 |
|  | 30KHz, 100MHz |  |  |  | ACK miss | -5.7 | -3.7 |
|  |  |  |  |  | NACK to ACK | -5.0 | -3.0 |
| Format2 | 15Khz, 5MHz | 4 | 1 | 4 | ACK miss | 3.50 | 5.50 |
|  |  | 22 | 2 | 9 | BLER@1% | -1.5 | 0.5 |
|  | 15Khz, 20MHz | 4 | 1 | 4 | ACK miss | 3.30 | 5.30 |
|  |  | 22 | 2 | 9 | BLER@1% | -0.3 | 1.7 |
|  | 30KHz, 10MHz | 4 | 1 | 4 | ACK miss | 3.5 | 5.5 |
|  |  | 22 | 2 | 9 | BLER@1% | -1.3 | 0.7 |
|  | 30KHz, 20MHz | 4 | 1 | 4 | ACK miss | 3.2 | 5.2 |
|  |  | 22 | 2 | 9 | BLER@1% | -0.5 | 1.5 |
|  | 30KHz, 100MHz | 4 | 1 | 4 | ACK miss | 3.0 | 5.0 |
|  |  | 22 | 2 | 9 | BLER@1% | -1.7 | 0.3 |
| Format3 | 15Khz, 5MHz | 16 | 14 | 1 | Without additional DMRS | -2.30 | 0.20 |
|  |  |  |  |  | With additional DMRS | -2.30 | 0.20 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -0.9 | 1.1 |
|  | 15Khz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -2.20 | 0.30 |
|  |  |  |  |  | With additional DMRS | -2.40 | 0.10 |
|  |  | 16 | 4 | 3 | Without additional DMRS | 0.5 | 2.5 |
|  | 30KHz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -0.90 | 1.60 |
|  |  |  |  |  | With additional DMRS | -1.10 | 1.40 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -0.8 | 1.2 |
|  | 30KHz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -1.80 | 0.70 |
|  |  |  |  |  | With additional DMRS | -2.00 | 0.50 |
|  |  | 16 | 4 | 3 | Without additional DMRS | 0.3 | 2.3 |
|  | 30KHz, 100MHz | 16 | 14 | 1 | Without additional DMRS | -1.4 | 1.1 |
|  |  |  |  |  | With additional DMRS | -1.9 | 0.6 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -0.4 | 1.6 |
| Format4 | 15Khz, 5MHz | 22 | 14 | 1 | Without additional DMRS | -0.30 | 2.20 |
|  |  |  |  |  | With additional DMRS | -0.20 | 2.30 |
|  | 15Khz, 20MHz |  |  |  | Without additional DMRS | -0.20 | 2.30 |
|  |  |  |  |  | With additional DMRS | -0.30 | 2.20 |
|  | 30KHz, 10MHz |  |  |  | Without additional DMRS | 1.00 | 3.50 |
|  |  |  |  |  | With additional DMRS | 1.10 | 3.60 |
|  | 30KHz, 20MHz |  |  |  | Without additional DMRS | 0.10 | 2.60 |
|  |  |  |  |  | With additional DMRS | 0.10 | 2.60 |
|  | 30KHz, 100MHz |  |  |  | Without additional DMRS | 0.4 | 2.9 |
|  |  |  |  |  | With additional DMRS | 0.3 | 2.8 |

Table 9: Ideal and impairment simulation result for NR PUCCH with 1T4R in FR1

| Format | SCS& BW | Number of bit | Numb of symbols | Number of RBs | Test metric | Ideal result | Impairment result |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Format 0 | 15Khz, 10MHz | 1 | 1 | 1 | ACK miss | 1.7 | 3.7 |
|  |  | 1 | 2 | 1 |  | -1.9 | 0.1 |
|  | 30KHz, 40MHz | 1 | 1 | 1 |  | 2.1 | 4.1 |
|  |  | 1 | 2 | 1 |  | -2.2 | -0.2 |
| Format 1 | 15Khz, 10MHz | 2 | 14 | 1 | ACK miss | -10.0 | -8.0 |
|  |  |  |  |  | NACK to ACK | -9.5 | -7.5 |
|  | 30KHz, 40MHz |  |  |  | ACK miss | -9.5 | -7.5 |
|  |  |  |  |  | NACK to ACK | -8.7 | -6.7 |
| Format2 | 15Khz, 10MHz | 4 | 1 | 4 | ACK miss | -2.4 | -0.4 |
|  |  | 22 | 2 | 9 | BLER@1% | -4.9 | -2.9 |
|  | 30KHz, 40MHz | 4 | 1 | 4 | ACK miss | -2.5 | -0.5 |
|  |  | 22 | 2 | 9 | BLER@1% | -5.0 | -3.0 |
| Format3 | 15Khz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -5.7 | -3.2 |
|  |  |  |  |  | With additional DMRS | -6.1 | -3.6 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -4.6 | -2.6 |
|  | 30KHz, 40MHz | 16 | 14 | 1 | Without additional DMRS | -5.2 | -2.7 |
|  |  |  |  |  | With additional DMRS | -5.5 | -3.0 |
|  |  | 16 | 14 | 3 | Without additional DMRS | -4.9 | -2.9 |
| Format4 | 15Khz, 10MHz | 22 | 14 | 1 | Without additional DMRS | -4.1 | -1.6 |
|  |  |  |  |  | With additional DMRS | -4.4 | -1.9 |
|  | 30KHz, 40MHz |  |  |  | Without additional DMRS | -3.6 | -1.1 |
|  |  |  |  |  | With additional DMRS | -3.7 | -1.2 |
| Format0 | 15Khz, 5MHz | 1 | 1 | 1 | ACK miss | 1.6 | 3.6 |
|  |  | 1 | 2 | 1 |  | -2.4 | -0.4 |
|  | 15Khz, 20MHz | 1 | 1 | 1 |  | 1.7 | 3.7 |
|  |  | 1 | 2 | 1 |  | -2.5 | -0.5 |
|  | 30KHz, 10MHz | 1 | 1 | 1 |  | 2.1 | 4.1 |
|  |  | 1 | 2 | 1 |  | -1.5 | 0.5 |
|  | 30KHz, 20MHz | 1 | 1 | 1 |  | 1.9 | 3.9 |
|  |  | 1 | 2 | 1 |  | -2.2 | -0.2 |
|  | 30KHz, 100MHz | 1 | 1 | 1 |  | 2.1 | 4.1 |
|  |  | 1 | 2 | 1 |  | -1.5 | 0.5 |
| Format1 | 15Khz, 5MHz | 2 | 14 | 1 | ACK miss | -10.4 | -8.4 |
|  |  |  |  |  | NACK to ACK | -9.9 | -7.9 |
|  | 15Khz, 20MHz |  |  |  | ACK miss | -10.5 | -8.5 |
|  |  |  |  |  | NACK to ACK | -9.9 | -7.9 |
|  | 30KHz, 10MHz |  |  |  | ACK miss | -9.6 | -7.6 |
|  |  |  |  |  | NACK to ACK | -9.3 | -7.3 |
|  | 30KHz, 20MHz |  |  |  | ACK miss | -10.3 | -8.3 |
|  |  |  |  |  | NACK to ACK | -9.8 | -7.8 |
|  | 30KHz, 100MHz |  |  |  | ACK miss | -10.0 | -8.0 |
|  |  |  |  |  | NACK to ACK | -9.4 | -7.4 |
| Format2 | 15Khz, 5MHz | 4 | 1 | 4 | ACK miss | -2.3 | -0.3 |
|  |  | 22 | 2 | 9 | BLER@1% | -5.7 | -3.7 |
|  | 15Khz, 20MHz | 4 | 1 | 4 | ACK miss | -2.4 | -0.4 |
|  |  | 22 | 2 | 9 | BLER@1% | -4.9 | -2.9 |
|  | 30KHz, 10MHz | 4 | 1 | 4 | ACK miss | -2.4 | -0.4 |
|  |  | 22 | 2 | 9 | BLER@1% | -5.6 | -3.6 |
|  | 30KHz, 20MHz | 4 | 1 | 4 | ACK miss | -2.5 | -0.5 |
|  |  | 22 | 2 | 9 | BLER@1% | -4.8 | -2.8 |
|  | 30KHz, 100MHz | 4 | 1 | 4 | ACK miss | -2.1 | -0.1 |
|  |  | 22 | 2 | 9 | BLER@1% | -5.4 | -3.4 |
| Format3 | 15Khz, 5MHz | 16 | 14 | 1 | Without additional DMRS | -6.2 | -3.7 |
|  |  |  |  |  | With additional DMRS | -6.6 | -4.1 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -5.5 | -3.5 |
|  | 15Khz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -6.3 | -3.8 |
|  |  |  |  |  | With additional DMRS | -6.7 | -4.2 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -4.6 | -2.6 |
|  | 30KHz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -5.4 | -2.9 |
|  |  |  |  |  | With additional DMRS | -5.7 | -3.2 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -5.7 | -3.7 |
|  | 30KHz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -6 | -3.5 |
|  |  |  |  |  | With additional DMRS | -6.4 | -3.9 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -4.8 | -2.8 |
|  | 30KHz, 100MHz | 16 | 14 | 1 | Without additional DMRS | -5.7 | -3.2 |
|  |  |  |  |  | With additional DMRS | -6.7 | -4.2 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -5.0 | -3.0 |
| Format4 | 15Khz, 5MHz | 22 | 14 | 1 | Without additional DMRS | -4.6 | -2.1 |
|  |  |  |  |  | With additional DMRS | -4.8 | -2.3 |
|  | 15Khz, 20MHz |  |  |  | Without additional DMRS | -4.7 | -2.2 |
|  |  |  |  |  | With additional DMRS | -5 | -2.5 |
|  | 30KHz, 10MHz |  |  |  | Without additional DMRS | -3.7 | -1.2 |
|  |  |  |  |  | With additional DMRS | -3.8 | -1.3 |
|  | 30KHz, 20MHz |  |  |  | Without additional DMRS | -4.5 | -2 |
|  |  |  |  |  | With additional DMRS | -4.7 | -2.2 |
|  | 30KHz, 100MHz |  |  |  | Without additional DMRS | -4.1 | -1.6 |
|  |  |  |  |  | With additional DMRS | -4.8 | -2.3 |

Table 10: Ideal and impairment simulation result for NR PUCCH with 1T8R in FR1

| Format | SCS& BW | Number of bit | Numb of symbols | Number of RBs | Test metric | Ideal result | Impairment result |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Format 0 | 15Khz, 10MHz | 1 | 1 | 1 | ACK miss | -2.5 | -0.5 |
|  |  | 1 | 2 | 1 |  | -5.3 | -3.3 |
|  | 30KHz, 40MHz | 1 | 1 | 1 |  | -2.2 | -0.2 |
|  |  | 1 | 2 | 1 |  | -5.4 | -3.4 |
| Format 1 | 15Khz, 10MHz | 2 | 14 | 1 | ACK miss | -13.2 | -11.2 |
|  |  |  |  |  | NACK to ACK | -12.9 | -10.9 |
|  | 30KHz, 40MHz |  |  |  | ACK miss | -13 | -11 |
|  |  |  |  |  | NACK to ACK | -12.7 | -10.7 |
| Format2 | 15Khz, 10MHz | 4 | 1 | 4 | ACK miss | -6.3 | -4.3 |
|  |  | 22 | 2 | 9 | BLER@1% | -8.4 | -5.4 |
|  | 30KHz, 40MHz | 4 | 1 | 4 | ACK miss | -6.3 | -4.3 |
|  |  | 22 | 2 | 9 | BLER@1% | -8.3 | -6.3 |
| Format3 | 15Khz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -9.1 | -6.6 |
|  |  |  |  |  | With additional DMRS | -9.6 | -7.1 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -8.5 | -6.5 |
|  | 30KHz, 40MHz | 16 | 14 | 1 | Without additional DMRS | -8.8 | -6.3 |
|  |  |  |  |  | With additional DMRS | -9.2 | -6.7 |
|  |  | 16 | 14 | 1 | Without additional DMRS | -8.5 | -6.5 |
| Format4 | 15Khz, 10MHz | 22 | 14 | 1 | Without additional DMRS | -7.7 | -5.2 |
|  |  |  |  |  | With additional DMRS | -8.1 | -5.6 |
|  | 30KHz, 40MHz |  |  |  | Without additional DMRS | -7.4 | -4.9 |
|  |  |  |  |  | With additional DMRS | -7.7 | -5.2 |
| Format0 | 15Khz, 5MHz | 1 | 1 | 1 | ACK miss | -2.5 | -0.5 |
|  |  | 1 | 2 | 1 |  | -5.5 | -3.5 |
|  | 15Khz, 20MHz | 1 | 1 | 1 |  | -2.5 | -0.5 |
|  |  | 1 | 2 | 1 |  | -5.6 | -3.6 |
|  | 30KHz, 10MHz | 1 | 1 | 1 |  | -2.4 | -0.4 |
|  |  | 1 | 2 | 1 |  | -5.1 | -3.1 |
|  | 30KHz, 20MHz | 1 | 1 | 1 |  | -2.2 | -0.2 |
|  |  | 1 | 2 | 1 |  | -5.3 | -3.3 |
|  | 30KHz, 100MHz | 1 | 1 | 1 |  | -2.4 | -0.4 |
|  |  | 1 | 2 | 1 |  | -5.1 | -3.1 |
| Format1 | 15Khz, 5MHz | 2 | 14 | 1 | ACK miss | -13.5 | -11.5 |
|  |  |  |  |  | NACK to ACK | -13.3 | -11.3 |
|  | 15Khz, 20MHz |  |  |  | ACK miss | -13.5 | -11.5 |
|  |  |  |  |  | NACK to ACK | -13.2 | -11.2 |
|  | 30KHz, 10MHz |  |  |  | ACK miss | -13.1 | -11.1 |
|  |  |  |  |  | NACK to ACK | -12.7 | -10.7 |
|  | 30KHz, 20MHz |  |  |  | ACK miss | -13.3 | -11.3 |
|  |  |  |  |  | NACK to ACK | -13.1 | -11.1 |
|  | 30KHz, 100MHz |  |  |  | ACK miss | -13.2 | -11.2 |
|  |  |  |  |  | NACK to ACK | -13.0 | -11.0 |
| Format2 | 15Khz, 5MHz | 4 | 1 | 4 | ACK miss | -6.3 | -4.3 |
|  |  | 22 | 2 | 9 | BLER@1% | -8.8 | -6.8 |
|  | 15Khz, 20MHz | 4 | 1 | 4 | ACK miss | -6.3 | -4.3 |
|  |  | 22 | 2 | 9 | BLER@1% | -8.3 | -6.3 |
|  | 30KHz, 10MHz | 4 | 1 | 4 | ACK miss | -6.3 | -4.3 |
|  |  | 22 | 2 | 9 | BLER@1% | -8.8 | -6.8 |
|  | 30KHz, 20MHz | 4 | 1 | 4 | ACK miss | -6.5 | -4.5 |
|  |  | 22 | 2 | 9 | BLER@1% | -8.3 | -6.3 |
|  | 30KHz, 100MHz | 4 | 1 | 4 | ACK miss | -5.8 | -3.8 |
|  |  | 22 | 2 | 9 | BLER@1% | -8.3 | -6.3 |
| Format3 | 15Khz, 5MHz | 16 | 14 | 1 | Without additional DMRS | -9.4 | -6.9 |
|  |  |  |  |  | With additional DMRS | -9.9 | -7.4 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -9 | -7 |
|  | 15Khz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -9.4 | -6.9 |
|  |  |  |  |  | With additional DMRS | -9.9 | -7.4 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -8.5 | -6.5 |
|  | 30KHz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -8.8 | -6.3 |
|  |  |  |  |  | With additional DMRS | -9.4 | -6.9 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -9.2 | -7.2 |
|  | 30KHz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -9.2 | -6.7 |
|  |  |  |  |  | With additional DMRS | -9.7 | -7.2 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -8.7 | -6.7 |
|  | 30KHz, 100MHz | 16 | 14 | 1 | Without additional DMRS | -9.1 | -6.6 |
|  |  |  |  |  | With additional DMRS | -10.1 | -7.6 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -8.5 | -6.5 |
| Format4 | 15Khz, 5MHz | 22 | 14 | 1 | Without additional DMRS | -8 | -5.5 |
|  |  |  |  |  | With additional DMRS | -8.3 | -5.8 |
|  | 15Khz, 20MHz |  |  |  | Without additional DMRS | -8 | -5.5 |
|  |  |  |  |  | With additional DMRS | -8.3 | -5.8 |
|  | 30KHz, 10MHz |  |  |  | Without additional DMRS | -7.5 | -5 |
|  |  |  |  |  | With additional DMRS | -7.9 | -5.4 |
|  | 30KHz, 20MHz |  |  |  | Without additional DMRS | -7.8 | -5.3 |
|  |  |  |  |  | With additional DMRS | -8.1 | -5.6 |
|  | 30KHz, 100MHz |  |  |  | Without additional DMRS | -7.7 | -5.2 |
|  |  |  |  |  | With additional DMRS | -8.5 | -6.0 |

Table 11: Ideal and impairment simulation result for NR PUCCH with 1T2R in FR2

| Format | SCS& BW | Number of bit | Numb of symbols | Number of RBs | Test metric | Ideal result | Impairment result |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Format 0 | 60Khz, 100MHz | 1 | 1 | 1 | ACK miss | 7.15 | 9.65 |
|  |  | 1 | 2 | 1 |  | 1.90 | 4.40 |
|  | 120KHz, 100MHz | 1 | 1 | 1 |  | 7.26 | 9.76 |
|  |  | 1 | 2 | 1 |  | 1.92 | 4.42 |
| Format 1 | 60Khz, 100MHz | 2 | 14 | 1 | ACK miss | -7.04 | -4.54 |
|  |  |  |  |  | NACK to ACK | -4.77 | -2.27 |
|  | 120KHz, 100MHz |  |  |  | ACK miss | -7.05 | -4.55 |
|  |  |  |  |  | NACK to ACK | -4.78 | -2.28 |
| Format2 | 60Khz, 100MHz | 4 | 1 | 4 | ACK miss | 2.36 | 4.86 |
|  |  | 22 | 2 | 9 | BLER@1% | -0.73 | 1.77 |
|  | 120KHz, 100MHz | 4 | 1 | 4 | ACK miss | 2.63 | 5.13 |
|  |  | 22 | 2 | 9 | BLER@1% | -0.80 | 1.70 |
| Format3 | 60Khz, 100MHz | 16 | 14 | 1 | Without additional DMRS | -1.34 | 1.16 |
|  |  |  |  |  | With additional DMRS | -2.10 | 0.40 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -0.32 | 2.18 |
|  | 120KHz, 100MHz | 16 | 14 | 1 | Without additional DMRS | -1.18 | 1.32 |
|  |  |  |  |  | With additional DMRS | -1.85 | 0.65 |
|  |  | 16 | 14 | 3 | Without additional DMRS | -0.03 | 2.47 |
| Format4 | 60Khz, 100MHz | 22 | 14 | 1 | Without additional DMRS | 2.53 | 5.03 |
|  |  |  |  |  | With additional DMRS | 1.88 | 4.38 |
|  | 120KHz, 100MHz |  |  |  | Without additional DMRS | 2.72 | 5.22 |
|  |  |  |  |  | With additional DMRS | 1.93 | 4.43 |
| Format0 | 60Khz, 50MHz | 1 | 1 | 1 | ACK miss | 7.12 | 9.62 |
|  |  | 1 | 2 | 1 |  | 2.40 | 4.90 |
|  | 120Khz, 50MHz | 1 | 1 | 1 |  | 7.35 | 9.85 |
|  |  | 1 | 2 | 1 |  | 2.33 | 4.83 |
|  | 120Khz, 200MHz | 1 | 1 | 1 |  | 7.21 | 9.71 |
|  |  | 1 | 2 | 1 |  | 1.29 | 3.79 |
| Format1 | 60Khz, 50MHz | 2 | 14 | 1 | ACK miss | -6.64 | -4.14 |
|  |  |  |  |  | NACK to ACK | -3.19 | -0.69 |
|  | 120Khz, 50MHz |  |  |  | ACK miss | -6.62 | -4.12 |
|  |  |  |  |  | NACK to ACK | -4.14 | -1.64 |
|  | 120KHz, 200MHz |  |  |  | ACK miss | -7.55 | -5.05 |
|  |  |  |  |  | NACK to ACK | -5.16 | -2.66 |
| Format2 | 60Khz, 50MHz | 4 | 1 | 4 | ACK miss | 2.15 | 4.65 |
|  |  | 22 | 2 | 9 | BLER@1% | 0.13 | 2.63 |
|  | 120Khz, 50MHz | 4 | 1 | 4 | ACK miss | 2.61 | 5.11 |
|  |  | 22 | 2 | 9 | BLER@1% | 0.6 | 3.1 |
|  | 120KHz, 200MHz | 4 | 1 | 4 | ACK miss | 2.8 | 5.3 |
|  |  | 22 | 2 | 9 | BLER@1% | -0.63 | 1.87 |
| Format3 | 60Khz, 50MHz | 16 | 14 | 1 | Without additional DMRS | -0.75 | 1.75 |
|  |  |  |  |  | With additional DMRS | -1.48 | 1.02 |
|  |  | 16 | 4 | 3 | Without additional DMRS | 0.27 | 2.77 |
|  | 120Khz, 50MHz | 16 | 14 | 1 | Without additional DMRS | -0.81 | 1.69 |
|  |  |  |  |  | With additional DMRS | -1.44 | 1.06 |
|  |  | 16 | 4 | 3 | Without additional DMRS | 0.53 | 3.03 |
|  | 120KHz, 200MHz | 16 | 14 | 1 | Without additional DMRS | -1.79 | 0.71 |
|  |  |  |  |  | With additional DMRS | -2.51 | -0.01 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -0.46 | 2.04 |
| Format4 | 60Khz, 50MHz | 22 | 14 | 1 | Without additional DMRS | 2.94 | 5.44 |
|  |  |  |  |  | With additional DMRS | 2.34 | 4.84 |
|  | 120Khz, 50MHz |  |  |  | Without additional DMRS | 3.07 | 5.57 |
|  |  |  |  |  | With additional DMRS | 2.29 | 4.79 |
|  | 120KHz, 200MHz |  |  |  | Without additional DMRS | 2.2 | 4.7 |
|  |  |  |  |  | With additional DMRS | 1.49 | 3.99 |

Table 12: Ideal and impairment simulation result for NR PRACH with 1T2R in FR1

| Format | SCS | Propagation condition | Frequency offset | Ideal results | Impairment results |
| --- | --- | --- | --- | --- | --- |
| Format 0 | 1.25KHz | AWGN | 0 | -16.38 | -14.38 |
|  |  | TDLC300-100 | 400 | -8.48 | -6.48 |
| Format A1 | 15KHz | AWGN | 0 | -10.86 | -8.86 |
|  |  | TDLC300-100 | 400 | -3.72 | -1.72 |
|  | 30KHz | AWGN | 0 | -10.73 | -8.73 |
|  |  | TDLC300-100 | 400 | -4.71 | -2.71 |
| Format A2 | 15KHz | AWGN | 0 | -13.3 | -11.3 |
|  |  | TDLC300-100 | 400 | -6.08 | -4.08 |
|  | 30KHz | AWGN | 0 | -13.18 | -11.18 |
|  |  | TDLC300-100 | 400 | -6.99 | -4.99 |
| Format A3 | 15KHz | AWGN | 0 | -14.6 | -12.6 |
|  |  | TDLC300-100 | 400 | -7.43 | -5.43 |
|  | 30KHz | AWGN | 0 | -14.47 | -12.47 |
|  |  | TDLC300-100 | 400 | -8.25 | -6.25 |
| Format B4 | 15KHz | AWGN | 0 | -16.87 | -14.87 |
|  |  | TDLC300-100 | 400 | -9.57 | -7.57 |
|  | 30KHz | AWGN | 0 | -16.69 | -14.69 |
|  |  | TDLC300-100 | 400 | -10.3 | -8.3 |
| Format C0 | 15KHz | AWGN | 0 | -8.29 | -6.29 |
|  |  | TDLC300-100 | 400 | -1.34 | 0.66 |
|  | 30KHz | AWGN | 0 | -8.15 | -6.15 |
|  |  | TDLC300-100 | 400 | -2.21 | -0.21 |
| Format C2 | 15KHz | AWGN | 0 | -13.23 | -11.23 |
|  |  | TDLC300-100 | 400 | -6.13 | -4.13 |
|  | 30KHz | AWGN | 0 | -13.18 | -11.18 |
|  |  | TDLC300-100 | 400 | -7.06 | -5.06 |

Table 13: Ideal and impairment simulation result for NR PRACH with 1T4R in FR1

| Format | SCS | Propagation condition | Frequency offset | Ideal results | Impairment results |
| --- | --- | --- | --- | --- | --- |
| Format 0 | 1.25KHz | AWGN | 0 | -18.88 | -16.88 |
|  |  | TDLC300-100 | 400 | -13.47 | -11.47 |
| Format A1 | 15KHz | AWGN | 0 | -13.27 | -11.27 |
|  |  | TDLC300-100 | 400 | -8.6 | -6.6 |
|  | 30KHz | AWGN | 0 | -13.17 | -11.17 |
|  |  | TDLC300-100 | 400 | -8.59 | -6.59 |
| Format A2 | 15KHz | AWGN | 0 | -15.49 | -13.49 |
|  |  | TDLC300-100 | 400 | -10.67 | -8.67 |
|  | 30KHz | AWGN | 0 | -15.42 | -13.42 |
|  |  | TDLC300-100 | 400 | -10.71 | -8.71 |
| Format A3 | 15KHz | AWGN | 0 | -16.77 | -14.77 |
|  |  | TDLC300-100 | 400 | -11.91 | -9.91 |
|  | 30KHz | AWGN | 0 | -16.73 | -14.73 |
|  |  | TDLC300-100 | 400 | -11.93 | -9.93 |
| Format B4 | 15KHz | AWGN | 0 | -18.89 | -16.89 |
|  |  | TDLC300-100 | 400 | -13.9 | -11.9 |
|  | 30KHz | AWGN | 0 | -18.77 | -16.77 |
|  |  | TDLC300-100 | 400 | -13.77 | -11.77 |
| Format C0 | 15KHz | AWGN | 0 | -10.84 | -8.84 |
|  |  | TDLC300-100 | 400 | -6.25 | -4.25 |
|  | 30KHz | AWGN | 0 | -10.71 | -8.71 |
|  |  | TDLC300-100 | 400 | -6.28 | -4.28 |
| Format C2 | 15KHz | AWGN | 0 | -15.49 | -13.49 |
|  |  | TDLC300-100 | 400 | -10.67 | -8.67 |
|  | 30KHz | AWGN | 0 | -15.41 | -13.41 |
|  |  | TDLC300-100 | 400 | -10.69 | -8.69 |

Table 14: Ideal and impairment simulation result for NR PRACH with 1T8R in FR1

| Format | SCS | Propagation condition | Frequency offset | Ideal results | Impairment results |
| --- | --- | --- | --- | --- | --- |
| Format 0 | 1.25KHz | AWGN | 0 | -21.25 | -19.25 |
|  |  | TDLC300-100 | 400 | -17.57 | -15.57 |
| Format A1 | 15KHz | AWGN | 0 | -15.52 | -13.49 |
|  |  | TDLC300-100 | 400 | -12.42 | -10.38 |
|  | 30KHz | AWGN | 0 | -15.41 | -13.41 |
|  |  | TDLC300-100 | 400 | -11.91 | -9.92 |
| Format A2 | 15KHz | AWGN | 0 | -17.65 | -15.63 |
|  |  | TDLC300-100 | 400 | -14.51 | -12.5 |
|  | 30KHz | AWGN | 0 | -17.51 | -15.52 |
|  |  | TDLC300-100 | 400 | -13.91 | -11.9 |
| Format A3 | 15KHz | AWGN | 0 | -18.83 | -16.8 |
|  |  | TDLC300-100 | 400 | -15.56 | -13.55 |
|  | 30KHz | AWGN | 0 | -18.78 | -16.7 |
|  |  | TDLC300-100 | 400 | -15.03 | -13.07 |
| Format B4 | 15KHz | AWGN | 0 | -20.79 | -18.76 |
|  |  | TDLC300-100 | 400 | -17.45 | -15.45 |
|  | 30KHz | AWGN | 0 | -20.67 | -18.67 |
|  |  | TDLC300-100 | 400 | -16.83 | -14.9 |
| Format C0 | 15KHz | AWGN | 0 | -13.28 | -11.28 |
|  |  | TDLC300-100 | 400 | -10.22 | -8.17 |
|  | 30KHz | AWGN | 0 | -13.16 | -11.14 |
|  |  | TDLC300-100 | 400 | -9.8 | -7.79 |
| Format C2 | 15KHz | AWGN | 0 | -17.67 | -15.62 |
|  |  | TDLC300-100 | 400 | -14.39 | -12.33 |
|  | 30KHz | AWGN | 0 | -17.57 | -15.54 |
|  |  | TDLC300-100 | 400 | -13.97 | -11.98 |

Table 15: Ideal and impairment simulation result for NR PRACH with 1T2R in FR2

| Format | SCS | Propagation condition | Frequency offset | Ideal results | Impairment results |
| --- | --- | --- | --- | --- | --- |
| Format A1 | 60KHz | AWGN | 0 | -10.7 | -8.7 |
|  |  | TDLC300-100 | 400 | -3.4 | -1.4 |
|  | 120KHz | AWGN | 0 | -10.8 | -8.8 |
|  |  | TDLC300-100 | 400 | -3.7 | -1.7 |
| Format A2 | 60KHz | AWGN | 0 | -13.1 | -11.1 |
|  |  | TDLC300-100 | 400 | -5.5 | -3.5 |
|  | 120KHz | AWGN | 0 | -12.8 | -10.8 |
|  |  | TDLC300-100 | 400 | -5.8 | -3.8 |
| Format A3 | 60KHz | AWGN | 0 | -14.5 | -12.5 |
|  |  | TDLC300-100 | 400 | -7.4 | -5.4 |
|  | 120KHz | AWGN | 0 | -14.4 | -12.4 |
|  |  | TDLC300-100 | 400 | -7.5 | -5.5 |
| Format B4 | 60KHz | AWGN | 0 | -17.3 | -15.3 |
|  |  | TDLC300-100 | 400 | -9.7 | -7.7 |
|  | 120KHz | AWGN | 0 | -17.3 | -15.3 |
|  |  | TDLC300-100 | 400 | -9.9 | -7.9 |
| Format C0 | 60KHz | AWGN | 0 | -8.1 | -6.1 |
|  |  | TDLC300-100 | 400 | -0.7 | 1.3 |
|  | 120KHz | AWGN | 0 | -8.1 | -6.1 |
|  |  | TDLC300-100 | 400 | -1.1 | 0.9 |
| Format C2 | 60KHz | AWGN | 0 | -13.1 | -11.1 |
|  |  | TDLC300-100 | 400 | -5.7 | -3.7 |
|  | 120KHz | AWGN | 0 | -13.1 | -11.1 |
|  |  | TDLC300-100 | 400 | -6 | -4 |

# 3	Conclusion

In this contribution, the updated ideal and impairment simulation results for NR PUSCH and PUCCH requirement finalization are provided.

# References

R4-1907241, WF on NR PUSCH demodulation requirements in Rel-15, Nokia

R4-1904718, WF on NR PUCCH demodulation requirements in Rel-15, Huawei