3GPP TSG-RAN WG4 Meeting #92bis    	R4-1911381

Chongqing, China, 14th – 18th Oct, 2019

Agenda item:	8.18.2

Source:	Samsung

Title:	View on BS performance requirements for NR performance enhancement

Document for:	Discussion

# 1	Introduction.

During the last RAN4 meeting, the NR performance requirement enhancement for Rel-16 is discussed based on the objective of this WI as follows:

- BS demodulation requirement - To study the tests coverage of PUSCH requirements for 30% TP test point. Limited test cases will be introduced if existing test coverage is insufficient - Additional BS demodulation requirement - For FR2 PUSCH 2T2R 16QAM, specify requirements for MCS lower than 16, considering the OTA testability limit, to replace the existing requirements for MCS 16

Some agreements are captured. In this contribution, the view on the remained issue of NR requirement enhancement for Rel-16 is provided.

# 2	Discussion

## 2.1	Background

In the last meeting, based on the objective of WI for NR performance enhancement, RAN4 group have discussed whether to introduce PUSCH requirement tests at 30%TP, however, this is no consensus for that.

In LTE, for the PUSCH requirements, two test SNR points with 30% and 70% TP are applied for specified.  For 30% test point, the target of this test point is to verify the requirement of cell edge UE and ability of HARQ combination.  For example, the minimum requirements for PUSCH with 1.4MHz channel under 1Tx2Rx and 2Tx2Rx are indicated as follows:

Table1: minimum requirements with PUSCH 1T2R under 1.4MHz for LTE

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | EPA 5Hz Low | A3-2 | 30% | -4.1 |
|  |  |  |  |  | 70% | 0.1 |
|  |  |  |  | A4-3 | 70% | 10.6 |
|  |  |  |  | A5-2 | 70% | 17.7 |
|  |  |  |  | A17-1 | 70% | 21.4 |
|  |  |  |  | A18-1 | 70% | [7.4] |
|  |  |  |  | A19-1 | 70% | [18.1] |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -2.7 |
|  |  |  |  |  | 70% | 1.8 |
|  |  |  |  | A4-1 | 30% | 4.4 |
|  |  |  |  |  | 70% | 11.3 |
|  |  |  |  | A5-1 | 70% | 18.6 |
|  |  |  | EVA 70Hz Low | A3-2 | 30% | -3.9 |
|  |  |  |  |  | 70% | 0.7 |
|  |  |  |  | A4-3 | 30% | 4.0 |
|  |  |  |  |  | 70% | 11.9 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -2.4 |
|  |  |  |  |  | 70% | 2.4 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -2.2 |
|  |  |  |  |  | 70% | 2.9 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | 4.8 |
|  |  |  |  |  | 70% | 13.5 |

Table2: minimum requirements with PUSCH 1T2R under 1.4MHz for LTE

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 2 | Normal | EPA 5Hz Low | A3-2 | 70% | 4.6 |
|  |  |  |  | A4-3 | 70% | 17.70 |
|  | 4 | Normal | EPA 5Hz Low | A3-2 | 70% | -0.1 |
|  |  |  |  | A4-3 | 70% | 11.9 |
|  | 8 | Normal | EPA 5Hz Low | A3-2 | 70% | -3.0 |
|  |  |  |  | A4-3 | 70% | 7.5 |

As for other channel bandwidth and Rx antenna configuration, the requirements have the same test parameters.  Based on the FRC table, A3-1(A4-1/2), are related with 1 PRB allocation requirements for QPSK and 16QAM. A3-2 (A4-3) is related with 6PRB allocation requirements for 1.4MHz, while there is no requirement with 30% TP for 2Tx2Rx configuration. It is straight forward that only single layer will be scheduled for cell edge UE. Meanwhile, in order to guarantee the acceptable link budget performance, small number of RB sizes and lower MCS level will be considered. Meanwhile, based on the minimum requirement specified in the spec, there is about 3dB difference compared with target SNR for 30% TP and 70% TP.

Observation 1:  30%TP test point requirement is only specified for 1Tx requirements with small RB allocation and lower MCS level in order to guarantee cell edge UE with acceptable link budget performance.

## 2.1	Requirements of 30% TP

For Rel-15 NR, RAN4 group have a lot of discussion and specified most of essential requirement of NR features.

- Channel

For UL channel, the following requirement are specified

- PUSCH with CP-OFDM

- PUSCH with DFT-s-OFDM

- UCI on PUSCH

- PUCCH with single slot for format 0, 1,2, 3 and 4

- PUCCH with multi slot for format 1

- PRACH with format 0 for long sequence, and formats A1, A2, A3, B4, C0 and C4 for short sequence without restricted set.

- SCS&BW

For SCS and channel bandwidth combination, the following requirements are specified:

- 15kHz,  5MHz, 10MHz, 20MHz

- 30kHz,  10MHz, 20MHz, 40MHz, 100MHz

- 60KHz, 50MHz, 100MHz

- 120KHz,  50MHz, 100MHz, 200MHz

- Time resource allocation

For time resource allocation, the following requirements are specified:

- Type A and Type  B with slot based for FR1

- Type B with non-slot based for FR2

- RS configuration

For RS configuration, the following requirements are specified:

- 1+1 DMRS configuration with type 1 and single-symbol structure for FR1 and FR2

- 1 DMRS configuration with type 1 and single-symbol structure for FR2

- PTRS configuration for MCS 16 and MCS 20 for FR2

- Without PTRS configuration for MCS 2,16 and MCS 20 for FR2

- Antenna configuration

For Antenna configuration, the following requirements are specified:

- 1/2Tx, 2/4/8 Rx antenna configuration for FR1

- 1/2Tx , 2 Rx antenna configuration for FR2

Due to the tight time schedule, not all NR features are covered. Generally, these requirements can satisfy the early deployment request for operators

Considering the purpose of 30% TP requirement, it is not realistic to duplicate the existing test cases for 30% TP. If RAN4 group agrees to introduce 30% TP requirement, down selection of existing test cases for requirement should be considered.  Furthermore, how to down select the corresponding test cases for 30% TP requirements should be further discussed.

Observation 2:  Considering the test purpose of 30% TP requirement, it is not realistic to duplicate the existing test cases for 30%TP, if requirements of 30% TP are introduced, how to down select of the existing test cases for requirement should be considered.

Based on the latest spec, the minimum requirements for PUSCH with different DMRS configuration, time domain allocation scheme, antenna configuration, SCS&BW combination, as well as waveform for low MCS modulation scheme are summarized as follows tables:

Table3: minimum requirements with PUSCH 2Rx in FR1

| SCS BW | MCS | Wave form | DMRS configuration | Resource allocation | SNR (dB)(70%) |
| --- | --- | --- | --- | --- | --- |
| 15KHz, 5MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-2.2] |
| 15KHz, 10MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-2.4] |
| 15KHz, 20MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-2.0] |
| 30KHz, 10MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-2.7] |
| 30KHz, 20MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-3.0] |
| 30KHz, 40MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-2.8] |
| 30KHz, 100MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-2.9] |
| 15KHz, 5MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-2.6] |
| 15KHz, 10MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-2.6] |
| 15KHz, 20MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-2.4] |
| 30KHz, 10MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-2.8] |
| 30KHz, 20MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-3.0] |
| 30KHz, 40MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-2.8] |
| 30KHz, 100MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-2.8] |
| 15KHz, 5MHz | MCS2 | DFT-s-OFDM | 1+1 | Type A | [-2.7] |
| 30KHz, 10MHz | MCS2 | DFT-s-OFDM | 1+1 | Type A | [-2.8] |
| 15KHz, 5MHz | MCS2 | DFT-s-OFDM | 1+1 | Type B | [-2.6] |
| 30KHz, 10MHz | MCS2 | DFT-s-OFDM | 1+1 | Type B | [-3.0] |

Table4: minimum requirements with PUSCH 4Rx in FR1

| SCS BW | MCS | Wave form | DMRS configuration | Resource allocation | SNR (dB)(70%) |
| --- | --- | --- | --- | --- | --- |
| 15KHz, 5MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-5.8] |
| 15KHz, 10MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-5.9] |
| 15KHz, 20MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-5.5] |
| 30KHz, 10MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-6.0] |
| 30KHz, 20MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-6.3] |
| 30KHz, 40MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-6.1] |
| 30KHz, 100MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-6.0] |
| 15KHz, 5MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-6.0] |
| 15KHz, 10MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-6.1] |
| 15KHz, 20MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-6.1] |
| 30KHz, 10MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-6.1] |
| 30KHz, 20MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-6.3] |
| 30KHz, 40MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-6.1] |
| 30KHz, 100MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-6.1] |
| 15KHz, 5MHz | MCS2 | DFT-s-OFDM | 1+1 | Type A | [-6.0] |
| 30KHz, 10MHz | MCS2 | DFT-s-OFDM | 1+1 | Type A | [-6.0] |
| 15KHz, 5MHz | MCS2 | DFT-s-OFDM | 1+1 | Type B | [-6.2] |
| 30KHz, 10MHz | MCS2 | DFT-s-OFDM | 1+1 | Type B | [-6.3] |

Table5: minimum requirements with PUSCH 8Rx in FR1

| SCS BW | MCS | Wave form | DMRS configuration | Resource allocation | SNR (dB)(70%) |
| --- | --- | --- | --- | --- | --- |
| 15KHz, 5MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-8.6] |
| 15KHz, 10MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-8.7] |
| 15KHz, 20MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-8.5] |
| 30KHz, 10MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-8.9] |
| 30KHz, 20MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-9.1] |
| 30KHz, 40MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-9.0] |
| 30KHz, 100MHz | MCS2 | CP-OFDM | 1+1 | Type A | [-9.0] |
| 15KHz, 5MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-9.0] |
| 15KHz, 10MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-9.4] |
| 15KHz, 20MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-8.8] |
| 30KHz, 10MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-9.1] |
| 30KHz, 20MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-9.3] |
| 30KHz, 40MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-9.1] |
| 30KHz, 100MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-9.1] |
| 15KHz, 5MHz | MCS2 | DFT-s-OFDM | 1+1 | Type A | [-8.8] |
| 30KHz, 10MHz | MCS2 | DFT-s-OFDM | 1+1 | Type A | [-8.7] |
| 15KHz, 5MHz | MCS2 | DFT-s-OFDM | 1+1 | Type B | [-9.0] |
| 30KHz, 10MHz | MCS2 | DFT-s-OFDM | 1+1 | Type B | [-9.1] |

Table6: minimum requirements with PUSCH 2Rx in FR2

| SCS BW | MCS | Wave form | DMRS configuration | Resource allocation | SNR (dB)(70%) |
| --- | --- | --- | --- | --- | --- |
| 60KHz, 50MHz | MCS2 | CP-OFDM | 1 | Type B | [-2.0] |
| 60KHz, 50MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-2.2] |
| 60KHz, 100MHz | MCS2 | CP-OFDM | 1 | Type B | [-2.1] |
| 60KHz, 100MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-2.4] |
| 120KHz, 50MHz | MCS2 | CP-OFDM | 1 | Type B | [-2.1] |
| 120KHz, 50MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-2.4] |
| 120KHz, 100MHz | MCS2 | CP-OFDM | 1 | Type B | [-2.4] |
| 120KHz, 100MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-2.5] |
| 120KHz, 200MHz | MCS2 | CP-OFDM | 1 | Type B | [-2.1] |
| 120KHz, 200MHz | MCS2 | CP-OFDM | 1+1 | Type B | [-2.4] |
| 60KHz, 50MHz | MCS2 | DFT-s-OFDM | 1 | Type B | [-1.8] |
| 60KHz, 50MHz | MCS2 | DFT-s-OFDM | 1+1 | Type B | [-1.9] |
| 120KHz, 50MHz | MCS2 | DFT-s-OFDM | 1 | Type B | [-1.8] |
| 120KHz, 50MHz | MCS2 | DFT-s-OFDM | 1+1 | Type B | [-1.9] |

With checking the target SNR of 30% TP, the minimum requirement will be around -6dB for 2Rx antenna configuration, for 4Rx and 8 Rx antenna configurations, the requirement will be around -9dB and -12dB. Obviously, the target SNR for 4Rx and 8Rx is very low, it should be no coverage issue. Meanwhile, the testability of different antenna configuration was already defined, hence, we prefer to only specify 30% TP requirement of 2Rx if needed.

In LTE, 30% TP requirement is specified with all the channel bandwidth, while for NR, several SCS and channel bandwidth combination are supported. Comparing with the requirement, the performance gap between different channel bandwidth is very small. Hence, in order to reduce the test effort, we prefer to only specify 30% TP requirement with one channel bandwidth for each SCS if needed.

For cell edge UE, it is straight forward that only one layer can be scheduled. Either CP-OFDM or DFT-s-OFDM can support one layer. We prefer to only specify 30% TP requirement with one waveform if needed.

Again, the performance gap between different time domain allocations is also very small. To reduce the test effort, we prefer to only specify 30% TP requirement with type A allocation scheme in FR1, and type B allocation scheme in FR2.

For different DMRS configuration, for FR1, we prefer to keep current 1+1 configuration as for the 30% TP requirement.  For FR2, both 1 and 1+1 DMRS configurations are specified for 70% TP requirement. In order to improve the channel estimation accuracy of cell edge UE, 1+1 configuration is preferred.

In summary, if 30% TP requirement is agreed, the requirement is only considered for 1 Tx requirements with small RB allocation and lower MCS level with limited test cases. The down selection of test cases is preferred as

Waveform: one of CP-OFDM and DFT-s-OFDM

Antenna configuration: 1x2

SCS&BW:  15KHz,  5MHz;

30KHz,  10MHz;

60KHz,  50MHz

120KHz, 5 0MHz

DMRS configuration: 1+1 for both FR1 and FR2

Time domain allocation: Type A

MCS: MCS2

Proposal 1:  If 30% TP requirement is introduced, the 30% TP requirement is only considered for 1Tx requirements with small RB allocation and lower MCS level with limited test cases. The down selection test cases are preferred as

Waveform: one of CP-OFDM and DFT-s-OFDM

Antenna configuration: 1x2

SCS&BW: 15KHz, 5MHz

30KHz, 10MHz

60KHz, 50MHz

120KHz, 50MHz

DMRS configuration: 1+1 for both FR1 and FR2

Time domain allocation: Type A

MCS: MCS2

## 2.2	Requirements for MCS12 with FR2 2T2R

In this subsection, considering the OTA test limitation, it is agreed to replace all the existing FR2 2T2R requirements with MCS 16 by MCS12 in TS 38.104 and TS 38.141-2, and reuse the previous agreed simulation assumption for FR2 2T2R cases with MCS16. As shown in table 7, the assumption is provided.

Table7: simulation assumption for FR2 2T2R with MCS 12

| Parameter | value |
| --- | --- |
|  | FR2 |
| Transform precoding | Disabled |
| Number of Tx | 2 |
| Number of Rx | 2 |
| Number of layers | 2 |
| Transmission scheme | Identity matrix (TPMI index 0) |
| DMRS type | type 1 |
| Number of DMRS symbols | 1(0), 1+1(0,8) |
| symbols length | 10 |
| Start symbol index | 0 |
| Time domain resource allocation type | type B |
| Frequency domain resource | Full RB allocation of the applicable BW |
| MCS index | 12 |
| Carrier frequency (GHz) | 30 |
| Propagation channel | MCS 12 TDLA 30 ns, 300Hz |
| SCS and BW | 60KHz (FR2): 50MHz, 100MHz; 120KHz: 50MHz, 100MHz, 200MHz |
| Timing offset | 0 |
| Frequency offset | 0 |
| PTRS | 16 (Configured with KP_TRS =2, LP_TRS =1) and (Not configured) |
| Code block group | Disabled |
| Frequency hopping | Disabled |
| Limited buffer rate matching | Disabled |
| Number of HARQ transmission | 4 |

As agreed in the previous meeting, the requirements of MCS2, MCS16 and MCS20 are specified for PUSCH. In order to cover two LDPC base graphs, MCS2 is based LDPC graph 2, MCS16 and MCS 20 are based LDPC graph 1. In order to alignment, the FRC tables with updated FR2 test cases are also provided as follows:

Proposal 2:  The FRC parameters for FR2 PUSCH performance requirements for MCS 12 are follows:

Table 8: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos0 and 2 transmission layer (16QAM, R=434/1024)

| Subcarrier spacing [kHz] | 60 | 60 | 120 | 120 | 120 |
| --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 434/1024 | 434/1024 | 434/1024 | 434/1024 | 434/1024 |
| Payload size (bits) | 24072 | 48168 | 11784 | 24072 | 48168 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 6 | 2 | 3 | 6 |
| Code block size including CRC (bits) (Note 2) | 8056 | 8056 | 5928 | 8056 | 8056 |
| Total number of bits per slot | 42768 | 85536 | 20736 | 42768 | 85536 |
| Total symbols per slot | 7128 | 14256 | 3456 | 7128 | 14256 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0 with l0= 0 as per Table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |

Table9: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 2 transmission layer (16QAM, R=434/1024)

| Subcarrier spacing [kHz] | 60 | 60 | 120 | 120 | 120 |
| --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 | 8 | 8 | 8 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 434/1024 | 434/1024 | 434/1024 | 434/1024 | 434/1024 |
| Payload size (bits) | 21504 | 43032 | 10504 | 21504 | 43032 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 6 | 2 | 3 | 6 |
| Code block size including CRC (bits) (Note 2) | 7200 | 7200 | 5288 | 7200 | 7200 |
| Total number of bits per slot | 50688 | 101376 | 24576 | 50688 | 101376 |
| Total symbols per slot | 12672 | 25344 | 6144 | 12672 | 25344 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0 with l0= 0 as per Table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |

# 3	Conclusion

In this contribution, our view on the NR performance enhancement is presented.

Observation 1:  30%TP test point requirement is only specified for 1Tx requirements with small RB allocation and lower MCS level in order to guarantee cell edge UE with acceptable link budget performance.

Observation 2:  Considering the test purpose of 30% TP requirement, it is not realistic to duplicate the existing test cases for 30%TP, if requirements of 30% TP are introduced, how to down select of the existing test cases for requirement should be considered.

Proposal 1:  If 30% TP requirement is introduced, the 30% TP requirement is only considered for 1Tx requirements with small RB allocation and lower MCS level with limited test cases. The down selection test cases are preferred as

Waveform: one of CP-OFDM and DFT-s-OFDM

Antenna configuration: 1x2

SCS&BW: 15KHz, 5MHz

30KHz, 10MHz

60KHz, 50MHz

120KHz, 50MHz

DMRS configuration: 1+1 for both FR1 and FR2

Time domain allocation: Type A

MCS: MCS2

Proposal 2:  The FRC parameters for FR2 PUSCH performance requirements for MCS 12 are follows:

Table8: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos0 and 2 transmission layer (16QAM, R=434/1024)

| Subcarrier spacing [kHz] | 60 | 60 | 120 | 120 | 120 |
| --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 434/1024 | 434/1024 | 434/1024 | 434/1024 | 434/1024 |
| Payload size (bits) | 24072 | 48168 | 11784 | 24072 | 48168 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 6 | 2 | 3 | 6 |
| Code block size including CRC (bits) (Note 2) | 8056 | 8056 | 5928 | 8056 | 8056 |
| Total number of bits per slot | 42768 | 85536 | 20736 | 42768 | 85536 |
| Total symbols per slot | 7128 | 14256 | 3456 | 7128 | 14256 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0 with l0= 0 as per Table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |

Table9: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 2 transmission layer (16QAM, R=434/1024)

| Subcarrier spacing [kHz] | 60 | 60 | 120 | 120 | 120 |
| --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 | 8 | 8 | 8 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 434/1024 | 434/1024 | 434/1024 | 434/1024 | 434/1024 |
| Payload size (bits) | 21504 | 43032 | 10504 | 21504 | 43032 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 6 | 2 | 3 | 6 |
| Code block size including CRC (bits) (Note 2) | 7200 | 7200 | 5288 | 7200 | 7200 |
| Total number of bits per slot | 50688 | 101376 | 24576 | 50688 | 101376 |
| Total symbols per slot | 12672 | 25344 | 6144 | 12672 | 25344 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0 with l0= 0 as per Table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |

# References

R4-1907241, WF on NR PUSCH demodulation requirements in Rel-15, Nokia

R4-1904718, WF on NR PUCCH demodulation requirements in Rel-15, Huawei