3GPP TSG-RAN WG4 Meeting #92bis              	R4-1911383

Chongqing, China, 14th – 18th Oct, 2019

Agenda item:	7.13.4.2

Source:	Samsung

Title:	Discussion and simulation results for PRACH with HST in LTE Rel-16

Document for:	Discussion

# 1	Introduction

During RAN4#92 meeting, the general issues about BS demodulation scenarios and requirements for high speed scenario in LTE Rel-16 were discussed. For PRACH, the initial simulation assumption was agreed and captured in the WF [1] as follows:

    - WF on PRACH restricted set type B

- In RAN4#90bis, the following agreement was captured

- After defining maximum Doppler shift, the necessity of a new test case for PRACH restricted set B would be discussed

- Agreement

- If we adopt the maximum Doppler shift more than 1875Hz for PUSCH HST, the new test case with frequency offset equivalent to or more the maximum Doppler shift value to be adopted will be introduced

- Simulation assumption for alignment

- Channel model: AWGN

- Antenna configuration: 1x2

- Frequency offset

- 1944Hz

- 1894Hz

- Other assumptions are same as existing PRACH restricted set type B requirement

In this contribution, the initial simulation results are presented for feasibility checking PRACH with high Frequency offset

# 2	Simulation results

## 2.1	Simulation assumption for PRACH

As captured in the WF [1], excepting for the Frequency offset, other assumptions are same as existing PRACH restricted set type B requirement.

- Test preambles

In LTE with PRACH restrict set type B, four formats requirements under high frequency offset were specified, the related test parameters are defined as

Table 1: test preambles for high speed Mode restricted set type B

| Burst format | Ncs | Logical sequence index | v |
| --- | --- | --- | --- |
| 0 | 15 | 30 | 30 |
| 1 | 100 | 168 | 20 |
| 2 | 118 | 204 | 10 |
| 3 | 137 | 264 | 0 |

- Test metric

Test metric is reusing the false alarm rate and error rate, where the target false alarm rate is 0.1% and target detection error rate is 1%.

- Timing estimation error

For AWGN, a timing estimation error occur if the estimation error of the timing of strongest path is larger than 1.04us

For AWGN, reusing the timing estimation error is 1.04us.

## 2.2	Simulation results for PRACH

In this subsection, the initial simulation results are provided with frequency offset for 1944Hz and 1875Hz as follows:

Table 2: Initial ideal simulation results for high speed mode restricted set type B

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 2 | AWGN | 1944Hz | -13.78 | -13.43 | -16.04 | -16.04 |
|  |  | AWGN | 1894Hz | -13.71 | -13.37 | -16.01 | -16.02 |

Observation 1: there is no signification different with 1944Hz and 1894Hz for restrict set type B.

Proposal 1: The requirement of frequency offset with 1944Hz can be defined for restrict set type B.

# 3	Conclusion

In this contribution, the initial simulation results are presented for alignment purpose.

Observation 1: there is no signification different with 1944Hz and 1894Hz for restrict set type B.

Proposal 1: The requirement of frequency offset with 1944Hz can be defined for restrict set type B.

# References

R4-1907747, Way forward on LTE HST BS demodulation performance requirement, NTT DOCOMO