3GPP TSG-RAN WG4 Meeting #92bis                                         R4-19XXXXX

Chongqing,PRC, 26th – 30th May, 2019

Agenda Item:   8.5.3

Source:        Samsung

Title:           Tx RF requirement for IAB node

Document for:  Approval

# Introduction

Regarding how to define IAB RF requirement RAN4 achieved below agreement in RAN4#92 meeting.

The RF requirements shall be defined in an architecture agnostic way for backhaul and access function. Both separate and shared architecture shall be kept and no priority is adopted at this stage.

Can be re-visited if any issues are found with this approach

Different types of IAB nodes could be specified if needed

This contribution will discuss IAB RX RF requirement further based on this consensus. And in companion contribution of [4], further discussion on how to define TX RF requirement is presented.

# Discussion

According to skeleton agreed in [1] for IAB TS there would be separated main clauses for conducted TRX requirement and OTA TRX requirement. For conducted serious requirements that is requested to be included to facilitate the testing effort for FR1 IAB.

According to the previous discussion the IAB-DU would play the role of purely BS in view of RAN4 as common understanding. However, there could be two implementation solutions for IAB as one is IAB-MT and IAB-DU convergence on most RF HW(category I), and the other is IAB to be built by two relative isolated components of BS(DU)+UE(MT)(category II) . And it is proposed to develop the RF requirement with the target to accommodate with both implementation solutions. For some of the UE specific behaviour(s) which cannot or doesn’t necessary be supported by category I IAB, the requirement can be defined dedicated for category II IAB node. On this other hand, some of the UE requirement may be stricter than BS.

Hence there are two options here for consideration in the angle of both system performance and implementation flexibility.

Option 1: define one identical requirement regardless the functionality

Option 2: define two sets of requirement but allow the flexibility for IAB declaration on which to be applied.

For example IAB can declare to support identical capability for DU and MT as black box then one set of requirement would be applied. If it declares different capability then different requirement would be applied to different connector(s), boundary or operating mode accordingly.

The other dimensionality considered here is whether to define requirement for downlink channel reception and/or uplink channel reception separately, dedicatedly or only. For most of the RX requirement except spurious emission and in-channel selectivity, it seems both FRC for DL physical channel and UL physical channel should be considered.

In following part the requirement would be discussed one by one on how to construct the RF RX specification higher order sub-clause.

## Conducted RX requirements for FR1

REFSENS

Both Downlink channel reception and Uplink channel reception for IAB node REFSENS should be defined. In Relay specification, the 1dB tightened NF compared to UE is used for Downlink channel reception, and Small cell BS NF is adopted for Uplink channel reception. This could be considered to be applied for FR1 conducted REFSENS.

Proposal 1: Both Downlink channel reception and Uplink channel reception for IAB node conducted REFSENS should be defined.

Dynamic range

For BS reception the dynamic range is verification to ensure BS can maintain normal operation in the presence of increased interference and high wanted signal levels. However, for UE receiver the dynamic range is defined as maximum input level derived by conditional BS TX power and MCL between BS and UE. According to co-existence study different Downlink channel maximum input level may be introduced compared to UE. Whether option2 will be applied for maximum input level will be based on outcome of evaluation.

Proposal 2-1: Dynamic range for Uplink reception and maximum input level for Downlink reception should be defined for IAB node

Proposal 2-2: FFS on whether option 2 will be applied for maximum input level

In band selectivity and blocking

Under this sub-clause both ACS requirement and in-band blocking should be addressed for both Downlink and Uplink channel reception. How to define the requirement would hinge on co-existence study.

Proposal 3: ACS and in-band blocking requirement will be FFS based on co-existence study.

Receiver spurious emission

The receiver spurious emission level for FR1 BS and UE are the same. For IAB node one common RX spurious emission requirement should be defined with TDD transmitter off mode and scaling factor will be reused for Hybrid IAB node.

Proposal 4: one common RX spurious emission should be defined for IAB node.

Out-of-band blocking

Then interference signal level and wanted signal level are different between BS and UE out-of-band blocking. Furthermore, the exclusion range of interference signal for out-of-band blocking is different between BS and UE. Consequently, the requirement on receiver out-of-band blocking will be defined for Downlink reception and Uplink reception respectively.

Proposal 5: the requirement on receiver out-of-band blocking will be defined for Downlink reception and Uplink reception respectively.

Receiver intermodulation

As mentioned in the beginning, Receiver intermodulation should be defined for both Downlink channel reception and Uplink channel reception.

Proposal 6: Receiver intermodulation should be defined for both Downlink channel reception and Uplink channel reception.

In channel selectivity

This requirement is applied for Uplink reception only. And requirement can refer to corresponding BS specification.

Proposal 7: In channel selectivity will be defined for Uplink reception only and refer to BS specification.

## OTA RX requirements for FR1 and FR2

For many aspects the similar consideration shared in sub-clause 2.1 for conducted requirement can be applied for corresponding OTA RX requirement. Hence in this sub-clause it will focus on the delta issues.

OTA sensitivity

This requirement is applied for 1-O and 1-H BS in BS specification. Consequently, this requirement can be defined for FR1 Uplink reception only.

Proposal 8: requirement on OTA sensitivity can be defined for FR1 Uplink reception only.

OTA REFSENS

This requirement is defined for BS type 1-O, 2-O and UE in NR specification. Similar to conducted REFSENS, both Downlink channel reception and Uplink channel reception for IAB node OTA REFSENS should be defined.

Similar as TX requirement, RX spherical coverage may not needed for Rel-16 fixed IAB. This can be discussed in later release when mobile IAB under discussion.

Proposal 9: both Downlink channel reception and Uplink channel reception for IAB node OTA REFSENS should be defined.

OTA dynamic range

This requirement is defined for BS type 1-O as receiver dynamic range and for UE as maximum input level. Hence at least for FR1 OTA IAB both dynamic range for Downlink reception and maximum input level for Uplink reception should be define. And for FR2 OTA IAB at least maximum input level for Uplink reception should be defined.

Proposal 10-1: for FR1 OTA IAB both dynamic range for Downlink reception and maximum input level for Uplink reception should be define.

Proposal 10-2: for FR2 OTA IAB at least maximum input level for Uplink reception should be defined.

OTA in-band selectivity and blocking

The same consideration and proposals in 2.1 for conducted ACS and in-band blocking can be applied here.

Proposal in 3 is applied for OTA in-band selectivity and blocking as well.

OTA receiver spurious emission

The same consideration and proposals in 2.1 for conducted in channel selectivity can be applied here. And the spurious emission defined for BS can be applied for IAB

Proposal 11: one common RX spurious emission, which is the same as requirement for BS, should be defined for IAB node.

Out-of-band blocking

This requirement is defined for BS type 1-O and BS type 2-O in NR specification. Hence it is proposed to adopt the BS out-of-band blocking for IAB node Uplink reception only.

Proposal 12: it is proposed to adopt the BS out-of-band blocking for IAB node Uplink reception only.

OTA receiver intermodulation

This requirement is defined for BS type 1-O and BS type 2-O in NR specification. Hence it is proposed to adopt the BS RX IM for IAB node Uplink reception only.

Proposal 13: it is proposed to adopt the BS OTA receiver intermodulation for IAB node Uplink reception only.

OTA in channel selectivity

The same consideration and proposals in 2.1 for conducted in channel selectivity can be applied here.

Proposal in 7 is applied for OTA in channel selectivity as well.

# Conclusion

The proposals for IAB RX RF requirement are summarized as tables below.

| Conducted RX items |  | Option 1 or option 2? | Distinguish for DL and UL? |
| --- | --- | --- | --- |
| REFSENS |  | Option 1 | Define both |
| Dynamic range | Dynamic range | Option 1 | Uplink reception only |
|  | Maximum input level | FFS | Downlink reception only |
| In band selectivity and blocking | ACS | FFS | FFS |
|  | In band blocking | FFS | FFS |
| Out of band blocking |  | Option 1 | Define both separately |
| Receiver spurious emission |  | Option 1 | NA |
| Receiver intermodulation |  | Option 1 | Define both |
| In channel selectivity |  | Option 1 | Uplink reception only |

| OTA RX items |  | Option 1 or option 2? | Distinguish for DL and UL? |
| --- | --- | --- | --- |
| OTA sensitivity |  | Option 1 | For FR1 Uplink reception only |
| OTA REFSENS |  | Option 1 | Define both separately |
| OTA dynamic range | Dynamic range | Option 1 | For FR1 Uplink reception only |
|  | Maximum input level | FFS | Downlink reception only |
| OTA in-band selectivity and blocking | ACS | FFS | FFS |
|  | In band blocking | FFS | FFS |
| OTA out-of-band blocking |  | Option 1 | Uplink reception only |
| OTA receiver spurious emission |  | Option 1 | NA |
| OTA Receiver intermodulation |  | Option 1 | Uplink reception only |
| OTA in channel selectivity |  | Option 1 | Uplink reception only |

# Reference

[1] R4-1910404, TS skeleton for NR IAB

[2]R4-1910589, Ad-hoc meeting minus for IAB

[3] R4-1908104, Consideration on IAB RF requirements