3GPP TSG-RAN WG4 Meeting #92bis                                                         R4-190xxxx

# Chongqing, China, 14th – 18th Oct, 2019

Agenda Item:	9.17.1.2

Source: 	Samsung

Title: 	Discussion on PMI simulation for the Tx ports larger than 8 and up to 32

Document for:	Discussion

# Introduction

In RAN4 92 meeting, a WF was approved regarding to the initial simulation assumptions for PMI reporting requirements with Tx port larger than 8 and up to 32. The agreements related to PMI performance requirements were reached in the WF as listed below:

- Type of requirements

  - PMI only

- Frequency range

  - FR1

- Number of Tx ports

  - Option 1: 16, 32

  - Option 2: 16

- Codebook type

  - Type I single panel codebook

- PMI tests:

  - Option 1: Cover wideband PMI and subband in different tests, not duplicate all the tests

  - Option 2: Only Wideband

- Test metric:

  - As baseline, Relative throughput ratio between following PMI and random PMI at SNR point corresponding to 90% TP with follow PMI

Companies were encouraged to provide initial simulation results and views on the assumptions for NR UE CSI feedback test cases.

In this contribution, we present our opinions and PMI results of 16, 32 antenna ports for alignment purpose.

# Simulation results

We provide simulation results of PMI tests with 16, and 32 CSI-RS ports as below:

Fig1: SNR--Throughput of 16x2 rank 2 MCS=20

Fig2: SNR--Throughput of 32(4,4,4,4)x2 rank 2 MCS=19

Fig3: SNR--Throughput of 32(8,2,4,4)x2 rank 2 MCS=19

Table1:  The SNR point at 90% of peak throughput and TP ratio are summarized as the following table:

|  | 16x2 (4,2,4,4) | 32x2 (4,4,4,4) | 32x2 (8,2,4,4) |
| --- | --- | --- | --- |
| SNR point/TP ratio | 11.5dB/4.14 | 10.5dB/6.375 | 10.5dB/6.375 |

# Discussion

It is observed that the performance between 32Tx with (N1=4, N2=4) and 32Tx with (N1=8, N2=2) is almost identical, thus only one 32Tx configuration is enough for performance evaluation. As for align with LTE assumptions, donwselection needs to be made, so we prefer to select 32Tx with (N1=4, N2=4) as NR PMI reporting requirement. Moreover, 16Tx with (N1=4, N2=2) is also align with corresponding LTE cases.

In WF[1], only FR1 FDD was considered , in fact TDD assumptions also should be taken into account in our opinion. Besides, wideband PMI and subband PMI in NR assumptions are covered simultaneously.

Based on the above observations, we propose the following:

Proposal1: It is preferred to select 32Tx with (N1=4, N2=4) as NR PMI reporting requirement.

Proposal2: It is advised to introduce TDD assumptions for Tx ports larger than 8 and up to 32 cases.

Proposal3: Wideband PMI and subband PMI in NR assumptions are covered simultaneously.

# Conclusions

In this contribution, we provide our views and initial PMI simulation results for Tx ports larger than 8 and up to 32 cases.  Based on the above analyses, we recommend that:

Proposal1: It is preferred to select 32Tx with (N1=4, N2=4) as NR PMI reporting requirement.

Proposal2: It is advised to introduce TDD assumptions for Tx ports larger than 8 and up to 32 cases.

Proposal3: Wideband PMI and subband PMI in NR assumptions are covered simultaneously.

# Reference

[1] R4-1910017,”Way forward on PMI reporting requirements for Tx ports larger than 8 and up to 32”, Ericsson, 3GPP RAN4 #92

[2] R4-1909242,”Simulation assumptions for CSI requirements for the number of Tx ports larger than 8 and up to 32”, Ericsson, 3GPP RAN4 #92

[3] 3GPP TS 36.101 V16.2.0, User equipment (UE) radio transmission and reception (Release 16)

# Appendix

The configurations are showed as following:

Table 1  Common parameters for all cases

| Parameter | Unit | Value |
| --- | --- | --- |
| DMRS parameters |  | Type 1, 1 front loaded symbol + 1 additional DMRS symbol |

Table 2  Simulation assumptions for 16x2

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 16 x 2 (N1,N2) = (4,2) (O1,O2) = (4,4) |
| Rank |  |  | 2 |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5 (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3 ) |  | Row 11 (0,2,4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (12,-) |
|  | CSI-RS interval and offset |  | 5/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 5/1 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | 5/1 |
| aperiodicTriggeringOffset |  |  | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00000001 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.1 FDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-3), this reported PMI cannot be applied at the eNB downlink before slot#(n+3). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 3 Simulation assumptions for 32x2

| Parameter |  | Unit | Option 1 (4x4) | Option 2 (8x2) |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 | 10 |
| Subcarrier spacing |  | kHz | 15 | 15 |
| Duplex Mode |  |  | FDD | FDD |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 |
| Antenna configuration |  |  | High XP 32 x 2 (N1,N2) = (4,4) (O1,O2) = (4,4) | High XP 32 x 2 (N1,N2) = (8,2) (O1,O2) = (4,4) |
| Rank |  |  | 2 | 2 |
| Beamforming Model |  |  | As specified in Annex B.4.1 | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) | (9,-) |
|  | CSI-RS interval and offset | slot | 5/1 | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 | 32 |
|  | CDM Type |  | CDM8 | CDM8 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 18, (0,2,8,10) | Row 18, (0,2,8,10) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) | (9,-) |
|  | CSI-RS interval and offset |  | 5/1 | 5/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Patten 0 | Patten 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 5/1 | 5/1 |
| ReportConfigType |  |  | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 1 | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 8 |
| csi-ReportingBand |  |  | 1111111 | 1111111 |
| CSI-Report interval and offset |  | slot | 5/1 | 5/1 |
| aperiodicTriggeringOffset |  |  | 0 | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,4) | (8,2) |
|  | CodebookSubsetRestriction |  | 0xFFFF | 0xFFFF |
|  | RI Restriction |  | 00000010 | 00000010 |
| Physical channel for CSI report |  |  | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 | 8 |
| Maximum number of HARQ transmission |  |  | 4 | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.2 FDD | R.PDSCH.1-6.2 FDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the eNB downlink before slot#(n+3). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |