3GPP TSG-RAN4 Meeting #92BIS 	R4-1911420

# Chongqing, China, 14 – 18 October, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for cross link interference measurements performance requirements (scetion 10) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2019-10-04 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add new section 10.1.22 for cross link interference measurement accuracy requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define measurement accuracy requirements for CLI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CLI feature is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | section10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----- << Start of Change 1>> -----

# 10	Measurement Performance requirements

## 10.1	NR measurements

### 10.1.22	CLI accuracy requirements

#### 10.1.22.1	SRS-RSRP accuracy requirements for FR1

Unless otherwise specified, the requirements for absolute accuracy of SRS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell within active BWP in FR1.

The accuracy requirements in Table 10.1.22.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for SRS-RSRP measurements are fulfilled according to Annex B.2.7 for a corresponding Band for each relevant SRS.

-	The bandwidth of SRS is 48 PRBs.

-	Different root sequence for SRS resources are not transmitted on the same symbol and same comb type.

-	The minimum distance between cyclic shifts of SRS resources on the same symbol and same comb is half the maximum number of cyclic shifts.

Table 10.1.22.1-1: SRS-RSRP absolute accuracy in FR1

|  |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Te | SRS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB |  | dB |  | dBm / SCSSRS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |  |
| [5] | TBD | < TCLI | [-3] dB | NR_TDD_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -70 |
|  |  |  |  | NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -70 |
|  |  |  |  | NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -70 |
| TBD | TBD | < TCLI | [-3] dB | NR_TDD_FR1_A, NR_TDD_FR1_C, NR_TDD_FR1_D, NR_TDD_FR1_E | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |

#### 10.1.22.2	SRS-RSRP accuracy requirements for FR2

Unless otherwise specified, the requirements for absolute accuracy of SRS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell within active BWP in FR2.

The accuracy requirements in Table 10.1.22.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for SRS-RSRP measurements are fulfilled according to Annex B.2.7 for a corresponding Band for each relevant SRS.

-	The bandwidth of SRS is 48 PRBs.

-	Different root sequence for SRS resources are not transmitted on the same symbol and same comb type.

-	The minimum distance between cyclic shifts of SRS resources on the same symbol and same comb is half the maximum number of cyclic shifts.

Table 10.1.22.2-1: SRS-RSRP absolute accuracy in FR2

|  |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Te | SRS Ês/Iot | Io Note 2 range |  |  |  |
|  |  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB |  | dB | dBm / SCSSRS Note 1 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSRS = 60 kHz | SCSSRS = 120 kHz |  |  |
| [6] | TBD | < TCLI | [-3] dB | Same value as SRS_RP in Table B.2.7-2, according to UE power class, operating band and angle of arrival |  | N/A | -70 |
| TBD | TBD | < TCLI | [-3] dB | N/A | N/A | -70 | -50 |
| NOTE 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 3: In the test cases, the SRS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |

#### 10.1.22.3	SRS-RSRP Measurement Reporting Mapping

The reporting range of SRS-RSRP for L3 reporting is defined from -140 dBm to -44 dBm with 1 dB resolution.

The mapping of measured quantity is defined in Table 10.1.22.3-1. The range in the signaling may be larger than the guaranteed accuracy range.

Table 10.1.22.3-1: SRS-SRSP measurement reporting mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| SRS_RSRP_0 | SRS-RSRP<-140 | dBm |
| SRS_RSRP_1 | -140≤ SRS-RSRP<-139 | dBm |
| SRS_RSRP_2 | -139≤ SRS-RSRP<-138 | dBm |
| SRS_RSRP_3 | -138≤ SRS-RSRP<-137 | dBm |
| SRS_RSRP_4 | -137≤ SRS-RSRP<-136 | dBm |
| ··· | ··· | ··· |
| SRS_RSRP_95 | -46≤ SRS-RSRP<-45 | dBm |
| SRS_RSRP_96 | -45≤ SRS-RSRP<-44 | dBm |
| SRS_RSRP_97 | -44≤ SRS-RSRP | dBm |
| SRS_RSRP_98Note1 | Infinity |  |
| NOTE1: ‘Infinity’ means that UE cannot detect SRS due to too strong signal to measure. |  |  |

#### 10.1.22.4	CLI-RSSI accuracy requirements for FR1

The CLI-RSSI requirements are specified in Table 10.1.22.4-1. The requirements apply for configured slot duration for CLI-RSSI measurement.

Table 10.1.22.4-1: CLI-RSSI accuracy in FR1

|  |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Te | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  |  | Maximum Io |
| dB | dB |  |  | dBm / SCS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |  |
| 3.5 | 6.5 | < TCLI | NR_TDD_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
|  |  |  | NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
| 5.5 | 8.5 | < TCLI | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirements as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

#### 10.1.22.5	CLI-RSSI accuracy requirements for FR2

The CLI-RSSI requirements are specified in Table 10.1.22.5-1. The requirements apply for configured slot duration for CLI-RSSI measurement.

Table 10.1.22.5-1: CLI-RSSI accuracy in FR2

|  |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Te | Io Note 2 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB |  | dBm / SCS Note 1 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCS = 60 kHz | SCS = 120 kHz |  |  |
| 4 | 7 | < TCLI | Same value as SRS_RP in Table B.2.7-2, according to UE power class, operating band and angle of arrival |  | N/A | -50 |
|  |  |  |  |  | N/A | -50 |
|  |  |  |  |  | N/A | -50 |
|  |  |  |  |  | N/A | -50 |
| 6 | 9 | < TCLI | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 3: In the test cases, the SRS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

#### 10.1.22.6	CLI-RSSI Measurement Reporting Mapping

The reporting range of CLI-RSSI for L3 reporting is defined from -100 dBm to -25 dBm with 1 dB resolution.

The mapping of measured quantity is defined in Table 10.1.22.6-1. The range in the signaling may be larger than the guaranteed accuracy range.

Table 10.1.22.6-1: CLI-RSSI measurement reporting mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CLI_RSSI_00 | CLI-RSSI -100 | dBm |
| CLI_RSSI_01 | -100 CLI-RSSI < -99 | dBm |
| CLI_RSSI_02 | -99 CLI-RSSI < -98 | dBm |
| ··· | ··· | ··· |
| CLI_RSSI_74 | -27 CLI-RSSI < -26 | dBm |
| CLI_RSSI_75 | -26 CLI-RSSI < -25 | dBm |
| CLI_RSSI_76 | -25 CLI-RSSI | dBm |

----- << End of Change 1>> -----