3GPP TSG-RAN4 Meeting #92bis 	R4-1911429

Chongqing, China, 14th– 18th October, 2019

Agenda item:	6.10.9

Source:	Apple

Title:	On SFTD measurement between NR PCell and intra-frequency NR neighbour cells for NR SA

Document for:	Discussion

# Introduction

In RAN2 #107, one LS was approved to send to RAN4 on SFTD measurement [1], and the main contents in that LS is duplicated as below,

RAN2 will not introduce inter-frequency SFTD measurement between NR PSCell and inter-frequency cell for EN-DC in Rel-15, it can be considered in Rel-16. RAN2 understands that current 38.133 only supports inter-frequency SFTD measurement in NR SA. From the perspective of RAN2, the problem spotted is that when the network does not have SFTD information of neighbour cells, it cannot configure accurate SMTC in the measConfig, so that the UE may be unable to search for SSBs of neighbour cells. Some companies in RAN2 think the problem exists also for intra-frequency measurement and see the need to have intra-frequency SFTD measurement. Therefore, RAN2 would like to ask RAN4 the following question: Question: Is it necessary and feasible to introduce SFTD measurement between NR PCell and intra-frequency NR neighbour cells for NR SA in Rel-15?

In this contribution, we have discussion on the necessity and feasibility of SFTD measurement between NR PCell and intra-frequency NR neighbour cells for NR SA and the draft LS is also provided in [2].

# Discussion

Before answering the questions from RAN2, we need to understand the definition NR SA in RAN4. In TS38.133, it was specified that

SA operation mode: Operation mode when the UE is configured with at least PCell and not any MR-DC. In this specification, - ‘cell’, ‘PCell’, ‘PSCell’ and ‘SCell’ refer to NR cell, NR PCell, NR PSCell, and NR SCell,

So that means NR SA can be interpreted as NR single carrier, NR-CA or NR-DC in RAN4. Then we can discuss the intra-frequency SFTD measurement case by case.

- Intra-frequency SFTD measurement on NR single carrier

In this case, the intra-frequency neighbour cell is on the same frequency layer as NR PCell. If the PCell is on FR1 TDD carrier or in FR2, it was specified in TS38.133 that “it is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.” The definition of the deriveSSB-IndexFromCell is deifned as:

## 7.7 deriveSSB-IndexFromCell tolerance ### 7.7.1 Minimum requirements When deriveSSB-IndexFromCell is enabled, the UE assumes frame boundary alignment (including half frame, subframe and slot boundary alignment) across cells on the same frequency carrier is within a tolerance not worse than min(2 SSB symbols, 1 PDSCH symbol) and the SFNs of all cells on the same frequency carrier are the same.

So it is unnecessary to configure UE to perform SFTD measurement between NR PCell and its intra-frequency neighbour cell in FR1 TDD and FR2.

Regarding FR1 FDD, the timing of PCell and its intra-frequency can be asynchronized, then it’s necessary to perform the SFTD measurement between NR PCell and its intra-frequency neighbour cell. Since both slot boundary and SFN info needs to be obtained in this case, the SFTD measurement cannot be simply performed simultaneously between serving cell and intra-frequency neighbour cell, and it’s feasible to perform this SFTD measurement with e.g. autonomous gap or interruption or normal measurement gap. However, there was no any discussion before on this kind of intra-frequency SFTD measurement, so we suggest to discuss this SFTD measurement in R16 scope.

Proposal 1: for NR single carrier case in FR1 TDD or FR2, it’s unnecessary to introduce SFTD measurement between NR PCell and intra-frequency NR neighbour cell.

Proposal 2: for NR single carrier case in FR1 FDD, it’s feasible and necessary to introduce SFTD measurement between NR PCell and intra-frequency NR neighbour cell, but it can be discussed in R16 scope due to the R15 time line.

- Intra-frequency SFTD measurement on NR-CA

In NR-CA the intra-frequency NR neighbour cell may be on the same frequency layer as NR PCell or may be on the same frequency layer as NR SCell (different from PCell).

If the intra-frequency NR neighbour cell is on the same frequency layer as NR PCell, then the conclusion on necessity and feasibility for NR-CA shall be same as NR single carrier case.

If the intra-frequency NR neighbour cell is on the same frequency layer as NR SCell but different from NR PCell, when the intra-frequency NR neighbour is FR1 TDD or in FR2, it shall be slot/SFN synchronous with NR SCell and then the time difference between this neighbour cell and NR PCell is very clear to the UE(UE maintains the timing difference between PCell and SCell in CA). Thus, it’s unnecessary to introduce SFTD measurement between NR PCell and intra-frequency NR neighbour cell if in intra-frequency NR neighbour cell is in FR1 TDD or in FR2 for NR-CA mode. When the the intra-frequency NR neighbour is FR1 FDD, it’s similar as NR single carrier case. Thus,

Proposal 3: If intra-frequency NR neighbour cell is in FR1 TDD or in FR2 for NR-CA mode, it’s unnecessary to introduce SFTD measurement between NR PCell and intra-frequency NR neighbour cell.

Proposal 4: If intra-frequency NR neighbour cell is in FR1 FDD, it’s feasible and necessary to introduce SFTD measurement between NR PCell and intra-frequency NR neighbour cell, but it can be discussed in R16 scope due to the R15 time line.

- Intra-frequency SFTD measurement on NR DC

In NR-DC the intra-frequency NR neighbour cell may be on the same frequency layer as NR PCell or may be on the same frequency layer as NR PSCell (different from PCell).

If the intra-frequency NR neighbour cell is on the same frequency layer as NR PCell in FR1, then the conclusion on necessity and feasibility for NR-DC shall be same as NR single carrier case of FR1.

However, different from NR-CA, we have scenario restriction for NR-DC in R15, it was defined as,

NR-NR dual connectivity which involves two gNB acting as Master gNB and Secondary gNB is referred to as “NR-NR dual connectivity” or “NR-DC”. NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2.

If the intra-frequency NR neighbour cell is on the same frequency layer as NR PSCell but different from NR PCell, that means this intra-frequency NR neighbour cell must be in FR2, and therefore this intra-frequency NR neighbour cell must slot/SFN synchronous with NR PSCell. The SFTD between NR PCell and PSCell is already specified in RAN4, so it is pointless to introduce SFTD measurement between NR PCell and intra-frequency NR neighbour cell for NR-DC if intra-frequency NR neighbour cell is on the same frequency layer as NR PSCell.

Proposal 5: If intra-frequency NR neighbour cell is on the same TDD frequency layer as NR PCell in NR-DC, it’s unnecessary to introduce SFTD measurement between NR PCell and intra-frequency NR neighbour cell.

Proposal 6: If intra-frequency NR neighbour cell is on the same FDD frequency layer as NR PCell in NR-DC, it’s feasible and necessary to introduce SFTD measurement between NR PCell and intra-frequency NR neighbour cell, but it can be discussed in R16 scope due to the R15 time line.

Proposal 7: If intra-frequency NR neighbour cell is on the same frequency layer as NR PSCell in NR-DC, it’s unnecessary to introduce SFTD measurement between NR PCell and intra-frequency NR neighbour cell.

The summary of the conclusion is as below,

| NR SA Mode | Frequency layer of Intra-frequency NR neighbour cell | Duplex mode and FR of Intra-frequency NR neighbour cell | Conclusion on SFTD measurement between PCell and intra-frequency NR neighbour cell |
| --- | --- | --- | --- |
| NR single carrier | Same as PCell | FR1 TDD or FR2 | unnecessary to introduce this SFTD measurement |
|  |  | FR1 FDD | feasible and necessary to introduce this SFTD measurement in R16 |
| NR-CA | Same as PCell or SCell | FR1 TDD or FR2 | unnecessary to introduce this SFTD measurement |
|  |  | FR1 FDD | feasible and necessary to introduce this SFTD measurement in R16 |
| NR-DC | Same as PCell | FR1 TDD | unnecessary to introduce this SFTD measurement |
|  |  | FR1 FDD | feasible and necessary to introduce this SFTD measurement in R16 |
|  | Same as PSCell | FR2 | unnecessary to introduce this SFTD measurement |

Proposal 8:

If intra-frequency NR neighbour cell is in FR1 TDD or in FR2, it’s unnecessary to introduce SFTD measurement between NR PCell and intra-frequency NR neighbour cell.

If intra-frequency NR neighbour cell is in FR1 FDD, it’s feasible and necessary to introduce SFTD measurement between NR PCell and intra-frequency NR neighbour cell from R16.

# Draft reply LS on SFTD measurement between NR PCell and intra-frequency NR neighbour cells for NR SA

Based on the discussion above, the draft reply LS is as below,

Question: Is it necessary and feasible to introduce SFTD measurement between NR PCell and intra-frequency NR neighbour cells for NR SA in Rel-15?

[RAN4]: If intra-frequency NR neighbour cell is in FR1 TDD or in FR2, it’s unnecessary to introduce SFTD measurement between NR PCell and intra-frequency NR neighbour cell.

If intra-frequency NR neighbour cell is in FR1 FDD, it’s feasible and necessary to introduce SFTD measurement between NR PCell and intra-frequency NR neighbour cell from R16.

# Conclusion

In this contribution, we have discussion on the feasibility and necessity of intra-frequency SFTD measurement for NR SA and the draft LS is also provided in [2].

# References

[1] 	R2-1911700, Reply LS on SFTD measurement, Huawei, RAN2 #107 meeting

[2]	R4-1911430, Reply LS on SFTD measurement between NR PCell and intra-frequency NR neighbour cells for NR SA, Apple, RAN4 #92bis meeting