3GPP TSG-RAN4 Meeting #92bis 	R4-1911431

Chongqing, China, 14th– 18th October, 2019

Agenda item:	6.10.9

Source:	Apple

Title:	On definition of SFTD measurement

Document for:	Discussion

# Introduction

In RAN1 #98, one LS was approved to send to RAN4 on SFTD measurement [1], and the main contents in that LS is duplicated as below,

RAN1 thanks RAN2 and RAN4 for the LS’s on SFTD measurement. For scenarios of EN-DC, NE-DC, NR and LTE standalone, RAN1 has discussions on the type of respective target cells on which SFTD measurement can be performed, i.e. intra-frequency or inter-frequency or inter-RAT, which is used as a label for defining a UE measurement quantity in both TS 38.215 and TS 36.214, as cited in Appendix. No RAN1 consensus is reached yet. RAN1 would like to ask RAN2 and RAN4 the following questions, Q1: For UEs configured with EN-DC, NE-DC and NR-DC, is the target cell of a SFTD measurement an intra-frequency or inter-frequency cell? Q2: For UEs configured with NR PCell but no LTE/NR PSCell, is the target NR cell of a SFTD measurement an intra-frequency or inter-frequency NR cell? Q3: For UEs configured with LTE PCell but no NR PSCell, is the target NR cell of a SFTD measurement an inter-RAT or inter-frequency NR cell? RAN1 also has discussions on how to capture in TS 38.215 the applicability “for NR SA” described in the received LS for the SFTD measurement between NR PCell and neighbouring cells, and would like to ask RAN2 and RAN4 the following questions, Q4: Is there any definition in RAN2 or RAN4 specification that could be a reference in SFTD definition?

In this contribution, we have discussion on the definition of SFTD measurement and the draft LS is also provided in [2].

# Discussion

Q1: For UEs configured with EN-DC, NE-DC and NR-DC, is the target cell of a SFTD measurement an intra-frequency or inter-frequency cell?

In RAN4 TS38.133, the SFTD measurement between PCell and PSCell for NR-DC was categorized in clause of “NR intra-frequency measurement” since the SFTD was performed on two serving cells. Even though SFTD measurement requirements of EN-DC and NE-DC has not been categorized into any intra-frequency or inter-frequency measurement clause, we would like to reuse the same logic as for NR-DC, i.e., the target cell of a SFTD measurement can be categorize as intra-frequency if it’s one of the serving cells in NR-DC, EN-DC or NE-DC.

Proposal 1: For UEs configured with EN-DC, NE-DC and NR-DC, the target cell of a SFTD measurement can be categorized as intra-frequency cell if it’s one of the serving cells other than PCell in NR-DC, EN-DC or NE-DC.

In RAN4 we have the definition for SSB based intra-frequency measurement, duplicated as below:

A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the SSB of the serving cell indicated for measurement and the centre frequency of the SSB of the neighbor cell are the same, and the subcarrier spacing of the two SSBs are also the same.

So,

if target cell of a SFTD measurement is NR cell,

- when the centre frequency of target cell SSB and the centre frequency of one NR serving cell SSB are same and the subcarrier spacing of the two SSBs are also the same, it can be defined as intra-frequency cell

- otherwise it’s inter-frequency cell

if target cell of a SFTD measurement is LTE cell,

- when the centre frequency of target cell and the centre frequency of one LTE serving cell are same, it can be defined as intra-frequency cell

- otherwise it’s inter-frequency cell

Proposal 2: For UEs configured with EN-DC, NE-DC and NR-DC,

if target cell of a SFTD measurement is NR cell,

- when the centre frequency of target cell SSB and the centre frequency of one NR serving cell SSB are same and the subcarrier spacing of the two SSBs are also the same, it can be defined as intra-frequency cell

- otherwise it’s inter-frequency cell

if target cell of a SFTD measurement is LTE cell,

- when the centre frequency of target cell and the centre frequency of one LTE serving cell are same, it can be defined as intra-frequency cell

- otherwise it’s inter-frequency cell

Q2: For UEs configured with NR PCell but no LTE/NR PSCell, is the target NR cell of a SFTD measurement an intra-frequency or inter-frequency NR cell?

If UE is configured with NR PCell but no LTE/NR PSCell, that means UE is operating on NR single carrier or NR-CA. Similar as the answer to question 1, if the centre frequency of target NR cell SSB for a SFTD measurement and the centre frequency of one NR serving cell(PCell or SCell) SSB are same and the subcarrier spacing of the two SSBs are also the same, it can be defined as intra-frequency NR cell, otherwise it’s inter-frequency.

Proposal 3: For UEs configured with NR PCell but no LTE/NR PSCell, if the centre frequency of target NR cell SSB for a SFTD measurement and the centre frequency of one NR serving cell(PCell or SCell) SSB are same and the subcarrier spacing of the two SSBs are also the same, it can be defined as intra-frequency NR cell, otherwise it’s inter-frequency.

Q3: For UEs configured with LTE PCell but no NR PSCell, is the target NR cell of a SFTD measurement an inter-RAT or inter-frequency NR cell?

In TS36.133 “E-UTRAN FDD – NR SFTD Measurements” has been categorized into the inter-RAT measurement clause, so for UEs configured with LTE PCell but no NR PSCell, the target NR cell of a SFTD measurement can be defined as an inter-RAT NR cell.

Proposal 4: for UEs configured with LTE PCell but no NR PSCell, the target NR cell of a SFTD measurement can be defined as an inter-RAT NR cell.

Q4: Is there any definition in RAN2 or RAN4 specification that could be a reference in SFTD definition?

In RAN4, SA operation mode means operation mode when the UE is configured with at least PCell and not any MR-DC, i.e. NR single carrier, NR-CA or NR-DC. So based on the existing RAN4 requirements, we have the following measurement metrics:

- inter-frequency SFTD measurement for NR single carrier (defined in TS38.133 section 9.3.8)

- inter-frequency SFTD measurement for NR-CA (defined in TS38.133 section 9.3.8)

- Intra-frequency SFTD measurement between PCell and PSCell for NR-DC (defined in TS38.133 section 9.2.5.4)

- Intra-frequency SFTD measurement for NR-DC (defined in TS38.133 section 9.3.8)

Proposal 5: NR SA operation mode in RAN4 means operation mode when the UE is configured with at least PCell and not any MR-DC, i.e. NR single carrier, NR-CA or NR-DC. RAN4 has the following measurement metrics in TS38.133,

-	inter-frequency SFTD measurement (defined in TS38.133 section 9.3.8)

- applied for NR single carrier, NR-CA and NR-DC

-	Intra-frequency SFTD measurement between PCell and PSCell for NR-DC (defined in TS38.133 section 9.2.5.4)

# Draft Reply LS on definition of SFTD measurement

Based on the discussion above, the draft reply LS is as below,

Q1: For UEs configured with EN-DC, NE-DC and NR-DC, is the target cell of a SFTD measurement an intra-frequency or inter-frequency cell?

[RAN4]: For UEs configured with EN-DC, NE-DC and NR-DC,

if target cell of a SFTD measurement is one of the serving cells other than PCell in NR-DC, EN-DC or NE-DC

- it can be defined as intra-frequency cell

else,

- if target cell of a SFTD measurement is NR cell,

  - when the centre frequency of target cell SSB and the centre frequency of one NR serving cell SSB are same and the subcarrier spacing of the two SSBs are also the same, it can be defined as intra-frequency cell

  - otherwise it’s inter-frequency cell

- if target cell of a SFTD measurement is LTE cell,

  - when the centre frequency of target cell and the centre frequency of one LTE serving cell are same, it can be defined as intra-frequency cell

  - otherwise it’s inter-frequency cell

Q2: For UEs configured with NR PCell but no LTE/NR PSCell, is the target NR cell of a SFTD measurement an intra-frequency or inter-frequency NR cell?

[RAN4]: For UEs configured with NR PCell but no LTE/NR PSCell, if the centre frequency of target NR cell SSB for a SFTD measurement and the centre frequency of one NR serving cell(PCell or SCell) SSB are same and the subcarrier spacing of the two SSBs are also the same, it can be defined as intra-frequency NR cell, otherwise it’s inter-frequency.

Q3: For UEs configured with LTE PCell but no NR PSCell, is the target NR cell of a SFTD measurement an inter-RAT or inter-frequency NR cell?

[RAN4]: for UEs configured with LTE PCell but no NR PSCell, the target NR cell of a SFTD measurement can be defined as an inter-RAT NR cell.

Q4: Is there any definition in RAN2 or RAN4 specification that could be a reference in SFTD definition?

[RAN4]: NR SA operation mode in RAN4 means operation mode when the UE is configured with at least PCell and not any MR-DC, i.e. NR single carrier, NR-CA or NR-DC. RAN4 has the following measurement metrics in TS38.133,

-	inter-frequency SFTD measurement (defined in TS38.133 section 9.3.8)

- applied for NR single carrier, NR-CA and NR-DC

-	Intra-frequency SFTD measurement between PCell and PSCell for NR-DC (defined in TS38.133 section 9.2.5.4)

# Conclusion

In this contribution, we have discussion on definition of SFTD measurement and the draft LS is also provided in [2].

Proposal 1: For UEs configured with EN-DC, NE-DC and NR-DC, the target cell of a SFTD measurement can be categorized as intra-frequency cell if it’s one of the serving cells other than PCell in NR-DC, EN-DC or NE-DC.

Proposal 2: For UEs configured with EN-DC, NE-DC and NR-DC,

if target cell of a SFTD measurement is NR cell,

- when the centre frequency of target cell SSB and the centre frequency of one NR serving cell SSB are same and the subcarrier spacing of the two SSBs are also the same, it can be defined as intra-frequency cell

- otherwise it’s inter-frequency cell

if target cell of a SFTD measurement is LTE cell,

- when the centre frequency of target cell and the centre frequency of one LTE serving cell are same, it can be defined as intra-frequency cell

- otherwise it’s inter-frequency cell

Proposal 3: For UEs configured with NR PCell but no LTE/NR PSCell, if the centre frequency of target NR cell SSB for a SFTD measurement and the centre frequency of one NR serving cell(PCell or SCell) SSB are same and the subcarrier spacing of the two SSBs are also the same, it can be defined as intra-frequency NR cell, otherwise it’s inter-frequency.

Proposal 4: for UEs configured with LTE PCell but no NR PSCell, the target NR cell of a SFTD measurement can be defined as an inter-RAT NR cell.

Proposal 5: NR SA operation mode in RAN4 means operation mode when the UE is configured with at least PCell and not any MR-DC, i.e. NR single carrier, NR-CA or NR-DC. RAN4 has the following measurement metrics in TS38.133,

-	inter-frequency SFTD measurement (defined in TS38.133 section 9.3.8)

- applied for NR single carrier, NR-CA and NR-DC

-	Intra-frequency SFTD measurement between PCell and PSCell for NR-DC (defined in TS38.133 section 9.2.5.4)

# References

[1] R1-1909819, Reply LS on SFTD measurement, Huawei, RAN1 #98 meeting

[2] R4-1911432, Reply LS on definition of SFTD measurement, Apple, RAN4 #92bis meeting