3GPP TSG-RAN4 Meeting #92bis 	R4-1911432

Chongqing, China, 14th– 18th October, 2019

Title:	Reply LS on definition for SFTD measurement

Reply To:	R1-1909819

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG 4

To:	RAN WG1

Cc:			RAN WG2

Contact Person:

Name:	Jie Cui

E-mail Address:	jie_cui@apple.com

Attachments:	N.A.

1. Overall Description:

RAN4 would like to thank RAN1 for the reply LS on SFTD measurement (R1-1909819). After discussion in RAN4, the answers to RAN1 questions are as followings,

Q1: For UEs configured with EN-DC, NE-DC and NR-DC, is the target cell of a SFTD measurement an intra-frequency or inter-frequency cell?

[RAN4]: For UEs configured with EN-DC, NE-DC and NR-DC,

if target cell of a SFTD measurement is one of the serving cells other than PCell in NR-DC, EN-DC or NE-DC

- it can be defined as intra-frequency cell

else,

- if target cell of a SFTD measurement is NR cell,

  - when the centre frequency of target cell SSB and the centre frequency of one NR serving cell SSB are same and the subcarrier spacing of the two SSBs are also the same, it can be defined as intra-frequency cell

  - otherwise it’s inter-frequency cell

- if target cell of a SFTD measurement is LTE cell,

  - when the centre frequency of target cell and the centre frequency of one LTE serving cell are same, it can be defined as intra-frequency cell

  - otherwise it’s inter-frequency cell

Q2: For UEs configured with NR PCell but no LTE/NR PSCell, is the target NR cell of a SFTD measurement an intra-frequency or inter-frequency NR cell?

[RAN4]: For UEs configured with NR PCell but no LTE/NR PSCell, if the centre frequency of target NR cell SSB for a SFTD measurement and the centre frequency of one NR serving cell(PCell or SCell) SSB are same and the subcarrier spacing of the two SSBs are also the same, it can be defined as intra-frequency NR cell, otherwise it’s inter-frequency.

Q3: For UEs configured with LTE PCell but no NR PSCell, is the target NR cell of a SFTD measurement an inter-RAT or inter-frequency NR cell?

[RAN4]: for UEs configured with LTE PCell but no NR PSCell, the target NR cell of a SFTD measurement can be defined as an inter-RAT NR cell.

Q4: Is there any definition in RAN2 or RAN4 specification that could be a reference in SFTD definition?

[RAN4]: NR SA operation mode in RAN4 means operation mode when the UE is configured with at least PCell and not any MR-DC, i.e. NR single carrier, NR-CA or NR-DC. RAN4 has the following measurement metrics in TS38.133,

-	inter-frequency SFTD measurement (defined in TS38.133 section 9.3.8)

- applied for NR single carrier, NR-CA and NR-DC

-	Intra-frequency SFTD measurement between PCell and PSCell for NR-DC (defined in TS38.133 section 9.2.5.4)

2. Actions:

To RAN WG1 group.

ACTION: 	RAN4 kindly asks RAN1 to take the above information into account in their future work.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 #93			18th – 22nd Nov 2019    	Reno, Nevada, US

TSG-RAN4 #94			24th – 28th Feb 2020 		Athens, Greece