3GPP TSG-RAN WG4 Meeting #92bis	R4-1911436

# Chongqing, China, October 14th– 18th 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | XXX | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR Correction to the units of interruption length in 38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-9-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the Tables in Section 8.2, the units of some interruption length are not specified. Also, the unit and value of interruption length in Table 8.2.2.2.1-2, Table 8.2.2.2.2-2, Table 8.2.4.2.1-2 and Table 8.2.4.2.2-2 should be corrected to ms. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the units of interruption length in 8.2 and correct the unit and value of interruption length in the aforementioned Tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The units of interruption length would remain missing. The unit and value of interruption length in the aforementioned Tables would remain wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

<start of the change>

8.2	Interruption

### 8.2.1	EN-DC Interruption

#### 8.2.1.1	Introduction

This section contains the requirements related to the interruptions on PSCell, and SCell, when

E-UTRA PCell transitions between active and non-active during DRX, or

E-UTRA PCell transitions from non-DRX to DRX, or

E-UTRA SCell in MCG or SCell in SCG is added or released, or

E-UTRA SCell in MCG or SCell in SCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either E-UTRA MCG or NR SCG, or

a supplementary UL carrier or an UL carrier is configured or de-configured, or

UL/DL BWP is switched on PSCell or SCell in SCG.

The requirements shall apply for E-UTRA-NR DC with an E-UTRA PCell.

This section contains interruptions where victim cell is PSCell or SCell belonging to SCG. Requirements for interruptions requirements when the victim cell is E-UTRA PCell or E-UTRA SCell belonging to MCG are specified in TS 36.133 [15].

For a UE which does not support per-FR measurement gaps, interruptions to the PSCell or activated SCG SCells may be caused by EUTRA PCell, EUTRA SCells or SCells on any frequency range. For UE which support per-FR gaps, interruptions to the PSCell or activated SCG SCells may be caused by EUTRA PCell, EUTRA SCells or SCells on the same frequency range as the victim cell.

8.2.1.2	Requirements

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX

Interruption on PSCell and the activated SCell if configured due to E-UTRA PCell transitions between active and non-active druing DRX when PSCell or SCell is in non-DRX are allowed with up to 1% probability of missed ACK/NACK when the configured E-UTRA PCell DRX cycle is less than 640 ms, and 0.625% probability of missed ACK/NACK is allowed when the configured E-UTRA PCell DRX cycle is 640 ms or longer. Each interruption shall not exceed X slot as defined in table 8.2.1.2.1-1.

Table 8.2.1.2.1-1: Interruption length X at transition between active and non-active during DRX

|  | NR Slot length (ms) | Interruption length X (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |
| 3 | 0.125 | 5 |  |

When both E-UTRA PCell and PSCell are in DRX, no interruption is allowed.

##### 8.2.1.2.2	Interruptions at transitions from non-DRX to DRX

Interruption on PSCell and the activated SCell if configured due to E-UTRA PCell transitions from non-DRX to DRX when PSCell or SCell is in non-DRX shall not exceed X slot as defined in table 8.2.1.2.1-1.

When PSCell and the activated SCell are in DRX, no interruption due to E-UTRA PCell transitions from non-DRX to DRX is allowed.

8.2.1.2.3	Interruptions at SCell addition/release

The requirements in this clause shall apply for the UE configured with PSCell.

When one E-UTRA SCell in MCG is added or released:

-	the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X1 slot, if the active serving cell is not in the same band as any of the E-UTRA SCells being added or released, or

-	of up to max{Y1 slot + TSMTC_duration, 5ms} if the active serving cells are in the same band as any of the E-UTRA SCells being added or released, provided the cell specific reference signals from the active serving cells and the E-UTRA SCells being added or released are available in the same slot, where TSMTC_duration is the longest SMTC duration among all above active serving cells in SCG;

Where X1 and Y1 are specified in Table 8.2.1.2.3-1.

When one SCell in SCG is added or released:

-	the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X1 slot, if the active serving cell is not in the same band as any of the SCells being added or released, or

-	of up to Y1 slot + TSMTC_duration if the active serving cells are in the same band as any of the SCells being added or released, provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot, where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being added when one SCell is added;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is released.

Where X1 and Y1 are specified in Table 8.2.1.2.3-2.

Table 8.2.1.2.3-1: Interruption length X1 and Y1 at E-UTRA SCell addition/Release

|  | NR Slot length (ms) | Interruption length X1 (slots) |  | Interruption length Y1 (slots) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Sync | Async | Sync | Async |
| 0 | 1 | 1 | 2 | 1 | 2 |
| 1 | 0.5 | 2 | 3 | 2 | 3 |
| 2 | 0.25 | 5 |  | 4 | 5 |
| 3 | 0.125 | 9 |  | 8 | 9 |

Table 8.2.1.2.3-2: Interruption length X1 and Y1 at SCell addition/Release

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) |  | Interruption length Y1 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  | 1 |
| 1 | 0.5 | 2 |  | 2 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 4 | 4 |
|  |  | Either aggressor cell or victim cell is on FR1 | 5 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 8 | 8 |
|  |  | Aggressor cell is on FR1 | 9 |  |

##### 8.2.1.2.4	Interruptions at SCell activation/deactivation

The requirements in this clause shall apply for the UE configured with PSCell and one SCell.

When one E-UTRA SCell in MCG is activated or deactivated:

-	the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X2 slot, if the active serving cell is not in the same band as any of the E-UTRA SCells being activated or deactivated, or

-	of up to max{Y2 slot + TSMTC_duration, 5ms} if the active serving cells are in the same band as any of the E-UTRA SCells being activated or deactivated, provided the cell specific reference signals from the active serving cells and the E-UTRA SCells being activated or deactivated are available in the same slot, where TSMTC_duration is the longest SMTC duration among all above active serving cells in SCG.

Where X2 and Y2 are specified in Table 8.2.1.2.4-1.

When one SCell in SCG is activated or deactivated:

-	an interruption on any serving cell in SCG:

-	of up to X2 slot, if the active serving cell is not in the same band as any of the SCells being activated or deactivated, or

-	of up to Y2 slot + TSMTC_duration if the active serving cells are in the same band as any of the SCells being activated or deactivated, provided the cell specific reference signals from the active serving cells and the SCells being activated or deactivated are available in the same slot, where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being activated when one SCell is activated;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is deactivated.

Where X2 and Y2 are specified in Table 8.2.1.2.4-2.

Table 8.2.1.2.4-1: Interruption length X2 and Y2 at E-UTRA SCell activation/deactivation

|  | NR Slot length (ms) | Interruption length X2 (slots) |  | Interruption length Y2 (slots) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Sync | Async | Sync | Async |
| 0 | 1 | 1 | 2 | 1 | 2 |
| 1 | 0.5 | 1 | 2 | 1 | 2 |
| 2 | 0.25 | 3 |  | 2 | 3 |
| 3 | 0.125 | 5 |  | 4 | 5 |

Table 8.2.1.2.4-2: Interruption length X2 and Y2 at SCell activation/deactivation

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  | 1 |
| 1 | 0.5 | 1 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |  |

##### 8.2.1.2.5	Interruptions during measurements on SCC

8.2.1.2.5.1	Interruptions during measurements on deactivated NR SCC

Interruption on PSCell and other activated NR SCell(s) during measurement on the deactivated NR SCC shall meet requirements in clause 8.2.2.2.3, where the term PCell in clause 8.2.2.2.3 shall be deemed to be replaced with PSCell.

8.2.1.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC

When one E-UTRA SCell in MCG is deactivated, the UE is allowed due to measurements on the E-UTRA SCC with the deactivated E-UTRA SCell:

-	an interruption on PSCell or any activated SCell with up to 0.5% probability of missed ACK/NACK when any of the configured measCycleSCell [15] for the deactivated E-UTRA SCells is 640 ms or longer.

-	an interruption on PSCell or any activated SCell with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell [15] for the deactivated E-UTRA SCells if indicated by the network using IE allowInterruptions [15].

Each interruption shall not exceed

-	X3 slot, if the PSCell or activated SCell is not in the same band as the E-UTRA deactivated SCC being measured, or

-	Y3 slot + SMTC duration, if the PSCell or activated SCell is in the same band as the E-UTRA deactivated SCC being measured, provided the cell specific reference signals from the PSCell or activated SCell and the E-UTRA deactivated SCC being measured are available in the same slot.

Table 8.2.1.2.5.2-1: Interruption length X3 and Y3 at measurements on deactivated E-UTRA SCC

|  | NR Slot length (ms) | Interruption length X3 (slots) |  | Interruption length Y3 (slots) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Sync | Async | Sync | Async |
| 0 | 1 | 1 | 2 | 1 | 2 |
| 1 | 0.5 | 1 | 2 | 1 | 2 |
| 2 | 0.25 | 3 |  | 2 | 3 |
| 3 | 0.125 | 5 |  | 4 | 5 |

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration

The requirements in this clause shall apply when a supplementary UL carrier or an UL carrier is configured or de-configured in NR non-standalone operation as defined in TS 38.331 [2].

When an UL carrier or supplementary UL carrier is configured or deconfigured, an interruption of up to X4 slot, is allowed during the RRC reconfiguration procedure [2] on E-UTRA PCell, all activated E-UTRA SCells, PSCell and all activated SCells within the same FR as the reconfigured uplink carrier. The interruption is for both uplink and downlink of E-UTRA PCell, all activated E-UTRA SCells, PSCell and all activated SCells within the same FR as the configured or de-configured UL.

Table 8.2.1.2.6-1: Interruption length X4 at UL carrier RRC reconfiguration

|  | NR Slot length (ms) | Interruption length X4 (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25 | 5 |  |
| 3 | 0.125 | 9 |  |

8.2.1.2.7	Interruption due to Active BWP switching Requirement

When UE receives a DCI indicating UE to switch its active BWP involving changes in any of the parameters listed in Table 8.2.1.2.7-2, the UE is allowed to cause interruption of up to X slot to other active serving cells if the UE is not capable of per-FR gap, or if the BWP switching involves SCS changing. When the BWP switch imposes changes in any of the parameters listed in Table 8.2.1.2.7-2 and the UE is capable of per-FR gap, the UE is allowed to cause interruption of up to X slot to other active serving cells in the same frequency range wherein the UE is performing BWP switching. X is defined in Table 8.2.1.2.7-1. The starting time of interruption is only allowed within the BWP switching delay TBWPswitchDelay as defined in clause 8.6.2. Interruptions are not allowed during BWP switch involving other parameter change.

When a BWP timer bwp-InactivityTimer defined in TS 38.331 [2] expires, UE is allowed to cause interruption of up to X slot to other active serving cells due to switching its active BWP involving changes in any of the parameters listed in Table 8.2.1.2.7-2 if the UE is not capable of per-FR gap, or if the BWP switching involves SCS changing. When the BWP switch imposes changes in any of the parameters listed in Table 8.2.1.2.7-2 and the UE is capable of per-FR gap the UE is allowed to cause interruption of up to X slot to other active serving cells in the same frequency range wherein the UE is performing BWP switching. X is defined in Table 8.2.1.2.7-1. The starting time of interruption is only allowed within the BWP switching delay TBWPswitchDelay as defined in clause 8.6.2. Interruptions are not allowed during BWP switch involving other parameter change.

When UE receives an RRC reconfiguration that only requests UE to switch its active BWP on one single CC, the UE is allowed to cause interruption of up to X slot to other active serving cells due to switching its active BWP involving changes in any of the parameters listed in Table 8.2.1.2.7-2 if the UE is not capable of per-FR gap, or if the BWP switching involves SCS changing. When the BWP switch imposes changes in any of the parameters listed in Table 8.2.1.2.7-2 and the UE is capable of per-FR gap the UE is allowed to cause interruption of up to X slot to other active serving cells in the same frequency range wherein the UE is performing BWP switching. X is defined in Table 8.2.1.2.7-1.The interruption is only allowed within the delay TRRCprocessingDelay + TBWPswitchDelayRRC defined in clause 8.6.3.

Table 8.2.1.2.7-1: interruption length X

|  | NR Slot length (ms) | Interruption length X (slotsnote 1) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 5 |
| Note1: If the BWP switch involves changing of SCS, the interruption due to BWP switch is determined by the larger one between the SCS before BWP switch and the SCS after the BWP switch. |  |  |

Table 8.2.1.2.7-2: Parameters which cause interruption other than SCS

| Parameters | Comment |
| --- | --- |
| locationAndBandwidth | From TS 38.331 [2] |
| nrofSRS-Ports |  |
| Editor’s note: More parameters can be added if identified |  |

### 8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation

#### 8.2.2.1	Introduction

This clause contains the requirements related to the interruptions on PCell and activated SCell if configured, when

up to 7 SCells are configured, deconfigured, activated or deactivated, or

a supplementary UL carrier or an UL carrier is configured or de-configured, or

measurements on SCC with deactivated SCell in NR SCG, or

UL/DL BWP is switched on PCell or SCell.

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

Editor’s Note:	The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure [2] for SCell addition/release or MAC control signalling [17] for SCell activation/deactivation command. How to specify this is FFS.

This clause additionally contains requirements related to interruptions at inter-frequency SFTD between PCell in FR1 and neighbour cell in FR2.

For a UE which does not support per-FR measurement gaps, interruptions to the PCell and activated SCell may be caused by SCells on any frequency range. For UE which support per-FR gaps, interruptions to PCell and activated SCell may be caused by SCells on the same frequency range as the victim cell.

#### 8.2.2.2	Requirements

8.2.2.2.1	Interruptions at SCell addition/release

When any number of SCells between one and 7 is added or released using the same RRCConnectionReconfiguration message as defined in TS 38.331 [2], the UE is allowed an interruption on any active serving cell during the RRC reconfiguration procedure as follows:

-	an interruption on any active serving cell:

-	of up to the duration shown in table 8.2.2.2.1-1, if the active serving cell is not in the same band as any of the SCells being added or released, or

-	of up to the duration shown in table 8.2.2.2.1-2, if the active serving cells are in the same band as any of the SCells being added or released, provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot.

Table 8.2.2.2.1-1: Interruption duration for SCell addition/release for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 | 1 |  |
| 1 | 0.5 | 2 |  |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 4 |
|  |  | Either aggressor cell or victim cell is on FR1 | 5 |
| 3 | 0.125 | Aggressor cell is on FR2 | 8 |
|  |  | Aggressor cell is on FR1 | 9 |
| Note: Void |  |  |  |

Table 8.2.2.2.1-2: Interruption duration for SCell addition/release for intra-band CA

|  | NR Slot length (ms) | Interruption length (ms) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration |
| 1 | 0.5 | 1 + TSMTC_duration |
| 2 | 0.25 | 1 + TSMTC_duration |
| 3 | 0.125 | 1 + TSMTC_duration |
| Note: TSMTC_duration is - the longest SMTC duration among all above active serving cells and the SCell being added when one SCell is added; - the longest SMTC duration among all active serving cells in the same band when one SCell is released. |  |  |

8.2.2.2.2	Interruptions at SCell activation/deactivation

When an intra-band SCell is activated or deactivated as defined in TS 37.340 [17], the UE is allowed

-	an interruption on any active serving cell:

-	of up to the duration shown in table 8.2.2.2.2-1, if the active serving cell is not in the same band as any of the SCells being activated or deactivated, or

-	of up to the duration shown in table 8.2.2.2.2-2, if the active serving cells are in the same band as any of the SCells being activated or deactivated provided the cell specific reference signals from the active serving cells and the SCells being activated or deactivated are available in the same slot.

Table 8.2.2.2.2-1: Interruption duration for SCell activation/deactivation for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |
| Note: Void |  |  |  |

Table 8.2.2.2.2-2: Interruption duration for SCell activation/deactivation for intra-band CA

|  | NR Slot length (ms) | Interruption length (ms) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration |
| 1 | 0.5 | 0.5 + TSMTC_duration |
| 2 | 0.25 | 0.5 + TSMTC_duration |
| 3 | 0.125 | 0.5 + TSMTC_duration |
| Note: TSMTC_duration is - the longest SMTC duration among all above active serving cells and the SCell being activated when one SCell is activated; - the longest SMTC duration among all active serving cells in the same band when one SCell is deactivated. |  |  |

8.2.2.2.3	Interruptions during measurements on SCC for intra-band CA

Interruptions on PCell due to measurements when an SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [2] is 640 ms or longer. The UE is only allowed to cause interruptions immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in Table 8.2.2.2.2-1 if the PCell is not in the same band as the deactivated SCell. Each interruption shall not exceed requirement in Table 8.2.2.2.2-2 if the PCell is in the same band as the deactivated SCell.

Interruptions on activated SCells due to measurements when an SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [2] is 640 ms or longer. The UE is only allowed to cause interruptions immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in Table 8.2.2.2.2-1 if the activated SCell is not in the same band as the deactivated SCell. Each interruption shall not exceed requirement in Table 8.2.2.2.2-2 if the activated SCell is in the same band as the deactivated SCell.

##### 8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration

The requirements in this clause shall apply when a supplementary UL carrier or an UL carrier is configured or de-configured in NR standalone carrier aggregation as defined in TS 38.331 [2].

When an UL carrier or supplementary UL carrier is configured or deconfigured, an interruption of up to the duration shown in table 8.2.2.2.4-1, is allowed during the RRC reconfiguration procedure [2] on PCell and all activated SCells within the same FR as the reconfigured uplink carrier. The interruption is for both uplink and downlink of PCell and all the activated SCells within the same FR as the configured or de-configured UL.

Table 8.2.2.2.4-1: Interruption duration for UL carrier RRC reconfiguration

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 4 |
| 3 | 0.125 | 8 |

8.2.2.2.5	Interruption due to Active BWP switching Requirement

When UE receives a DCI indicating UE to switch its active BWP involving changes in any of the parameters listed in Table 8.2.2.2.5-2, the UE is allowed to cause interruption of up to X slot to other active serving cells if the UE is not capable of per-FR gap, or if the BWP switching involves SCS changing. When the BWP switch imposes changes in any of the parameters listed in Table 8.2.2.2.5-2 and the UE is capable of per-FR gap the UE is allowed to cause interruption of up to X slot to other active serving cells in the same frequency range wherein the UE is performing BWP switching. X is defined in Table 8.2.2.2.5-1. The starting time of interruption is only allowed within the BWP switching delay TBWPswitchDelay as defined in clause 8.6.2. Interruptions are not allowed during BWP switch involving other parameter change.

When a BWP timer bwp-InactivityTimer defined in TS 38.331 [2] expires, UE is allowed to cause interruption of up to X slot to other active serving cells due to switching its active BWP involving changes in any of the parameters listed in Table 8.2.2.2.5-2 if the UE is not capable of per-FR gap, or if the BWP switching involves SCS changing. When the BWP switch imposes changes in any of the parameters listed in Table 8.2.2.2.5-2and the UE is capable of per-FR gap the UE is allowed to cause interruption of up to X slot to other active serving cells in the same frequency range wherein the UE is performing BWP switching. X is defined in Table 8.2.2.2.5-1. The starting time of interruption is only allowed within the BWP switching delay TBWPswitchDelay as defined in clause 8.6.2. Interruptions are not allowed during BWP switch involving other parameter change.

When UE receives an RRC reconfiguration that only requests UE to switch its active BWP on one single CC, the UE is allowed to cause interruption of up to X slot to other active serving cells due to switching its active BWP involving changes in any of the parameters listed in Table 8.2.2.2.5-2 if the UE is not capable of per-FR gap, or if the BWP switching involves SCS changing. When the BWP switch imposes changes in any of the parameters listed in Table 8.2.2.2.5-2 and the UE is capable of per-FR gap the UE is allowed to cause interruption of up to X slot to other active serving cells in the same frequency range wherein the UE is performing BWP switching. X is defined in Table 8.2.2.2.5-1.The interruption is only allowed within the delay TRRCprocessingDelay + TBWPswitchDelayRRC defined in clause 8.6.3.

Table 8.2.2.2.5-1: Interruption length X

|  | NR Slot length (ms) | Interruption length X (slotsnote 1) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 5 |
| Note1: If the BWP switch involves changing of SCS, the interruption due to BWP switch is determined by the larger one between the SCS before BWP switch and the SCS after the BWP switch. |  |  |

Table 8.2.2.2.5-2: Parameters which cause interruption other than SCS

| Parameters | Comment |
| --- | --- |
| locationAndBandwidth | From TS 38.331 [2] |
| nrofSRS-Ports |  |
| Editor’s note: More parameters can be added if identified |  |

##### 8.2.2.2.6	Interruptions at inter-frequency SFTD measurement

The requirements in this clause concern interruptions on PCell in FR1, as well as on activated SCells in MCG, when the UE is performing SFTD measurements on inter-frequency neighbour cell(s) in FR1 or FR2. The following requirements apply when no PSCell is configured.

For a UE with per-FR gap capability:

-	for neighbour cell in FR1:

-	the percentage of interrupted slots on uplink and downlink during the SFTD measurement period Tmeasure_SFTD1 specified in Clause 9.3.8 shall not exceed the percentages specified in Table 8.2.2.2.6-1.

-	the length of each interruption shall not exceed the number of slots specified in Table 8.2.2.2.6-2.

-	for neighbour cell in FR2:

-	no interruptions are allowed during the SFTD measurement period Tmeasure_SFTD1 specified in Clause 9.3.8.

For a UE with per-UE gap capability:

-	for neighbour cell in FR1:

-	the percentage of interrupted slots on uplink and downlink during the SFTD measurement period Tmeasure_SFTD1 specified in Clause 9.3.8 shall not exceed the percentages specified in Table 8.2.2.2.6-1.

-	the length of each interruption shall not exceed the number of slots specified in Table 8.2.2.2.6-2.

-	for neighbour cell in FR2:

-	the percentage of interrupted slots on uplink and downlink during the SFTD measurement period Tmeasure_SFTD1 specified in Clause 9.3.8 shall not exceed the percentages specified in Table 8.2.2.2.6-3.

-	the length of each interruption shall not exceed the number of slots specified in Table 8.2.2.2.6-4.

Table 8.2.2.2.6-1: Requirements on maximum percentage of interrupted slots in FR1 serving cell in inter-frequency SFTD with neighbour cell in FR1

| SFTD configuration | Serving cell µ | Neighbour cell SMTC periodicity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5ms | 10ms | 20ms | 40ms | 80ms | 160ms |
| With RSRP report | 0 | [8.4%] | [11.6%] | [6.8%] | [4.7%] | [3.7%] | [3.2%] |
|  | 1 |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |
| Without RSRP report | 0 | [11.4%] | [8.6%] | [5.7%] | [4.6%] | [4.1%] | [3.8%] |
|  | 1 |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |

Table 8.2.2.2.6-2: Interruption duration for FR1 serving cell in inter-frequency SFTD with neighbour cell in FR1

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | [1] |
| 1 | 0.5 | [2] |
| 2 | 0.25 | [4] |

Table 8.2.2.2.6-3: Requirements on maximum percentage of interrupted slots in FR1 serving cell in inter-frequency SFTD with neighbour cell in FR2

| SFTD configuration | Serving cell µ | Neighbour cell SMTC periodicity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5ms | 10ms | 20ms | 40ms | 80ms | 160ms |
| With RSRP report | 0 | [8.4%] | [11.6%] | [6.8%] | [4.7%] | [3.7%] | [3.2%] |
|  | 1 | [4.2%] | [5.8%] | [3.4%] | [2.4%] | [1.8%] | [1.6%] |
|  | 2 | [4.2%] | [5.8%] | [3.4%] | [2.4%] | [1.8%] | [1.6%] |
| Without RSRP report | 0 | [11.4%] | [8.6%] | [5.7%] | [4.6%] | [4.1%] | [3.8%] |
|  | 1 | [5.7%] | [4.3%] | [2.9%] | [2.3%] | [2.1%] | [1.9%] |
|  | 2 | [5.7%] | [4.3%] | [2.9%] | [2.3%] | [2.1%] | [1.9%] |

Table 8.2.2.2.6-4: Interruption duration for FR1 serving cell in inter-frequency SFTD with neighbour cell in FR2

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | [1] |
| 1 | 0.5 | [1] |
| 2 | 0.25 | [2] |

8.2.3	NE-DC Interruptions

8.2.3.1	Introduction

This section contains the requirements related to the interruptions on PCell and SCell, when

E-UTRA PSCell transitions between active and non-active during DRX, or

E-UTRA PSCell transitions from non-DRX to DRX, or

E-UTRA PSCell/SCell in SCG or SCell in MCG is added or released, or

E-UTRA PSCell/SCell in SCG or SCell in MCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either E-UTRA SCG or NR MCG or

PUSCH/PUCCH carrier configuration and deconfiguration in NR MCG, or

UL/DL BWP is switched on PCell or SCell in MCG.

The requirements shall apply for NE-DC with an NR PCell.

This section contains interruptions where victim cell is PCell or SCell belonging to MCG, or E-UTRA PSCell or E-UTRA SCell belonging to SCG. Requirements for interruptions requirements when the victim cell is E-UTRA PSCell or E-UTRA SCell belonging to SCG are specified in TS 36.133 [15].

For a UE which does not support per-FR measurement gaps, interruptions to the PCell, E-UTRA PSCell or activated MCG SCells may be caused by EUTRA PSCell, EUTRA SCells or SCells on any frequency range. For UE which support per-FR gaps, interruptions to the PCell, E-UTRA PSCell or activated MCG SCells may be caused by EUTRA PSCell, EUTRA SCells or SCells on the same frequency range as the victim cell.

8.2.3.2	Requirements

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX

Interruption on PCell and the activated SCell if configured due to E-UTRA PSCell transitions between active and non-active during DRX when PCell or SCell is in non-DRX are allowed with up to 1% probability of missed ACK/NACK when the configured E-UTRA PSCell DRX cycle is less than 640 ms, and 0.625% probability of missed ACK/NACK is allowed when the configured E-UTRA PCell DRX cycle is 640 ms or longer. Each interruption shall not exceed X slot as defined in table 8.2.3.2.1-1.

Each interruption shall not exceed X slot as defined in table 8.2.3.2.1-1.

Table 8.2.3.2.1-1: Interruption length X at transition between active and non-active during DRX

|  | NR Slot length (ms) | Interruption length X (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |
| 3 | 0.125 | 5 |  |

When both PCell and E-UTRA PSCell are in DRX, no interruption is allowed.

##### 8.2.3.2.2	Interruptions at transitions from non-DRX to DRX

Interruption on PCelland the activated SCell if configured due to E-UTRA PSCell transitions from non-DRX to DRX when PCell or SCell is in non-DRX shall not exceed X slot as defined in table 8.2.3.2.1-1.

8.2.3.2.3	Interruptions at PSCell/SCell addition/release

The requirements in this clause shall apply for the UE configured with E-UTRA PSCell.

When one E-UTRA PSCell/SCell in SCG is added or released:

-	the UE is allowed an interruption on any active serving cell in MCG:

-	of up to X1 slot, if the active serving cell is not in the same band as any of the E-UTRA PSCell/SCells being added or released, or

-	of up to max{Y1 slot + TSMTC_duration, 5ms} if the active serving cells are in the same band as any of the E-UTRA PSCell/SCells being added or released, provided the cell specific reference signals from the active serving cells and the E-UTRA PSCell/SCells being added or released are available in the same slot, where TSMTC_duration is the longest SMTC duration among all above activated serving cells in MCG;

Where X1 and Y1 are specified in Table 8.2.3.2.3-1.

When one SCell in MCG is added or released:

-	the UE is allowed an interruption on any activated serving cell in MCG:

-	of up to X1 slot, if the active serving cell is not in the same band as any of the SCells being added or released, or

-	of up to Y1 slot + TSMTC_duration if the active serving cells are in the same band as any of the SCells being added or released, provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot, where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in MCG and the SCell being added when one SCell is added;

-	the longest SMTC duration among all above active serving cells in MCG when one SCell is released.

Where X1 and Y1 are specified in Table 8.2.3.2.3-2.

Table 8.2.3.2.3-1: Interruption length X1 and Y1 at E-UTRA PSCell/SCell addition/release

|  | NR Slot length (ms) | Interruption length X1 (slots) |  | Interruption length Y1 (slots) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Sync | Async | Sync | Async |
| 0 | 1 | 1 | 2 | 1 | 2 |
| 1 | 0.5 | 2 | 3 | 2 | 3 |
| 2 | 0.25 | 5 |  | 4 | 5 |
| 3 | 0.125 | 9 |  | 8 | 9 |

Table 8.2.3.2.3-2: Interruption length X1 and Y1 at SCell addition/Release

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) |  | Interruption length Y1 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  | 1 |
| 1 | 0.5 | 2 |  | 2 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 4 | 4 |
|  |  | Either aggressor cell or victim cell is on FR1 | 5 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 8 | 8 |
|  |  | Aggressor cell is on FR1 | 9 |  |

##### 8.2.3.2.4	Interruptions at SCell activation/deactivation

The requirements in this clause shall apply for the UE configured with E-UTRA PSCell and one SCell.

When one E-UTRA SCell in SCG is activated or deactivated:

-	the UE is allowed an interruption on any active serving cell in MCG:

-	of up to X2 slot, if the active serving cell is not in the same band as any of the E-UTRA SCells being activated or deactivated, or

-	of up to max{Y2 slot + TSMTC_duration, 5ms} if the active serving cells are in the same band as any of the E-UTRA SCells being activated or deactivated, provided the cell specific reference signals from the active serving cells and the E-UTRA SCells being activated or deactivated are available in the same slot, where TSMTC_duration is the longest SMTC duration among all above active serving cells in MCG.

Where X2 and Y2 are specified in Table 8.2.3.2.4-1.

When one SCell in MCG is activated or deactivated:

-	the UE is allowed an interruption on any serving cell in MCG:

-	of up to X2 slot, if the active serving cell is not in the same band as any of the SCells being activated or deactivated, or

-	of up to Y2 slot + TSMTC_duration if the active serving cells are in the same band as any of the SCells being activated or deactivated, provided the cell specific reference signals from the active serving cells and the SCells being activated or deactivated are available in the same slot, where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in MCG and the SCell being activated when one SCell is activated;

-	the longest SMTC duration among all above active serving cells in MCG when one SCell is deactivated.

Where X2 and Y2 are specified in Table 8.2.3.2.4-2.

Table 8.2.3.2.4-1: Interruption length X2 and Y2 at E-UTRA SCell activation/deactivation

|  | NR Slot length (ms) | Interruption length X2 (slots) |  | Interruption length Y2 (slots) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Sync | Async | Sync | Async |
| 0 | 1 | 1 | 2 | 1 | 2 |
| 1 | 0.5 | 1 | 2 | 1 | 2 |
| 2 | 0.25 | 3 |  | 2 | 3 |
| 3 | 0.125 | 5 |  | 4 | 5 |

Table 8.2.3.2.4-2: Interruption length X2 and Y2 at SCell activation/deactivation

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  | 1 |
| 1 | 0.5 | 1 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |  |

##### 8.2.3.2.5	Interruptions during measurements on SCC

8.2.3.2.5.1	Interruptions during measurements on deactivated NR SCC

Interruption on PCell and other activated SCell(s) during measurement on the deactivated NR SCC shall meet requirements in clause 8.2.2.2.3.

8.2.3.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC

When one E-UTRA SCell in SCG is deactivated, the UE is allowed due to measurements on the E-UTRA SCC with the deactivated E-UTRA SCell:

-	an interruption on PCell or any activated SCell with up to 0.5% probability of missed ACK/NACK when any of the configured measCycleSCell [15] for the deactivated E-UTRA SCells is 640 ms or longer.

-	an interruption on PCell or any activated SCell with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell [15] for the deactivated E-UTRA SCells if indicated by the network using IE allowInterruptions [15].

Each interruption shall not exceed

-	X3 slot, if the PCell or activated SCell is not in the same band as the E-UTRA deactivated SCC being measured, or

-	Y3 slot + SMTC duration, if the PCell or activated SCell is in the same band as the E-UTRA deactivated SCC being measured, provided the cell specific reference signals from the PCell or activated SCell and the E-UTRA deactivated SCC being measured are available in the same slot.

Where X3 and Y3 are specified in Table 8.2.3.2.5-1

Table 8.2.3.2.5-1: Interruption length X3 and Y3 at measurements on deactivated E-UTRA SCC

|  | NR Slot length (ms) | Interruption length X3 (slots) |  | Interruption length Y3 (slots) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Sync | Async | Sync | Async |
| 0 | 1 | 1 | 2 | 1 | 2 |
| 1 | 0.5 | 1 | 2 | 1 | 2 |
| 2 | 0.25 | 3 |  | 2 | 3 |
| 3 | 0.125 | 5 |  | 4 | 5 |

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration

The requirements in this clause shall apply when a supplementary UL carrier or an UL carrier is configured or de-configured in NE-DC.

When an UL carrier or supplementary UL carrier is configured or deconfigured, an interruption of up to X4 slot as specified in Table 8.2.3.2.6-1, is allowed during the RRC reconfiguration procedure [2] on PCell, all activated SCells within the same FR as the reconfigured uplink carrier. The interruption is for both uplink and downlink of PCell, all activated within the same FR as the configured or de-configured UL.

Table 8.2.3.2.6-1: Interruption length X4 at UL carrier RRC reconfiguration

|  | NR Slot length (ms) | Interruption length X4 (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25 | 5 |  |
| 3 | 0.125 | 9 |  |

8.2.3.2.7	Interruption due to Active BWP switching Requirement

When UE receives a DCI indicating the UE to switch its active BWP, or when a BWP timer bwp-InactivityTimer defined in TS 38.331 [2] expires, or when the UE receives an RRC command indicating the UE to switch its active BWP, the UE is allowed an interruption on PCell and any activated SCells as defined in clause 8.2.2.2.5.

8.2.4	NR-DC: Interruptions

8.2.4.1	Introduction

This section contains the requirements related to the interruptions on PCell, PSCell and activated SCell if configured, when

up to TBD SCells are configured, deconfigured, activated or deactivated deactivated or,

a supplementary UL carrier or an UL carrier is configured or de-configured, or

measurements on SCC with deactivated SCell in NR SCG, or

UL/DL BWP is switched on PCell, PSCell or SCell.

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

Editor’s Note:	The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure [2] for SCell addition/release or MAC control signalling [17] for SCell activation/deactivation command. How to specify this is FFS.

The requirements shall apply for NR-DC with an NR PCell, PSCell or SCell.

For a UE which does not support per-FR measurement gaps, interruptions to the PCell and activated SCell may be caused by SCells on any frequency range. For UE which support per-FR gaps, interruptions to PCell, PSCell and activated SCell may be caused by SCells on the same frequency range as the victim cell.

8.2.4.2	Requirements

8.2.4.2.1	Interruptions at PSCell/SCell addition/release

When PSCell or any number of SCells between one and TBD is added or released using the same RRCConnectionReconfiguration message as defined in TS 38.331 [2], the UE is allowed an interruption on any activated serving cell during the RRC reconfiguration procedure as follows:

-	an interruption on any active serving cell:

-	of up to the duration shown in table 8.2.4.2.1-1, if the active serving cell is not in the same band as any of the SCells being added or released, or

-	of up to the duration shown in table 8.2.4.2.1-2, if the active serving cells are in the same band as any of the SCells being added or released, provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot.

Table 8.2.4.2.1-1: Interruption duration for PSCell/SCell addition/release for inter-band DC/CA

|  | NR Slot length (ms) of victim cell | Interruption length (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 | 1 |  |
| 1 | 0.5 | 2 |  |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 4 |
|  |  | Either aggressor cell or victim cell is on FR1 | 5 |
| 3 | 0.125 | Aggressor cell is on FR2 | 8 |
|  |  | Aggressor cell is on FR1 | 9 |
| Note: Void |  |  |  |

Table 8.2.4.2.1-2: Interruption duration for SCell addition/release for intra-band DC/CA

|  | NR Slot length (ms) | Interruption length (ms) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration |
| 1 | 0.5 | 1 + TSMTC_duration |
| 2 | 0.25 | 1 + TSMTC_duration |
| 3 | 0.125 | 1 + TSMTC_duration |
| Note: TSMTC_duration is - the longest SMTC duration among all above activeserving cells and the SCell being added when one SCell is added; - the longest SMTC duration among all active serving cells in the same band when one SCell is released. |  |  |

8.2.4.2.2	Interruptions at SCell activation/deactivation

When a SCell is activated or deactivated as defined in TS 37.340 [17], the UE is allowed

-	an interruption on any active serving cell:

-	of up to the duration shown in table 8.2.4.2.2-1, if the active serving cell is not in the same band as any of the SCells being activated or deactivated, or

-	of up to the duration shown in table 8.2.4.2.2-2, if the active serving cells are in the same band as any of the SCells being activated or deactivated provided the cell specific reference signals from the active serving cells and the SCells being activated or deactivated are available in the same slot.

Table 8.2.4.2.2-1: Interruption duration for SCell activation/deactivation for inter-band DC/CA

|  | NR Slot length (ms) of victim cell | Interruption length (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 | 1 |  |
| 1 | 0.5 | 1 |  |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |

Table 8.2.4.2.2-2: Interruption duration for SCell activation/deactivation for intra-band DC/CA

|  | NR Slot length (ms) | Interruption length (ms) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration |
| 1 | 0.5 | 0.5 + TSMTC_duration |
| 2 | 0.25 | 0.5 + TSMTC_duration |
| 3 | 0.125 | 0.5 + TSMTC_duration |
| Note: TSMTC_duration is - the longest SMTC duration among all above active serving cells and the SCell being activated when one SCell is activated; - the longest SMTC duration among all active serving cells in the same band when one SCell is deactivated. |  |  |

8.2.4.2.3	Interruptions during measurements on SCC

Interruptions on PCell due to measurements when an SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [2] is 640 ms or longer. The UE is only allowed to cause interruptions immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in Table 8.2.2.2.2-1 if the PCell is not in the same band as the deactivated SCell. Each interruption shall not exceed requirement in Table 8.2.2.2.2-2 if the PCell is in the same band as the deactivated SCell.

Interruptions on activated SCell due to measurements when an SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [2] is 640 ms or longer. The UE is only allowed to cause interruptions immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in Table 8.2.2.2.2-1 if the activated SCell is not in the same band as the deactivated SCell. Each interruption shall not exceed requirement in Table 8.2.2.2.2-2 if the activated SCell is in the same band as the deactivated SCell.

##### 8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration

The requirements in this clause shall apply when a supplementary UL carrier or an UL carrier is configured or de-configured in NR-DC as defined in TS 38.331 [2].

When an UL carrier or supplementary UL carrier is configured or deconfigured, an interruption of up to the duration shown in table 8.2.4.2.4-1, is allowed during the RRC reconfiguration procedure [2] on all the other activated serving cells within the same FR as the reconfigured uplink carrier. The interruption is for both uplink and downlink of all the other serving cells within the same FR as the configured or de-configured UL.

Table 8.2.4.2.4-1: Interruption duration for UL carrier RRC reconfiguration

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 4 |
| 3 | 0.125 | 8 |

8.2.4.2.5	Interruption due to Active BWP switching Requirement

When UE receives a DCI indicating the UE to switch its active BWP, or when a BWP timer bwp-InactivityTimer defined in TS 38.331 [2] expires, or when the UE receives an RRC command indicating the UE to switch its active BWP, the UE is allowed to cause an interruption on any other serving cells as defined in clause 8.2.2.2.5.

<end of the change>