3GPP TSG-RAN4 Meeting #92BIS 	R4-1911437

# Chongqing, China, 14 – 18 October, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for conditions for cross link interference measurements (scetion B) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2019-10-04 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add new section B.2.7 for conditions for cross link interference measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define conditions for cross link interference measurements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CLI feature is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | section B.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----- << Start of Change 1>> -----

# B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state

## B.2.7	Conditions for SRS-RSRP measurements

This clause defines the following conditions for SRS-RSRP measurement and corresponding procedures performed based on SRSs: SRS_RP and SRS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.7-1 for FR1 NR cells.

The conditions are defined in Table B.2.7-2 for FR2 NR cells.

Table B.2.7-1: Conditions for SRS-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  |  | SRS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSSRS |  |  | dB |
|  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |
| Conditions | NR_TDD_FR1_A | -124 | -121 | -118 | [-3] |
|  | NR_TDD_FR1_C | -123 | -120 | -117 |  |
|  | NR_TDD_FR1_D | -122.5 | -119.5 | -116.5 |  |
|  | NR_TDD_FR1_E | -122 | -119 | -116 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

Table B.2.7-2: Conditions for SRS-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SRS_RP Note 2, Note 3 |  |  |  |  | SRS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSRS |  |  |  |  | dB |
|  |  |  | SCSSRS = 60 kHz |  |  |  | SCSSRS = 120 kHz |  |
|  |  |  | UE Power class |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -130.9 | -122.8 | -119.1 | -130.4 | (Value for SCSSRS = 60 kHz) +3dB | ≥[-3] |
|  |  | n258 | -130.9 | -122.8 | -119.1 | -130.4 |  |  |
|  |  | n260 | -127.9 |  | -116.5 | -128.4 |  |  |
|  |  | n261 | -130.9 | -122.8 | -119.1 | -130.4 |  |  |
|  | Spherical coverage Note 1 | n257 | -1229 | -111.8 | -106.5 | -121.4 | (Value for SCSSRS = 60 kHz) +3dB | ≥[-3] |
|  |  | n258 | -122.9 | -111.8 | -106.5 | -121.4 |  |  |
|  |  | n260 | -119.9 |  | -103.9 | -119.4 |  |  |
|  |  | n261 | -122.9 | -111.8 | -106.5 | -121.4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SRS Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

----- << End of Change 1>> -----