3GPP TSG-RAN WG4 Meeting #92-Bis                                                            R4-1911445

Chongqing, China, 14 – 18 October, 2019

Agenda Item:	9.16.1

Source:	Samsung

Title:		Link budget analysis for n259

Document for:	Discussion

- Background

In the last RAN4 meeting in August, a WF on link budget parameters for Tx/Rx of n259 [1] was approved for encouraging companies to provide their analysis in this meeting as follows:

- To define the maximum output power and Refsens OTA requirements for n259, companies are encouraged to provide the link budget parameters for Tx and Rx in the next meeting according to the tables in the WF. - PC3 is considered as the highest priority - For the analysis and simulations, packaging environment shall be considered (plastic, glass)

This paper presents our view on each parameter in the WF [1] based on our previous simulation efforts on n260 [2] and the latest device which is now on the market supporting FR2 bands to finalize the requirements on time.

- Discussion

Real-product implementation aspects for the peak EIRP and spherical coverage had been considered in Rel-15 of RAN4 discussion which significantly impact the achievable UE requirements. Similarly, before defining the Tx/Rx requirements of n259 UE, the aspects related to practical implementation and modern trend of design should be taken into account, and RAN4 needs to see all the facts as done for simulation assumptions of other FR2 bands last year [3]. As part of this effort, some features of our real products including the packaging environment are summarized as below.

| Released in May, 2019 | NR Frequency Bands (FR2) n261 (28GHz), n260 (39GHz) Main Display Size 169.2mm (6.7" full rectangle) / 165.8mm (6.5" rounded corners) Cover materials Glass (Back) / Metal (Side) |
| --- | --- |
| Released in Sep, 2019 | NR Frequency bands (FR2) n261 (28GHz), n260 (39GHz) Main Display Size 171.6mm (6.8" full rectangle) / 170.5mm (6.7" rounded corners) Cover materials Glass (Back) / Metal (Side) |

Figure 1: Features of commercialized devices affecting link budget analysis

By comparing the simulation results provided by multiple companies for other FR2 bands in Rel-15, RAN4 had studied and found that at least full display and the cover material of practical UE have a considerable impact on the requirements [4]. Moreover, as our devices supporting FR2 bands already have the features in Figure 1, the impact of such facts should be considered again for n259, and shall be captured as a basic assumption before the discussion to specify the requirements for the band.

Observation 1: At least, full display and advanced cover materials shall be captured as a basic assumption before the discussion to specify the requirements for n259.

Based on the observation, we have filled out the table in [1] for the Tx/Rx requirements of n259 as below. The analysis considers features which our UE has and how it works in real world.

Table 1: Link budget for Tx

| Parameter | Unit | Nominal value | Contribution to tolerance |
| --- | --- | --- | --- |
| Frequency range | GHz | 37 - 40 GHz (n260) |  |
| Pout per element | dBm | 10.50 |  |
| # of antennas in an array |  | 4 |  |
| Total conducted power per polarization | dBm | 16.50 | -1.00 |
| Avg antenna element gain | dBi | 4.50 |  |
| Antenna rolloff loss versus frequency | dB | -1.50 |  |
| Realized antenna array gain | dBi | 9.00 |  |
| [Polarization gain] | dB | 2.80 |  |
| Mismatch and transmission line loss including load pull | dB | -2.00 | -0.70 |
| Beam forming loss (phase shifter and amplitude error) | dB | -0.25 | -0.25 |
| Finite beam table | dB | -0.25 | 0.00 |
| Beam forming loss (one beam table fits all) | dB | -0.25 | 0.00 |
| Form factor integration losses | dB | -3.00 | -1.50 |
| Total implementation loss (nominal) | dB | -5.75 |  |
| Total implementation loss (worst case) | dB |  | -8.20 |
| Peak EIRP (Nominal) | dBm | 22.55 |  |
| Tolerance (+/-) | dB |  | 3.45 |
| Peak EIRP (Minimum) | dBm | 19.1 |  |
| Peak EIRP (Maximum) | dBm | 26.0 |  |

Table 2: Link budget for Rx

| Parameter | Unit | Value |
| --- | --- | --- |
| Band number |  | n259 |
| Frequency range | GHz | 39.5 – 43.5 |
| Bandwidth | MHz | 50 |
| Thermal noise | dBm/Hz | -174 |
| Noise Figure | dB | 11.6 |
| Number of antenna in an array |  | 4 |
| Array gain | dB | 4.5 |
| Element gain | dBi | 4.5 |
| Diversity gain | dB | 0 |
| Antenna gain roll-off over frequency | dB | 1.5 |
| Beamforming loss | dB | 0.25 |
| Total insertion loss | dB | 8.2 |
| REFSENS | dBm | -84.5 |

Observation 2: For n259, our proposed value is 19.1 dBm peak EIRP, and -84.5 dBm for REFSENS based on the analysis of our commercial devices supporting FR2 bands as summarized in Table 1 and 2, respectively.

- Conclusion

In this contribution, we provide our view on EIRP and EIS values for the band n259 based on our experience of previous simulation efforts and the UE supporting FR2 bands. Two observations are summarized as below.

Observation 1: At least, full display and advanced cover materials shall be captured as a basic assumption before the discussion to specify the requirements for n259.

Observation 2: For n259, our proposed value is 19.1 dBm peak EIRP, and -84.5 dBm for REFSENS based on the analysis of our commercial devices supporting FR2 bands as summarized in Table 1 and 2, respectively.

- Reference

[1] R4-1910551, “WF on link budget parameters for Tx/Rx of n259,” Huawei et al., RAN4#92, Aug 2019

[2] R4-1714445, “WF on pi/2 BPSK spectrum shaping and power class in FR2,” Intel et al., RAN4#85, Nov 2017

[3] R4-1801202, “WF on EIRP CDF for spherical coverage,” Samsung et al., RAN4 AH-1801, Jan 2018

[4] R4-1805768, “WF on output power requirements on FR2,” Samsung et al., RAN4#86Bis, Apr 2018