3GPP RAN WG4 Meeting #92bis		R4-1911516

Chongqing, China, 14 – 18th October 2019

Agenda item:	10.2.3.1

Source:	Apple Inc.

Title:	Views on the MPAC probe layout for FR1 MIMO OTA

WI/SI:	FS_NR_MIMO_OTA_test

Release:	Rel-16

Document for:	Discussion

# Introduction

This contribution considers the different options proposed for the probe layout configuration for FR1 two-dimensional multi-probe anechoic chambers.

The agreements related to MPAC probe layout reached during the RAN4 #92 meeting have identified two options [1]:

Option 1: 16 probes with 8 uniformly spaced probes (to guarantee LTE backward compatibility) and remaining 8 probes aligned on a 2D sector for NR FR1.

Option 2: 16 probes ring (uniformly spaced) for NR FR1 MPAC system.

This contribution provides the disadvantages for the probe layout architecture prosed in Option 1 and recommends using option 2 for NR FR1 2D MPAC chambers.

# Discussion

The hybrid approach which is stated option 1 uses 8 uniformly spaced probes for LTE MIMO OTA and 8 probes aligned on the 2D sector for NR FR1 MIMO OTA performance evaluation in the 2D MPAC chambers.

The 8 uniformly spaced probes guarantees the backward compatibility for LTE test methodology proposed by different standard bodies.

The layout structure proposed in option 2 that is 16 uniformly distributed probes in the ring of the chamber re uses the existing layout for NR FR1 performance evaluation.

The hybrid approach proposes to implement completely new chamber architecture for NR FR1 MIMO OTA performance evaluation. This will require development and validation efforts, which could potentially delay the NR FR1 MIMO performance evaluation efforts.

The new hybrid system will require architectural and layout changes to the existing 2D MPAC systems. This will involve redoing the entire chamber resulting in additional infrastructure deployments costs and efforts, which is highly undesirable. Accommodating the new probe layout to the existing system will be also pose structural challenges to the existing 2D MPAC systems.

The compatibility of the interference-limited scenarios, that is the CTIA test methodology is unclear in the new hybrid approach.  For with interference-limited test case, the noise has to be uniformly distributed over all the probes. The new architecture will not be able to support this.  CTIA and 3GPP diverging the in probe layout for 2D MPAC chamber is highly undesirable.

  - Proposal 1:  Reusing the existing 16 probe uniformly distributed layout of the 2D MPAC chambers for NR FR1 MIMO OTA performance evaluation is strongly recommended.

# Conclusions

Based on the analysis provided in this paper, the following proposal can be made:

Proposal 1: Reusing the existing 16 probe uniformly distributed probe layout of the 2D MPAC chambers for NR FR1 MIMO OTA performance evaluation is strongly recommended.

# References

[1] R4-1910609, WF on NR MIMO OTA, CAICT, 3GPP RAN4 #92, August 2019

[2] R4-1904160, WF on NR MIMO OTA, CAICT, 3GPP RAN4#90bis, April 2019.

[3] R4-1906829, On 2D NR FR1 MPAC Implementations, 3GPP RAN4#91, May 2019

[4] R4-1900498, 3D MPAC System Proposal for FR1 NR MIMO OTA Testing, 3GPP WG4#90, Mar 2019

[5] R4-1905695, Discussion on probe placement for FR1 MPAC, 3GPP RAN4#91, May 2019

[6] CTIA, Test Plan for 2x2 Downlink MIMO and Transmit Diversity Over-the-Air Performance

[7] T. Laitinen, P. Kyösti, J. Nuutinen and P. Vainikainen, "On the number of OTA antenna elements for plane-wave synthesis in a MIMO-OTA test system involving a circular antenna array," Proceedings of the Fourth European Conference on Antennas and Propagation, Barcelona, 2010, pp. 1-5.