3GPP TSG-RAN WG4 Meeting #92bis	R4-1911525

Chongqing, P.R. China 14th – 18th Sept 2019

Source: 	Nokia, Nokia Shanghai Bell

Title: 					FR2 Intraband Non-contiguous UL CA MPR simulations

Agenda Item: 			8.14.6

Document for:	Discussion

# 1	Introduction

This contribution presents MPR simulation results for non-contiguous uplink intra-band CA. Emission requirements are as agreed in [1] except for OBW, reason for this is explained later.

# 2	Discussion

## 2.1	Simulation scenarios

Two different sub-block scenarios were simulated, one for 2 sub-block case and another for 3 sub-block case. In both cases sub-block bandwidths were 100 MHz and the gap(s) were also 100 MHz. For 2 sub-block case and edge carriers in 3 sub-block case the allocations were always in the outer edges of the sub-block that is in the low edge of the lowest   sub-block and high edge of the highest sub-block. Middle sub-block allocation in 3 sub-block case was placed such that the center of the allocation was equal distance from the center of the lower sub-block allocation and center of the higher sub-block allocation. For 2 sub-block case all possible allocation size combinations were simulated but for 3   sub-block case only a subset.

Figure 1: Allocation placement

## 2.2	Simulation assumptions

PA calibration point was 100 MHz, 120 kHz, QPSK, DFT-S-OFMA, 20RB23with 0 dB MPR

IQ-Image and LO leakage = 25 dBc

CIM3 = 60 dBc

SEM and as in [1]

Table 6.5.2.1-1: General NR spectrum emission mask for frequency range 2.

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Measurement bandwidth |
| 0-5 | -5 | -5 | -5 | -5 | 1 MHz |
| 5-10 | -13 | -5 | -5 | -5 | 1 MHz |
| 10-20 | -13 | -13 | -5 | -5 | 1 MHz |
| 20-40 | -13 | -13 | -13 | -5 | 1 MHz |
| 40-100 | -13 | -13 | -13 | -13 | 1 MHz |
| 100-200 |  | -13 | -13 | -13 | 1 MHz |
| 200-400 |  |  | -13 | -13 | 1 MHz |
| 400-800 |  |  |  | -13 | 1 MHz |
| NOTE 1: Void |  |  |  |  |  |

General spur = -13 dB/1 MHz and as in [1]

ACLR=17 dB and as in [1]

OBW = 99% of the emissions of ALL allocations were inside sub-block bandwidths.

Equal PSD in sub-blocks and equal back-off.

### 2.2.1	OBW

Regarding the occupied bandwidth, 3GPP TS 38.101-2 as well as the ITU and FCC regulation define OBW as the bandwidth that contains 99% of the total transmitted power and state that OBW may not exceed the channel bandwidth, or aggregated channel bandwidth in carrier aggregation.

In non-contiguous CA, OBW cannot be defined sub-blockwise when RBs are allocated in more than one subblock because there is no meaningful way to define how to assign frequency ranges outside the subblocks to each subblock to calculate their total power. On the other hand, omitting the OBW limit may be against regulation.

In our simulations, we used a different approach. The OBW requirement can be rewritten in an equivalent ACLR-like form. The channel must contain at least 99 % of total transmitted power, i.e.,

power_within_channelpower_below_channel+power_above_channel≥0.990.01

Notice that in this alternative form of the requirement, the OBW is not measured at all.

Now, the requirement can be defined for NC CA as

power_within_subblockipower_below_subblocks+power_in_gapk+power_above_subblocks≥0.990.01

It is important that a sufficient number of IMD lobes are included in the measurement of power below and above the subblocks.

Observation: The OBW limit is the gating factor of MPR for most allocations in FR2 NC CA.

In LTE and NR FR1, spurious emission was almost always the gating factor for NC CA. However, the more relaxed spurious limit of FR2 changes the situation. Futhermore, with the larger channel bandwidths of FR2, the widest allocations have too low a PSD to violate the spurious emission limit due to the relatively narrow measurement bandwidth of 1 MHz.

## 2.3	Simulation results

### 2.3.1	2 sub-blocks

Results are presented as squares where x-axis represent allocation size in lower sub-block and y-axis represent allocation size in upper sub-block. Lower left hand corner is the 1RB+1RB case and higher right hand corner is the full allocation on both sub-blocks.

Left hand side figure is the required back-off and right hand side figure is the gating factor. Results for CP-OFDM and DFT-s-OFDM are in Figures 2 and 3 respectively. When comparing the amount of required backoff i.e. MPR we can see that CP-OFDM and DFT-s-OFDM do not differ much. This is due to the fact that as DFT-s-OFDM has two sub-blocks the PAPR increases compared to single carrier case and approaches CP-OFDM PAPR.

Most demanding case from BO point of view is 1RB+1RB as always and easiest is when one of the sub-blocks has very narrow allocation (grey in Fig2, left) and the other sub-block much wider allocation (cyan in Fig2, left).

Figure 2: CP-OFDM

Figure 3: DFT-s-OFDM

### 2.3.2	3 sub-blocks

For aggregation of three CCs, we run a simulation on the so-called triple-beat scenario where the center frequencies of allocations in all three CCs are equally spaced. As a result, several IMD components fall onto each other on an equally spaced grid, thus maximizing the power of IMD lobes in the spectrum. The configuration was 100+100+100MHz (one CC per subblock) with 100 MHz subblocks gaps, QPSK, SCS = 60 kHz.

Figure 4 depicts the simulated back-off as function of the total allocation size and also shows the gating factor of each allocation.

Figure 4: Back-off as function of total allocation size for OFDM (left) and DFT-S-OFDM (right) in triple-beat scenario with 100+100+100 MHz channel combination with 100 MHz gaps between channels.

Another scenario with high emissions is when the centre CC has no allocation. Then all power is concentrated in the allocations of the outermost CCs, resulting in high emission lobes, especially if the CC allocations are narrow. These cases are practically equivalent with the two-CC scenario. They could apply the MPR specification of the two-CC case and do not need to be simulated separately.

# 4	References

[1] R4-1910282, WF on Intra-band non-contiguous ULCA general requirements, Qualcomm, Nokia, Intel, Apple, Huawei, RAN4#92

# 5	Appendix