3GPP TSG-RAN WG4 Meeting #92bis	R4-1911535

Chongqing, P.R. China 14th – 18th Sept 2019

Source: 	Nokia, BT

Title: 					TP to TR 37.716-21-21: Addition of DL_7C_n1A-n78A

Agenda Item: 			9.8.2

Document for:	Approval

# 1	Introduction

This contribution is a TP to TR 37.716-21-21 to add DL_7C_n1A-n78A_UL_7A_n1A, DL_7C_n1A-n78A_UL_7A_n78A, DL_7C_n1A-n78A_UL_7C_n1A and DL_7C_n1A-n78A_UL_7C_n78A

************************** Start of TP *******************************************

## 6.X	DC_7C_n1A-n78A

### 6.X.1	Operating bands for DC

Table 6.X.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n1-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.X.2	Channel bandwidths per operating band for DC

Table 6.X.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  |  | UL Configuration |  | E-UTRA and NR Band |  | Subcarrier spacing (kHz) |  |  |  | 5 MHz |  | 10 MHz |  |  | 15 MHz |  | 20 MHz |  | 25 MHz |  | 30 MHz |  | 40 MHz |  | 50 MHz |  | 60 MHz |  | 80 MHz |  | 90 MHz |  | 100 MHz | Maximum aggregated bandwidth [MHz] |  |  |
| DC_7C_n1A-n78A |  | DC_7A_n1A DC_7C_n1A DC_7A_n78A DC_7C_n78A |  |  |  |  | 7 |  |  | 15 |  |  |  |  | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 160 |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.X.3	Co-existence studies

Co-existence studies of Band 7 + Band n1 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

### 6.X.4	∆TIB and ∆RIB values

For DC_7C_n1A-n78A, the TIB,c and RIB values are same as for DC_1-7_n78 and are given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n1-n78 | 7 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 6.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n1-n78 | 7 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

### 6.X.5	MSD

Based on the co-existence studies in TR 37.716-21-21 [2] DC_7_n1-n78 need to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

************************** End of TP *******************************************