3GPP TSG-RAN WG4 Meeting #92bis	R4-1911536

Chongqing, P.R. China 14th – 18th Sept 2019

Source: 	Nokia, BT

Title: 					TP to TR 37.716-21-21: Addition of DL_3C_n1A-n78A

Agenda Item: 			9.8.2

Document for:	Approval

# 1	Introduction

This contribution is a TP to TR 37.716-21-21 to add DL_3C_n1A-n78A_UL_3A_n1A and DL_3C_n1A-n78A_UL_3A_n78A.

******************************* Start of TP *********************************

## 6.X	DC_3C_n1A-n78A

### 6.X.1	Operating bands for DC

Table 6.X.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.X.2	Channel bandwidths per operating band for DC

Table 6.X.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

|  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | Uplink Configuration |  | E-UTRA and NR Band |  |  | Subcarrier spacing (kHz) |  |  |  | 5 MHz |  | 10 MHz |  |  | 15 MHz |  | 20 MHz |  | 25 MHz |  | 30 MHz |  | 40 MHz |  | 50 MHz |  | 60 MHz |  | 80 MHz |  | 90 MHz |  | 100 MHz | Maximum aggregated bandwidth [MHz] |  |  |
| DC_3C_n1A-n78A |  |  | DC_3A_n1A DC_3A_n78A |  |  |  | 3 |  |  | 15 |  |  |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 160 |  |
|  |  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.X.3	Co-existence studies

Co-existence studies presented in sub-clause 6.8.3 of this TR for DC_3A_n1A-n78A can be reused for DC_3C_n1A-n78A.

### 6.X.4	∆TIB and ∆RIB values

For DC_3C_n1A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n1-n78 | n1 | 0.6 |
|  | 3 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

Table 6.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n1-n78 | n1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

### 6.X.5	MSD

MSD studies presented in sub-clause 6.8.5 of this TR for DC_3A_n1A-n78A can be reused for DC_3C_n1A-n78A.

******************************* Start of TP *********************************