3GPP TSG-RAN WG4 Meeting #92bis	R4-1911556

Chongqing, China, 14 October – 18 October 2019

Source:	ZTE

Title:	Remaining issues on NR reporting criteria

Agenda item:	6.10.2

Document for:	Discussion

1. Introduction

In the RAN4#91 meeting, there were changes on reporting criteria requirements for NR. However the changes bring some issues on measurement reporting configuration on NR serving frequencies. Furthermore requirements for NR reporting criteria has not been finalized. In the RAN4#92 meeting, the remaining issues were discussed in [1]. However no agreement was reached during the meeting.

In this contribution, we further provide our views on remaining issues on reporting criteria for NR.

2. Discussion

2.1. EN-DC reporting criteria

The reporting criteria requirements for EN-DC in TS 38.133 are specified as below.

The UE shall be able to support in parallel per category up to Ecat reporting criteria according to Table 9.1.4.2-1. For the measurement categories belonging to intra-frequency, inter-frequency, and inter-RAT measurements (i.e. without counting other categories that the UE shall always support in parallel), the UE need not support more than the total number of reporting criteria as follows: - For UE configured with EN-DC: Ecat,EN-DC,NR+Ecat,EN-DC,E-UTRA, where Ecat,EN-DC,NR=10+9×n is the total number of NR reporting criteria applicable for UE configured with EN-DC according to Table 9.1.4.2-1, and is the number of configured NR serving frequencies, including PSCell and SCells carrier frequencies, Ecat,EN-DC,E-UTRA is the total number of E-UTRA reporting criteria configured by E-UTRA PCell except PSCell and SCell carrier frequencies, as specified in TS 36.133 [15] for UE configured with EN-DC.

It is clearly specified in TS 38.133 that total number of E-UTRA reporting criteria Ecat,EN-DC,E-UTRA does not include NR serving cells carrier frequencies configured by E-UTRA PCell.

The reporting criteria requirements for EN-DC in 36.133 are specified as below.

The UE capable of supporting EN-DC operation with NR PSCell and one or more NR carrier frequencies in total shall be able to support in parallel per category up to Ecat reporting criteria according to table 8.2.2-1. For the measurement categories belonging to measurements on: E-UTRA intra-frequency cells, E-UTRA inter-frequency cells, inter-RAT per supported RAT, and NR cells on serving and non-serving carrier frequencies (i.e. without counting other categories that the UE shall always support in parallel), the UE need not support more than the number of reporting criteria, excluding reporting criteria specified in TS 38.133 [50] that are applicable for the UE configured with EN-DC operation, as follows: - [36] reporting criteria if the UE is not configured with any SCell or PSCell carrier frequency or NR SCell or NR PSCell, - [36] reporting criteria if the UE is not configured with any SCell or NR SCell but configured with one NR PSCell carrier frequency.

In Table 8.2.2-1 in TS36.133 the measurement category for NR carrier frequency is Inter-RAT NR measurement only. According to the description of measurement category on inter-RAT NR carrier frequency in the table, which is copied as follows, the measurements on inter-RAT NR is only applicable for NR non-serving carrier frequencies.

| Inter-RAT NR carrier frequency Note 5 | [10] | Events for NR cells on all inter-RAT NR carrier frequencies for UE capable of EN-DC operation. Only applicable for UE with this capability and measurements on any of the NR carrier frequencies other than the carrier frequency of the NR PSCell or NR SCell. |
| --- | --- | --- |

However when E-UTRA PCell configures measurements on any NR carrier frequencies, including NR serving carrier frequencies, it is always to be inter-RAT measurements from reporting criteria perspective. In general E-UTRA PCell doesn’t know the NR serving carrier frequencies. Consequently E-UTRA PCell may not be able configure 10 reporting criteria for NR non-serving carrier frequencies.

Observation 1: E-UTRA PCell may not be able configure 10 reporting criteria for NR non-serving carrier frequencies if NR serving carrier frequencies is not known to E-UTRA PCell.

On the other hand, there is one note in Table 9.1.4.2-1 in TS 38.133 as follows.

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Intra-frequency Note 1,2,3,4,5 | 9 | Events for any one or a combination of intra-frequency SS-RSRP, SS-RSRQ, and SS-SINR for NG-RAN intra-frequency cells |
| Inter-frequency Note 2,3,4,5 | 10 | Events for any one or a combination of inter-frequency SS-RSRP, SS-RSRQ, and SS-SINR for NG-RAN inter-frequency cells |
| NOTE 1: When the UE is configured with PSCell and SCell carrier frequencies, Ecat for Intra-frequency is applied per corresponding NR serving frequency. NOTE 2: Applicable for UE configured with SA NR operation mode. NOTE 3: Applicable for UE configured with EN-DC operation mode. NOTE 4: Applicable for UE configured with NE-DC operation mode. NOTE 5: Applicable for UE configured with NR-DC operation mode. |  |  |  |

The reporting criteria for intra-frequency measurement is 9 per NR serving frequency. The reporting criteria for inter-frequency measurement is 10 in total. The total number of NR reporting criteria is Ecat,EN-DC,NR=10+9×n. This means that the NR reporting criteria only takes measurements configured by NR PSCell into account. This would bring some issues for measurement configuration for E-UTRA PCell as the reporting criteria for inter-RAT measurements on NR serving carriers is not specified. When E-UTRA PCell configures Inter-RAT measurements on NR serving carrier frequencies, it may exceeds UE capability of reporting criteria. It is possible if NR PSCell configures NR reporting criteria at maximum total number for intra frequency measurements allowed by UE capability.

Observation 2: It may exceed UE capability of reporting criteria if E-UTRA PCell configures inter-RAT measurements on NR serving carrier frequencies.

Therefore the total number of NR reporting criteria would be better to be increased to take the reporting criteria configured by E-UTRA PCell for inter-RAT measurements on NR serving carrier frequencies into account. However during the discussion in the last meeting chipset vendor expressed concerns on increasing the number of NR reporting criteria. Considering it is late to change UE capability in Rel-15 and RAN2 may consider to solve the issue in RAN2 scope, the current requirements can be kept unchanged.

The reporting criteria for measurement configured by E-UTRA PCell on NR serving cell carrier frequencies is specified in TS38.133 according to current spec structure. However in TS36.133 it states that reporting criteria includes measurements on NR serving cells. This breaks the reporting criteria requirements. In our view reporting criteria for NR serving cells should be removed from TS36.133.

Proposal 1. The reporting criteria requirements for EN-DC in TS36.133 should be revised by removing measurements on NR serving cells.

There is no reporting criteria in TS 36.133 for EN-DC when E-UTRA SCell(s) and/or NR SCell(s) are configured. It is not clear what the E-UTRA reporting criteria would be when SCell(s) is configured. It is necessary to add corresponding requirements for this scenario. Similar requirements as in TS 38.133 for serving carrier frequencies, i.e. scaled by the number of serving carrier frequencies, can be specified.

Proposal 2. The E-UTRA reporting criteria for EN-DC when E-UTRA SCell(s) and NR SCell(s) are configured is necessary to be specified.

Proposal 3. Reporting criteria for EN-DC is 36+9*n when the UE is configured with SCell or PSCell carrier frequencies and with one NR PSCell carrier frequency, and n is the number of E-UTRA SCell and PSCell carrier frequencies.

2.1. NE-DC reporting criteria

The structure of reporting criteria requirements for NE-DC is quite similar to EN-DC. The reporting criteria is separated for measurement categories on NR carrier frequencies and E-UTRA carrier frequencies. Further reporting criteria for E-UTRA carrier frequencies is separated for serving carrier frequencies and non-serving carrier frequencies.

- For UE configured with NE-DC: Ecat,NE-DC,NR+Ecat,NE-DC,E-UTRA, where Ecat,NE-DC,NR=10+9×n is the total number of NR reporting criteria according to Table 9.1.4.2-1, and is the number of configured NR serving frequencies, including PCell and SCells carrier frequencies, Ecat,NE-DC,E-UTRA=Ecat,NE-DC,E-UTRA,inter-RAT+Ecat,NE-DC,E-UTRA,intra-RAT, where Ecat,NE-DC,E-UTRA,inter-RAT is the total number of inter-RAT E-UTRA reporting criteria configured by PCell except E-UTRA PSCell and E-UTRA SCell carrier frequencies, according to Table 9.1.4.2-1, Ecat,NE-DC,E-UTRA,intra-RAT is the total number of E-UTRA reporting criteria including E-UTRA PSCell and SCell carrier frequencies as specified in TS 36.133 [15] for UE configured with NE-DC.

Since measurements on NR carrier frequencies can only be configured by NR PCell, the current reporting criteria for measurements on NR carrier frequencies is fine when UE is in NE-DC operation. For reporting criteria for measurements on E-UTRA carrier frequencies, the number is not specified yet.

The UE capable of supporting and configured with NE-DC operation with PSCell and NR PCell and one or more NR carrier frequencies in total shall be able to support in parallel per category up to Ecat reporting criteria according to table 8.2.2-1. For the measurement categories belonging to measurements on: E-UTRA intra-frequency cells and E-UTRA inter-frequency cells, inter-RAT per supported RAT, and NR cells on serving and non-serving carrier frequencies (i.e. without counting other categories that the UE shall always support in parallel), the UE need not support more than the number of reporting criteria, excluding reporting criteria specified in TS 38.133 [50] that are applicable for the UE configured with NE-DC operation, as follows: - [TBD] reporting criteria if the UE is not configured with any SCell or NR SCell.

Regarding the number of reporting criteria for E-UTRA carriers specified in TS36.133 we propose to use similar approach as in TS 38.133 for NR carriers which makes the spec more consistently. The requirements should be specified for both single carrier and CA case.

Proposal 4. For NE-DC, the total number of E-UTRA reporting criteria is Ecat,NE-DC,E-UTRA=10+9×n, and  is the number of configured E-UTRA serving frequencies, including PSCell and SCells carrier frequencies.

3. Conclusion

In this contribution, we provide our views on remaining issues on reporting criteria for NR. Based on the observations following proposals are present.

Proposal 1. The reporting criteria requirements for EN-DC in TS36.133 should be revised by removing measurements on NR serving cells.

Proposal 2. The E-UTRA reporting criteria for EN-DC when E-UTRA SCell(s) and NR SCell(s) are configured is necessary to be specified.

Proposal 3. Reporting criteria for EN-DC is 36+9*n when the UE is configured with SCell or PSCell carrier frequencies and with one NR PSCell carrier frequency, and n is the number of E-UTRA SCell and PSCell carrier frequencies.

Proposal 4. For NE-DC, the total number of E-UTRA reporting criteria is Ecat,NE-DC,E-UTRA=10+9×n, and  is the number of configured E-UTRA serving frequencies, including PSCell and SCells carrier frequencies.

4. References

- R4-1909535	Remaining issues on NR reporting criteria, ZTE