3GPP TSG-WG RAN4 Meeting #92bis	R4-1911587

# Chongqing, China, 14 – 18 October, 2019

Source: 	Nokia, Nokia Shanghai Bell, Orange

Title: 					TP to TR 36.716-04-01: CA_1-3-7-7-20

Agenda Item: 			7.4.2

Document for:	Approval

# Introduction

This TP introduces the following LTE inter-band CA for 4 bands DL with 1 band UL for the CA bands CA_1-3-7-7-20.

- CA_1A-3A-7A-7A-20A-BCS0

**************************** Start of changes ********************************************

## 5.X	CA_1-3-7-7-20

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 4-band DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_1A-3A-7A-7A-20A | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
|  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.X.2	∆TIB and ∆RIB values

The ∆TIB and ∆RIB values of CA_1-3-7-7-20 is proposed to be the same as CA_1-3-7-20 already specified in TS 36.101.

Table 5.X.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-3-7-7-20 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.3 |

Table 5.X.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-3-7-7-20 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |

### 5.X.3	REFSENS requirements

REFSENS requirements of CA_1A-3A-7A-20A defined in TS 36.101 can be applied for CA_1A-3A-7A-7A-20A.

Table 7.3.1A-0bD1 and Table 7.3.1A-0bD2 in TS 36.101 shall include the REFSENS and uplink configurations according to the following tables.

Table 5.X.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for four bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-7A-7A-20A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 9: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 5.X.3-2: Uplink configuration for the low band (exceptions for four bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-7A-7A-20A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 |  |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

**************************** End of changes ********************************************