3GPP TSG-RAN WG4 Meeting #92bis                              R4-1911597

Chongqing, China, Oct. 14th – 18th, 2019

Source: 	MediaTek Inc.

Title: 	n259 PC3 minimum peak EIRP and REFSENS link budgets

Agenda Item:	9.16.1

Document for:	Discussion

# Introduction

Following the WF on link budget parameters for Tx/Rx of n259 as approved in last RAN4 meeting [1], in this contribution, we share our link budget numbers for PC3 minimum peak EIRP and REFSENS for consideration.

# Discussion

Table 2-1 and Table 2-2 summarize our n259 PC3 minimum peak EIRP and REFSENS link budget numbers respectively. Notice that for minimum peak EIRP, we only provide the worst-case numbers as the nominal values would not be relevant to the intended specifications.

| Parameter | Unit | Nominal value | Worst value |
| --- | --- | --- | --- |
| Frequency range | GHz | 39.5 - 43.5 GHz |  |
| Pout per element | dBm |  | 10.5 |
| # of antennas in an array |  | 4 | 4 |
| Total conducted power per polarization | dBm |  | 16.5 |
| Avg. antenna element gain | dBi |  | 3.3 |
| Antenna roll-off loss versus frequency | dB |  | -1.6 |
| Realized antenna array gain | dBi |  | 7.7 |
| Polarization gain | dB |  | 2.5 |
| Mismatch and transmission line loss including load pull | dB |  | -1.5 |
| Beam forming loss (phase shifter and amplitude error) | dB |  | -1.4 |
| Finite beam table | dB |  | 0 |
| Beam forming loss (one beam table fits all) | dB |  | -0.5 |
| Form factor integration losses | dB |  | -5.0 |
| Total implementation loss (nominal) | dB |  |  |
| Total implementation loss (worst case) | dB |  |  |
| Peak EIRP (Nominal) | dBm |  |  |
| Tolerance (+/-) | dB |  |  |
| Peak EIRP (Minimum) | dBm |  | 18.3 |
| Peak EIRP (Maximum) | dBm |  |  |

Table 2-1 n259 PC3 minimum peak EIRP link budget numbers

| Parameter | Unit | Value |
| --- | --- | --- |
| Band number |  | n259 |
| Frequency range | GHz | 39.5 – 43.5 |
| Modulation |  | QPSK |
| SNR Requirement | dB | -1.0 |
| Bandwidth | MHz | 50 |
| Thermal noise | dBm/Hz | -174.0 |
| Noise Figure | dB | 11.5 |
| Number of antenna in an array |  | 4 |
| Array gain | dB | 6.0 |
| Element gain | dBi | 3.3 |
| Diversity gain | dB | 0 |
| Antenna gain roll-off over frequency | dB | -1.6 |
| Beamforming loss | dB | -1.5 |
| Total insertion loss | dB | -8.0 |
| REFSENS | dBm | -84.7 |

Table 2-2 n259 PC3 REFSENS link budget numbers

# Conclusion

In this contribution, we share our n259 PC3 link budget numbers for minimum peak EIRP and REFSENS for consideration.

# References

- R4-1910511 “WF on link budget parameters for Tx/Rx of n259”, Huawei, Apple, 3GPP TSG-RAN WG4 Meeting #92, Ljubljana, Slovenia, August 26th – 30th, 2019