3GPP TSG-RAN WG4 Meeting #92				  												R4-1911606

Chongqing, China, 10th Oct. 2019 - 18th Oct. 2019

Source: 	Nokia, T-Mobile

Title: 						TP for 37.716-21-21 to introduce DC_2A-66A_n41A-n261

Agenda Item: 			9.8.3 [DC_R16_xBLTE_2BNR_yDL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.716-21-21 (v0.7.0) to introduce DC_2A-66A_n41A-n261 which is listed in WID

### EN-DC of LTE inter band CA for x bands DL with 1 band UL(x=1, 2, 3, 4) + NR inter-band CA for 2 bands DL with 1 band  UL

| EN-DC configuration | Uplink EN-DC Configuration | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_2A-66A_n41A-n261A | DC_2A_n41A | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom | new | (new) DL_2A_n41A-n261A_UL_2A_n41A |
| DC_2A-66A_n41A-n261A | DC_66A_n41A | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom | new | (new) DL_66A_n41A-n261A_UL_66A_n41A |
| DC_2A-66A_n41A-n261(2A) | DC_2A_n41A | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom | new | (new) DL_2A_n41A-n261(2A)_UL_2A_n41A (new) DL_2A-66A_n41A-n261A_UL_2A_n41A |
| DC_2A-66A_n41A-n261(2A) | DC_66A_n41A | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom | new | (new) DL_66A_n41A-n261(2A)_UL_66A_n41A (new) DL_2A-66A_n41A-n261A_UL_66A_n41A |

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 7.x	DC_2A-66A_n41A-n261

### 7.x.1	Operating bands for DC

Table 7.x.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-66A-n41A-n261 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |

### 7.x.2	Channel bandwidths per operating band for DC

Table 7.x.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_2A-66A_n41A-n261A | DC_2A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| DC_2A-66A_n41A-n261A | DC_66A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| DC_2A-66A_n41A-n261(2A) | DC_2A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 940 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(2A) in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-66A_n41A-n261(2A) | DC_66A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 940 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | § | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(2A) in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.x.3	Co-existence studies

Co-existence studies of DC_2A-n41A are captured in TR 37.716-01-01, for DC_66_n41 are captured in TR 37.716-11-11 and for EN-DC between E-UTRA and FR2 DC_2A_n261A or DC_66A_n261A there will no requirements specified.

### 7.x.4	∆TIB and ∆RIB values

For DC_2A-66A_n41A-n261A, the TIB,c and RIB,c values are given in the tables below based on DC_66A_n41A-n261A and DC_2A_n41A-n261A in 37.716-21-21.

Table 7.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-66A_n41A-n261A | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
|  | n261 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 7.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2A-66A_n41A-n261A | 2 | 0.3 |
|  | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n261 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

### 7.x.5	MSD

MSD due to close proximity is needed and is already specified in TS 38.101-3 for DC_66_n41.

*********************** End of the TP ***************************************************

# 3	References